# Title 7

# PUBLIC WAYS AND PROPERTY

Street And Right of Way Improvements	7-1

7-1-1:  Purpose					   	             pg.  4

7-1-2:  Permits Required

7-1-3:  Application; Requirements; Fees

7-1-4:  Construction Inspection

7-1-5:  Permit Revocation

7-1-6:  Penalty

Sidewalk Maintenance and Construction	7-2

7-2-1:  Definitions							        pg.  8

7-2-2:  Permit Required

7-2-3:  Construction Or Repair Specifications

7-2-4:  Sidewalk Maintenance

7-2-5:  City Council Duties

7-2-6:  Mayoral Responsibilities

7-2-7:  Notice To Property Owner

7-2-8:  Failure to Comply

7-2-9:  Special Taxes; Cost Assessment

7-2-10: Payment Of Tax; Penalty for Nonpayment

7-2-11: Penalty

Road Naming and Addressing	7-3

7-3-1:  General Provisions						   pg. 13

7-3-2:  Road Designations

7-3-3:  Addressing Procedures

7-3-4:  Road Naming

7-3-5:  Signing

7-3-6:  Administrative Procedures

7-3-7:  Appeals

7-3-8:  Controversial Or Disputed Road Names

7-3-9:  Violation; Penalty

Drinking Water Regulations                                   7-4

7-4-1:  Definitions                                       pg. 22

7-4–2:  Ownership and Management of Water System

7-4–3:  Connection Required

7-4–4:  Water System and Fees Established

7-4–5:  Hookup Fee

7-4–6:  Water Capacity Buy-in Fee

7-4–7:  Monthly Service Fees

7-4–8:  Cross-Connection Control Program

7-4–9:  Water Service Lines

7-4-10: Private Water Mainline Extensions

7-4-11: Extension of City Water Mains

7-4-12: Water System Design and Materials

7-4-13: Subdivision Developer's or Property Owner

Responsibility

7-4-14: Fire Hydrants

7-4-15: Miscellaneous Provisions

7-4-16: Liability for Damages

7-4-16: Enforcement

Attachment: Division 400 Supplement to the 2020 Edition of Idaho Standards for Public Works Construction

Individual Water and Wastewater Systems	7-4A

7-4A-1: Water and Wastewater Systems				   pg. 59

7-4A-2: Exceptions

7-4A-3: Procedures for Sandpoint Wells Not to Exceed Twenty 
          Feet

7-4A-4: Violations and Penalties

SEWER USE REGULATIONS                                       7-5

7-5-1:  Definitions                                      pg. 63

7-5-2:  Ownership and Management of Sewer System

7-5-3:  Connection Required

7-5-4:  Sewer System and Fees Established

7-5-5:  Hookup Fee

7-5-6:  Sewer Capacity Buy-in Fee

7-5-7:  Monthly Service Fees

7-5-8:  Use of Public Sewer

7-5-9:  Sewer Service Lines

7-5-10: Private Sewer Mainline Extensions

7-5-11: Extension of City Sewer Mains

7-5-12: Sewer System Design and Materials

7-5-13: Subdivision Developer's or Property Owner's 
          Responsibility

7-5-14: Miscellaneous Provisions

7-5-15: Enforcement

Attachment:	Division 500 Supplement to the 2020 Edition of the Idaho Standards for Public Works Construction

City Park	7-6

7-6-1: Park Rules And Regulations					   pg. 97

Subsequent Connection Utility Fees	7-7

7-7-1: Purpose									   pg. 98

7-7-2: Establishment Of Fees

7-7-3: Basis For Calculation

7-7-4: Payment Required Before Connection

7-7-5: Fees Established by Resolution; Analysis Required

7-7-6: Administrative Officials Enabled

Alleys	7-8

7-8-1: Purpose And Intent						  pg. 101

7-8-2: Dumping In Alleys, Restriction Prohibited

7-8-3: Placement For Pick Up

7-8-4: Alleys To Be Kept Clear of Vehicles

7-8-5: Violation; Penalty

CHAPTER 1

STREET AND RIGHT OF WAY IMPROVEMENTS

SECTION:

7-1-1: 	Purpose

7-1-2: 	Permits Required

7-1-3: 	Application; Requirements; Fees

7-1-4: 	Construction Inspection

7-1-5: 	Permit Revocation

7-1-6: 	Penalty

7-1-1: 	PURPOSE:  The purpose of this chapter is to regulate 			     the construction and/or capital maintenance of public facilities or the facilities of private utilities within the public rights of way by issuance of construction permits and the oversight of inspection of such facilities in order to assure, to the greatest extent practical, that utility construction work and/or other construction work within the public right of way complies with applicable city ordinances and construction standards adopted by the city. (2025 Code)

7-1-2: 	PERMITS REQUIRED:

A. 	Street Improvements: All construction of new street 			improvements, including, but not limited to, street base 		preparation, paving, installation of curbs, gutters, 			sidewalks, storm water facilities, water line construction, 	replacement of such facilities, or construction, 				replacement, or capital maintenance of any public utility 
     or other facilities desiring to make use of the public 
     right of way, including facilities providing electrical, 
     telephone, natural gas, irrigation water, and cable 
     television service shall require a right of way 
     construction permit issued by the city.

B. 	Public Right of Way: Any construction work in the public 		right of way which will affect the condition of existing 		public facilities or will result in the construction of 		facilities which will be subject to public maintenance 
     shall be subject to the provisions of this chapter.

C. 	New Subdivisions: A public facility construction permit 		shall be required prior to the initiation of construction 		work on a new subdivision. Said permits may be issued by 
     the city engineer or his authorized representatives after 
     review and approval of plans and specifications, and 
     payment of required fees. No fee shall be charged for 
     construction undertaken pursuant to direction by the city.

D. 	Stockpiling Of Materials: No temporary stockpiling, storage 	of materials, demolition, storage of equipment, or other 		activities which would obstruct a public right of way shall 	be allowed without a permit issued pursuant to this 
     chapter.

E. 	Improvement Specifications: The mayor and/or council is 		hereby authorized to develop construction and improvement 		specifications for installations in public rights of way 		within the city. Said standards shall be presented to the 		city council for review and shall be effective upon passage 	of a resolution adopting said standards. Such standards may 	be changed by subsequent resolution of the city council. 		(2025 Code)

7-1-3: 	APPLICATION; REQUIREMENTS; FEES:

A. 	Application: Applications for permits required by this 			chapter shall be made to the city on forms provided for 
     that purpose. No work subject to the provisions of this 
     chapter shall be initiated until an application has been 
     completed and approved according to the provisions of this 
     chapter.

B. 	Requirements: Permit requests shall include site plans, 		engineering plans, specifications, and such other 				information as may be necessary to allow the city to 			determine that standards have been complied with.

C. 	Fees: No permit application shall be deemed complete until 		the fees required for said application have been paid in 		full. The fees for permit issuance and for construction 		inspection oversight are authorized by this chapter. Said 		fees may be set, and changed from time to time, by 			resolution of the city council.

D. 	Advance Timing: Applications for permits required by this 		chapter shall be made in advance of the date upon which 
     work is scheduled to commence. Advance timing shall be 
     variable depending upon the scope of the work to be 
     completed and the degree of prior review which has been 
     conducted. Review periods may be waived by the city in the 
     event of emergencies. The permit shall provide a time limit 
     within which the work shall be completed. The permit shall 
     be void if the work is not commenced and completed within 
     the date specified on the permit unless an extension of 
     time, for good cause, is granted by the city.

E. 	Insurance Required: Any applicant for a permit authorized 
     by this chapter shall demonstrate that it has general 
     liability insurance in the amount of one hundred thousand 
     dollars ($100,000.00) which insures said work against 
     damage to public facilities and against damage to the 
     public in general. No permit shall be issued without proof 
     of such insurance. The city is authorized to maintain a 
     registration of qualified contractors for purposes of 
     simplifying administration of this requirement. Any such 
     proof of insurance shall require at least thirty (30) days' 
     advance notice of cancellation or expiration to the city.

F. 	Bond: A cash or surety bond in the amount specified by the 		city is required prior to issue of a permit. The amount of 		bond will be determined and set by the city. Bond will be 		released upon completion of all right of way work and as 		built drawings are approved and accepted by the city and 		other agencies as specified.

G. 	Permit Binding on Applicant: Any permit required by this 		chapter shall be binding upon the applicant until all 			obligations established by the terms of this chapter and by 	the terms of the specific permit have been complied with in 	their entirety. A permit issued pursuant to this chapter 		shall not be transferable to any other person or entity. 
     The city may require written proof that the person making 			application is a bona fide representative of the identified 	permit applicant. (2025 Code)

7-1-4: 	CONSTRUCTION INSPECTION:  Primary responsibility for 			     construction inspection for new subdivisions shall rest with the permittee's licensed professional engineer. The permittee's engineer shall provide the city with documentation of compliance with city construction standards and attendant construction observation necessary to verify compliance, as required by the city. The city retains an absolute right to inspect and to require production of test results to assure compliance with city standards. If test results are not produced or inspection is impeded, the permit may be revoked in the sole discretion of the city. (2025 Code)

7-1-5: 	PERMIT REVOCATION:

A. 	Material Noncompliance: Notwithstanding compliance with the 	application procedure set forth in this chapter or with 		resolutions adopted pursuant to this chapter, any permit 		issued pursuant to the authority established by this 
     chapter shall be revocable, for cause, in the sole 
     discretion of the city. Revocation may occur in summary 
     fashion upon the happening of any material noncompliance 
     with permit terms, material noncompliance with terms of 
     this chapter or standards adopted pursuant to it, or for 
     such other substantial reason as the city might determine, 
     including the submittal of inaccurate or misleading 
     information on any application.

B. 	Work Cessation: Revocation of any such permit shall require 	that work cease unless specifically allowed to continue, by 	written order of the city, to eliminate a hazard or to 
     preserve the integrity of work in progress. Issuance of a 
     permit pursuant to this chapter shall not convey any 
     ongoing right to construct in or occupy any public right of 
     way. The permission to construct authorized by this chapter 
     shall be revocable at will and without recourse. All such 
     permits are temporary and shall be effective only for the 
     limited time frame authorized in the permit.

C. 	Remedial Action: The city reserves the right to remedy any 		condition associated with construction in a public right of 	way which it deems to pose an imminent threat to the public 	health, safety and welfare. The costs of said remedial 			action shall constitute a charge against the permit 			applicant, which cost shall be reimbursed to the city prior 	to restart of construction. (2025 Code)

7-1-6: 	PENALTY:  Violations of this chapter shall constitute 
          a misdemeanor. This chapter may also be enforced by initiation of civil legal action to compel compliance with the terms of this chapter or to restrain violation thereof. All costs of civil enforcement shall be borne by the violator. (2025 Code)

CHAPTER 2

SIDEWALK MAINTENANCE AND CONSTRUCTION

SECTION:

7-2-1: 	Definitions

7-2-2: 	Permit Required

7-2-3: 	Construction Or Repair Specifications

7-2-4: 	Sidewalk Maintenance

7-2-5: 	City Council Duties

7-2-6: 	Mayoral Responsibilities

7-2-7: 	Notice To Property Owner

7-2-8: 	Failure To Comply

7-2-9: 	Special Taxes; Cost Assessment

7-2-10: 	Payment Of Tax; Penalty for Nonpayment

7-2-11: 	Penalty

7-2-1: 	DEFINITIONS:  As used in this chapter, the following 			     words and terms shall have the meanings ascribed to them in this section:

AUTHORIZED 	Any wheeled mode of transportation specifically WHEELED		designed to enhance the mobility of the   TRAFFIC:	     physically handicapped or of children not yet 
               capable of walking.

PUBLIC 		A strip of land not exceeding ten feet (10') in SIDEWALK:		width which is physically located on each and 
               both sides of every part of public right of way 
               platted as a city street. Said sidewalk, be it 
               unimproved or of permanent concrete or other 
               construction, is reserved exclusively for the use 
               of foot and authorized wheeled traffic. (2025 
               Code)

7-2-2: 	PERMIT REQUIRED:  Prior to work beginning on any 				     sidewalk removal, replacement, construction or repair, a permit must first be obtained through the office of the city clerk-treasurer. (2025 Code)

7-2-3: 	CONSTRUCTION OR REPAIR SPECIFICATIONS:  Before any 			     sidewalk is constructed or repaired upon a public right of way within the corporate limits of the city, that person who shall perform such work shall first obtain from the city written specifications to which such work shall adhere. (2025 Code)

7-2-4: 	SIDEWALK MAINTENANCE:  It shall be the duty and 				     responsibility of the property owner to maintain any sidewalk and/or gutter abutting his/her property in a clean, litter/weed and brush/snow/ice free condition, and to ensure that the surface of said sidewalk is maintained in a safe and proper physical condition, said physical condition presenting a flat, unbroken surface upon which foot and authorized wheeled traffic shall traverse smoothly and unimpeded. This duty of the property owner shall be continuously in effect whether or not the property owner has received a notice from the city. (2025 Code)

7-2-5: 	CITY COUNCIL DUTIES:

A. 	Direct Removal, Replacement, Construction Or Repair By 			Owners: It shall be the duty of the council, whenever in 
     its judgment or opinion it may be necessary or proper to, 
     by council resolution, require and direct certain property 		owners within the corporate limits of the city to 
     construct, remove, replace, repair or otherwise maintain 
     the public sidewalk located upon that portion of the public 
     right of way abutting the property of said certain property 
     owners.

B. 	Action By Resolution: Whenever the council shall take 
     action pursuant to subsection A of this section, such 
     action to require and direct the initial construction, 
     removal or replacement, repair or other maintenance of 
     permanent type sidewalks shall be accomplished in the form 
     of a council resolution. Such a resolution shall:

1. Identify a sidewalk, portion of a sidewalk or intended 		site of a new permanent type of sidewalk by stipulating the 	name of the street and the numbers of the lot and block of 		the property abutting such a sidewalk or site.

2. Specify the reason for the ordering of such required 		action.

3. Specify the time limit within which the property owner 		must complete the required action after receipt of the 			city's notice to do so. (2025 Code)

7-2-6: 	MAYORAL RESPONSIBILITIES:

A. 	Direct Cleaning By Owners: It shall be the duty of the 			mayor, whenever in his/her judgment or opinion it may be 		necessary or proper, to require and direct certain property 	owners within the corporate limits of the city to remove 
     any litter, debris, weeds, brush or other object or 
     material from any sidewalk and/or gutter and to remove any 
     snow or ice from any permanent type sidewalk, any and all 
     of which may be situated upon that portion or the public 
     right of way abutting the property of said certain property 
     owners.

B. 	Failure To Comply: In the event that a request to a 
     property owner, delivered in accordance with subsections 7-
     2-7B1 and B2 of this chapter should fail to achieve the 
     desired results within ten (10) days, the mayor shall cause 
     said cleanup to be accomplished and the city's actual cost 
     shall ultimately be borne by that property owner failing to 
     comply with the request, said cost to be collected by the 
     city through the procedures detailed in sections 7-2-8, 7-
     2-9 and 7-2-10 of this chapter. (2025 Code)

7-2-7: 	NOTICE TO PROPERTY OWNER:

A. 	Notice To Property Owner: Whenever the council shall pass a 	resolution pursuant to subsection 7-2-5B of this chapter, 		the mayor shall be responsible for ascertaining from the 		county assessor's current records the name and mailing 			address of the owner of any property described in said 			council resolution. To each property owner, the mayor shall 	cause to be delivered, in the manner prescribed in 			subsection B of this section, a written notice which shall 		be substantially in the following format:

To: (Owner's Name). According to the current records of the county assessor of             County, Idaho, you are the owner of the property described as Block          , Lot(s)           within the corporate limits of the city of Ririe, Idaho.

The city council of the city of Ririe, Idaho, pursuant to Idaho Code section 50-316, and the resulting ordinance         of the city of Ririe, has declared that portion of the public sidewalk which is situated on                                                    Street/Avenue and                            Street/Avenue, and upon which your lot(s) abut:

□ Shall be converted by you, at your expense, to a permanent type (type of material) sidewalk.

□ Is cracked or otherwise deformed to the extent that it no longer presents a smooth, level, safe surface over which public traffic may pass and, thus, must be removed and replaced by you, at your expense, with a new, permanent-type sidewalk of (type of material).

□ Is damaged and must be repaired by you, at your expense, as follows: (Detailed description of repairs to be done.)

□ Other work to be performed at your expense (describe).

This is to inform you that both you and the city could be held legally liable for any damages incurred by accident or injury to any citizen resulting from the present condition of the above-described sidewalk, and YOU ARE HEREWITH ORDERED TO COMPLY WITH THE ABOVE in accordance with Council resolution number      , passed on                  20   . Said work shall be completed by 3:30 P.M. on                                20   .

Before the start of any sidewalk construction or repair, a permit must first be obtained from the city of Ririe.

Failure to comply with this notice within the time limit stated above will result in the city causing the prescribed work to be performed and the cost of such removal, replacement, construction or repair will be at your expense, collectable either directly from you or through a tax lien against your property, as prescribed by Idaho Code.

B. 	Method Of Delivery of Notice to Property Owner:

1. Such notice, as described in subsection A of this 
     section shall be served by delivering the same to such 
     owner or by leaving it at the house of his usual abode with 
     some adult person residing therein; provided, however, that 
     if such owner is not a resident of the city, then such 
     notice may be served by mail by depositing such notice in 
     the post office, addressed to the person to whom it is to 
     be served at his place of residence, if known, and if not 
     known, then addressed to the last known place of residence, 
     with postage prepaid.

2. The mayor shall make such notice, as prescribed in 			subsection 7-2-7A of this section in duplicate and on one 		copy he shall make an affidavit of service. Such affidavit 		shall be made before the city clerk-treasurer or a notary 		public, and shall be filed with the city clerk-treasurer. 		(2025 Code)

7-2-8: 	FAILURE TO COMPLY:  If, after such resolution as 				     prescribed in subsection 7-2-5A of this chapter is passed by the council and notice, as prescribed in subsection 7-2-7A of this chapter has been delivered to a property owner, said property owner refuses to construct or repair such sidewalk within the time required by the resolution and notice, then the mayor shall, without any further orders from the council, cause the construction or repair to be accomplished, either by day labor or by contract work, and the full cost thereof shall be assessed on and against the property abutting such repair or construction, and the full amount of such costs shall become a lien upon the property and shall, at the time provided by law, be certified by the council and/or city clerk-treasurer to the county tax collector to be placed on the proper tax roll to be collected as other taxes are collected and shall be in addition to all other taxes legally assessed against such property. (2025 Code)

7-2-9: 	SPECIAL TAXES; COST ASSESSMENT:  All such assessments, 		as referred to in sections 7-2-8 and 7-2-10 of this chapter, shall be known as special tax assessments for improvements and shall be levied and collected as a special tax. All money received from such special assessments shall be held by the city clerk-treasurer as a special fund to be applied to the payment of the improvement for which the assessment was made. The money shall be used for no other purpose whatever except to reimburse the city for money expended for such improvement. (2025 Code)

7-2-10: 	PAYMENT OF TAX; PENALTY FOR NONPAYMENT:  Any    
          assessment against the property as provided in section 7-2-8 of this chapter may be paid by the property owner to the clerk-treasurer in full, and have the lien thereof discharged. If the amount assessed against any such property is not paid on or before August 20 following the completion of construction or repairs, there shall be added to the amount so assessed against the property whatever amount the county shall charge for the collection of such money, plus interest on the unpaid amount at the prevailing legal rate from the date of completion of work. (2025 Code)

7-2-11: 	PENALTY:  Any person found to be in violation of this 			     chapter shall be guilty of committing a misdemeanor offense and shall be punished in accordance with section 1-4-1 of this code. (2025 Code)

CHAPTER 3

ROAD NAMING AND ADDRESSING

SECTION:

7-3-1: 	General Provisions

7-3-2: 	Road Designations

7-3-3: 	Addressing Procedures

7-3-4: 	Road Naming

7-3-5: 	Signing

7-3-6: 	Administrative Procedures

7-3-7: 	Appeals

7-3-8: 	Controversial Or Disputed Road Names

7-3-9: 	Violation; Penalty

7-3-1: 	GENERAL PROVISIONS:

A. 	Title: This chapter shall be known as the ROAD NAMING AND 		ADDRESSING ORDINANCE OF THE CITY OF RIRIE, IDAHO.

B. 	Authority: This chapter is authorized under the provisions 		of the Idaho constitution article 12, section 2, and by 		Idaho Code title 50, chapter 13 and title 67, chapter 65, 
     as amended and subsequently codified.

C. 	Purpose: This chapter further implements portions of the 		city's comprehensive plan and supplements the applicable 		zoning and subdivision ordinances. Its purpose is to 
     provide the residents of the city with a uniform and 
     standardized system of road naming and addressing to:

1. Minimize future road name and addressing conflicts.

2. Provide a database for city records and enhanced 911 		service.

3. Expedite property identification by emergency services.

4. Comply with the following addressing guidelines: 
     National Emergency Number Association (NENA), "recommended 			abbreviations", GTE E911 addressing guidelines, practice 		444-400-015, and U.S. postal service publication 28 – 			addressing.

D. 	Jurisdiction: These regulations shall apply to the naming 
     of all public and common private roads and the addressing 
     of all structures and parcels within the incorporated 
     territory of the city, or mutually defined by both city and 
     Jefferson and Bonneville Counties under the requirements 
     and authority of Idaho Code sections 50-1306, 67-6518, or 
     67-6526, as amended or subsequently modified.

E. 	Applicability: This chapter shall apply to all property and 	to all public and private roads within the city.

F. 	Implementation And Responsibilities:

1. The city council shall have the authority to implement, 		enforce and maintain the addressing and road naming 			standards as defined by this chapter.

2. If violations of this chapter are not corrected 
     by the date required herein, the city may perform 		the work and bill the property owner for the reasonable 		costs thereof.

3. Property owners are responsible for placing alphanumeric 	identification in accordance with subsections 7-3-5A and B 		of this chapter, and for erecting private road signs in 		accordance with subsection 7-3-5C of this chapter. Posting 		of addresses and road names must be done within four (4) 		months of issuance of the address and road name.

4. The street department is authorized to erect and 
     maintain public road signs at public road intersections. 
     Private road signs shall be erected as required by this 
     chapter.

5. The individual property owner, or the developer in the 		case of a subdivision, is responsible for the designation 
     of the access points on the public or private roads and 			installation and maintenance of all required road signs. 		These designated access points shall be utilized to 			determine the address of the parcel. Except as such access 		points exist at the time of adoption of the ordinance 			codified herein, all future access points shall comply with 	this chapter and other ordinances of the city.

6. All addresses shall comply with the addressing 
     guidelines outlined in subsection C4 of this section to the 
     extent that such compliance is possible. The order of 
     priority shall be NENA, GTE and USPS.

7. For the purposes of addressing, the city will hold and 		maintain all official data.

8. The mayor and all administrative staff subject to the 		mayor's supervision are hereby authorized to undertake such 	administrative steps as may be necessary to implement the 		provisions of this chapter. In taking such steps, city 
     staff is encouraged to apply common sense and practicality 
     to achieving the ends intended by this chapter.

G. 	Guidelines Of Interpretation: Words used in the present 		tense include the future; words used in singular number 		include the plural and vice versa. The word "shall" is 			mandatory, the word "should" is directory, and the word 		"may" indicates the use of discretion. Unless the subject 		matter clearly requires otherwise, the words or phrases 
     used shall have the meanings commonly appropriate to their 			context and the purposes of this chapter. (2025 Code)

7-3-2: 	ROAD DESIGNATIONS:

A. 	All named roads that extend from incorporated areas into 		unincorporated areas shall retain the same name.

B. 	Roads that have a definite north-south directional course 		should be designated as an "avenue". Roads that have a 			definite east-west directional course should be designated 		as a "street".

C. 	Roads that do not have a definite directional course should 	be designated as a "road", "drive", "trail", "way", or 			"lane".

D. 	A dead-end road or cul-de-sac less than one thousand feet 		(1,000') in length, when not an extension of an existing 		road or a continuation of a proposed road, should be called 	a "court".

E. 	A road that has its ingress and egress on the same road 		should be designated a "loop".

F. 	A road that circles back upon itself should be designated as 	a "circle".

G. 	Special scenic routes or park drives may be designated 			"parkway" upon review and approval by the administrator.

H. 	If a road accesses three (3) or more properties, it shall be 	assigned a road name and the parcels shall be addressed in 		accordance with this chapter.

I. 	The naming of spur roads shall comply with subsection H of 		this section.

J. 	Duplicate road names are prohibited. Existing duplicated 		names shall be corrected in accordance with this chapter.

K. 	Road names which sound like other road names or which are 		not readily capable of easy pronunciation or which would be 	distasteful to a reasonable person are prohibited. (2025 		Code)

7-3-3: 	ADDRESSING PROCEDURES:

A. 	Address Grid: Addressing along the city roads shall be 
     based on the address grid established by the city.

B. 	General Procedures:

1. Address numbers will run consecutively to the north, 		south, east and west from the point of beginning.

2. All addresses shall be defined with a direction letter 		(N, S, E, W) following the address number, pursuant to the 		grid defined in subsection A of this section.

3. Even numbers shall appear on the south and east sides of 	roads and odd numbers on the north and west sides.

C. 	Addressing Roads:

1. If a building has a number of entrances, each serving a 		separate occupant, then the building shall be assigned 			individual addresses at each entrance.

2. A multi-family dwelling structure with one main entrance 	shall be assigned one number, with the owner of the 			structure responsible for providing a designated individual 	numbering of each unit before an address will be issued.

3. Mobile home parks shall be assigned one number for each 		lot within the park based on the addressing system 			coordinates. The owner of the mobile home park shall be 		responsible for providing designated individual numbering 
     of each mobile home space. Roads within the mobile home 
     park shall be signed according to subsection 7-3-5C of this 	chapter. Alphanumeric combinations shall not be used.

4. A diagonal or meandering road shall be assigned numbers 		depending upon the address baseline that it most favors.

5. Circle and loop road direction designations shall be 		determined by the road's predominant direction.

a. For circle roads, the numbering shall start at the 		
          intersection point of the road closest to the address 			     origination point and shall proceed in a clockwise 
          direction using a consecutive numerical order with the 
          odd/even numbers based on the starting point of the 
          circle road as if the road were straight.

b. For loop roads, the beginning of the road is 			
          designated by the closest intersection to the 
          origination point and increases numerically to that 
          point that is the farthest from the origination point.

6. Properties with structures, or bare land parcels, that 		have a driveway shall be assigned an address based on a 		primary access point on a named road or common driveway.

7. Properties that have multiple driveways shall have a 		designated primary access point designated by the property 		owner that will be used for the address assignment point. 
     If this primary access point is not designated by the 
     property owner, the city council shall make the official 			determination and designate a primary access point that 
     will be used for address assignment.

8. All public roads, private roads and common driveways 		shall be assigned a road name and all adjacent parcels 			and/or structures with driveways shall be addressed in 			accordance with this chapter.

9. Parcels accessed by a common right of way or easement 		where the rule of three (3) does not apply shall be 
     assigned a primary access point at the point where the 
     right of way or easement intersects a public road, private 
     road or common driveway centerline. This primary access 
     point will be used to assign an official address for each 
     adjacent parcel.

10. All plats produced as a result of subdivisions, short 		plats, planned unit developments, or other projects that 		require the establishment of primary access points from 		public or private roads for driveways shall have 				predesignated primary access points. These points shall be 		utilized to assign addresses to the associated parcels. In 		all cases, the primary access points shall be verified by 		the planning and building department prior to the receipt of 	a certificate of occupancy for new structures to determine 		the actual address. The property owner may be required to 		pay a fee to cover the costs incurred to provide this 			service. (2025 Code)

7-3-4: 	ROAD NAMING:

A. 	Road Naming Policy: In selecting road names, consideration 		shall be given to the following:

1. Centerline alignment road name standards should be 			observed for noncontinuous roads, unless there is no 			possibility for extension of the road to make it a 			continuous through road.

2. There shall be no duplication of existing names. The 
     city shall assemble, update and maintain an official list 
     of all road names throughout the city for use by all 
     jurisdictions.

3. Names of similar pronunciation and/or spelling shall be 		prohibited (example: Briar Lane, Brier Lane).

4. Variations of the same name with a different road 			designation shall be prohibited within the first word of 
     the two (2) word title or in the road extension (example: 
     Pine Road, Pine Drive, White Pine Road, White Lilly Lane).

5. No road name shall consist of more than two (2) words or 	contain more than fourteen (14) letters, excluding the road 	direction (N, S, E, W) and extension (street, lane, court, 		etc.).

6. No road shall have more than one name.

7. No road name shall contain the words "north", "south", 		"east" or "west", or any combination thereof, within the 		road name.

B. 	Naming Of New Roads Or Existing Unnamed Roads: Any new road 	to be established within city, public or private, or any 		existing unnamed road, public or private, shall require a 		road name approved by the city director.

1. In the case of plats, approved road names shall be 			specified on the final plat map.

2. In the case of other new roads or naming of unnamed 			roads, the owner or owners and/or contract buyers of 			properties abutting said road, may petition, in writing on a 	form provided by the city, to the city council, to request 		an official name for a new road. The council shall consider 
     any and all road name petitions, only after 	all road 
     naming requirements of this chapter are met. The council 
     shall officially designate the road name having the 		greatest percentage of approval in the event a fifty one 		percent (51%) approval is not obtained.

C. 	Renaming Of Existing Duplicated Road Names:

1. Where duplicate names exist, roads may be immediately 		renamed by the council to eliminate the duplication. The 		council shall decide which roads shall be renamed using the 	following criteria:

a. When the road was originally named.

b. The number of improved properties served by the 			     road.

c. Other factors as deemed appropriate by the city 			     council.

2. When a road name within the unincorporated area adjacent 	to the city is duplicated by a road name within the city, 		the city shall work with Jefferson or Bonneville County to 		decide which road shall be renamed. Roads shall be renamed 		in accordance with this chapter and similar ordinance 			provisions of Jefferson or Bonneville County. Consistency 
     is the goal.

D. 	Official Designation of Existing Road Names: With the 			adoption of this chapter, all roads named as a result of 
     the Jefferson or Bonneville County mapping and addressing 			project for enhanced 911 shall be designated as the 
     official road names. By resolution, the city council should 
     confirm the approved list of street and road names. 
     Subsequent adjustments and changes may be made by further 
     resolutions.

E. 	Renaming Of Other Roads: In cases where property owners 		request to change the name of a road which has an existing 		city approved name, the property owners may petition the 		city council for such a change. Property owner-initiated 		requests under this subsection shall require seventy five 		percent (75%) approval of property owners abutting the road 	and the payment of an applicable fee. Changes shall only be 	authorized when determined to be in the public interest. 		(2025 Code)

7-3-5: 	SIGNING:

A. 	Address Numbers: Physical address numbers shall be clearly 		readable from the roadway and shall contrast with 			background color pursuant to the international fire code. 
     If a structure is more than seventy-five feet (75') from 
     the road or is otherwise not clearly visible from the 
     road, its address shall be posted at the intersection of 
     its access road and public or private road. The address 
     sign shall be no less than four feet (4') or more than six 
     feet (6') above the ground on a substantial, maintained 
     post or standard. The view of the address from the road 
     must be unobstructed and maintained. All primary letters, 
     numbers and symbols shall be a minimum of four inches (4") 
     in height, with a one-half inch (1/2") stroke, and shall be 
     reflectorized and contrasting with the background color.

B. 	Signs: Road signs shall be placed between six feet (6') and 	eight feet (8') above the roadway, clearly visible at 			intersections. All primary letters, numbers and symbols 		shall be a minimum of six inches (6") in height, with a 		one-half inch (1/2") stroke, and shall be reflectorized and 		contrasting with the background color of the sign in 			accordance with the "Manual of Uniform Traffic Control 			Devices". Specifically, public road signs shall be green 		with white lettering. Private road signs shall be blue with 	white letters. All public and private road signs shall be 		constructed and installed to the standards of the street 		department and shall include the road name and address 			number derived from the grid. Private road signs shall be 		erected consistent with the construction standards or 			methods employed by the street department.

C. 	Road Sign Installation Standards: Proper positioning of 		signs is essential to obtain maximum safety, efficiency and 	observance. The "Manual for Uniform Traffic Control 
     Devices" (MUTCD) establishes such standards within the 
     city. The street department may require compliance with 
     standards for sign placement applicable to the department. 
     (2025 Code)

7-3-6: 	ADMINISTRATIVE PROCEDURES:  The city council may, by 			     resolution, adopt formal administrative procedures, create fee schedules, etc., to facilitate implementation of the purpose and intent of this chapter. The mayor is authorized to take such steps as may be necessary to implement the provisions of this chapter. (2025 Code)

7-3-7: 	APPEALS:  Appeals concerning interpretation or 				     administration of this chapter may be filed by any affected person. Such appeals shall be filed within a reasonable time of the action being appealed, not to exceed thirty (30) days, by filing with the city clerk-treasurer a notice of appeal specifying the grounds of the appeal. The clerk-treasurer shall transmit to the mayor all papers constituting the record upon which the action appealed was taken. The council shall schedule the item for a hearing to be commenced promptly after filing a completed notice of appeal and shall give due notice to the parties of interest. The mayor shall decide the matter as circumstances allow. The decision of the mayor may be appealed to the city council. Enforcement actions may not be appealed. (2025 Code)

7-3-8: 	CONTROVERSIAL OR DISPUTED ROAD NAMES:  The council 			     shall have the discretion to refer any disputed road name, addressing issues or controversial road name changes to the mayor for approval or resolution. (2025 Code)

7-3-9: 	VIOLATION; PENALTY:

A. 	Violation: It shall be unlawful for any person to:

1. Erect or install a street name sign not in accordance 		with this chapter;

2. Remove, alter, change or deface a street name sign or 		address identification erected or installed as provided 		herein;

3. Place or post addresses not approved by this chapter; or

4. Fail to place an address visible from the road and/or 		waterway.

B. 	Penalty: Any person who violates or fails to comply with 
     any of the provisions of this chapter shall be guilty of an 	infraction, and upon conviction thereof, shall be subject 
     to penalty as provided in section 1-4-1 of this code. (2025 	Code)

CHAPTER 4

DRINKING WATER REGULATIONS

SECTION:

7-4-1:	Definitions

7-4-2:	Ownership and Management of Water System

7-4-3:	Connection Required

7-4-4:	Water System and Fees Established

7-4-5:	Hookup Fee

7-4-6:	Water Capacity Buy-in Fee

7-4-7:	Monthly Service Fees

7-4-8:	Cross-Connection Control Program

7-4-9:	Water Service Lines

7-4-10:	Private Water Mainline Extensions

7-4-11:	Extension of City Water Mains

7-4-12:	Water System Design and Materials

7-4-13:	Subdivision Developer's or Property Owner

Responsibility

7-4-14:	Fire Hydrants

7-4-15:	Miscellaneous Provisions

7-4-16:	Liability for Damages

7-4-16:	Enforcement

Attachment:	Division 400 Supplement to the 2020 Edition  
               of the Idaho Standards for Public Works 
               Construction

ORDINANCE A-2021-01

BE IT ORDAINED BY THE MAYOR AND THE CITY COUNCIL OF THE CITY OF RIRIE, IDAHO:

An ordinance explaining and documenting the need for Capacity Fees for the City of Ririe, Idaho; establishing the dollar amounts of the water and sewer Ririe Water System Capacity Fees and Ririe Sewer System Capacity Fees; establishing the application of such fees; establishing a severance clause;

establishing an effective date.

WHEREAS:

In the last several years there has seen significant growth in Bonneville and Jefferson Counties, and the State of Idaho is ranked as one of the top states for growth in the U.S.; and,

The City of Ririe has likewise seen an uptick in real estate being sold and developed in the City and in its Ririe-Jefferson County Impact Area; and,

There has been a definable growth trend in the City of Ririe for development including the former school property of 14 lots, the Old Fairgrounds property being bought for potential development, the 50+ year old Mulberry Estates plat still in play, annexation of large parcels of property into the City, and developers recently working with the City Planner for submission of preliminary plats on two parcels, including a 40 acre parcel and a 67 acre parcel, for example. Such real and potential growth opportunities in Ririe and in its impact areas could easily double the number of residences in Ririe in the near future; and,

Ririe is a remote community with few if any social and medical services available to its residents. Because Ririe already has a preponderance of small residences, the City's Comprehensive Plan speaks to the need for a variety of housing, and this has been interpreted by both the City Planning and Zoning Commission and the Mayor and Council in their meetings to mean that larger residences and larger lots are desired as the focus for future development, and further this would be in accordance with the City's Comprehensive Plan; and,

The Ririe Comprehensive Plan duly adopted by the City and its residents stresses the desire to maintain a small town feel, and generally to stay a smaller city. The Ririe Comprehensive Plan also encourages some growth in commercial areas such as a hardware store, a medical clinic, and a grocery store. In order to see a moderate amount of growth in these three commercial endeavors, the City of Ririe recognizes a need to plan for and encourage a controlled and responsible amount of growth; and,

With the greatly increasing needs and commensurate increasing costs of infrastructure upgrades and maintenance as growth of the City occurs; and,

With the firm belief that growth should pay for itself and not be an additional tax and fees burden on existing residents who have already paid for the building and maintenance of the current Water System and the Sewer System (as established in open meetings); and,

The Ririe Mayor and City Council engaged the engineering services of Schiess and Associates, as the duly qualified and designated Ririe City engineering firm, Schiess & Associates in November 2019 to conduct the Equity Buy-In Study of the City's water and sewer infrastructure. Schiess & Associates was also involved in a concurrent Impact Fee Study concluded in May 2020; and,

The Ririe Impact Fee Advisory Committee was duly established and was engaged in the exploration of Impact Fees in Ririe, as required by Idaho State Law; and,

With the required hearings being conducted as required by Idaho State Laws; and,

Because of these foregoing reasons and others as discussed in open City Council meetings in the recent past, the City of Ririe adopts the following capacity fees:

Ririe Water System Capacity Buy-In Fee: $9,905.00

Ririe Sewer System Capacity Buy-In Fee: $5,918.00

These new capacity fees shall be paid to the City Clerk at the time of the building permit application and applies to:

1. Each and all new subdivision lots platted after the effective date;

2. Each and all existing lots or replated lots that did not have a City water system stub prior to the effective date shall be assessed the Ririe Water System Capacity Buy-In

Fee;

3. Each and all existing lots or replated lots that did not  
have a City sewer system stub prior to the effective date 
shall be assessed the Ririe Sewer System Capacity Buy-In 
Fee..

ADDITIONALLY,

1. The City reserves the right to place on hold / shut  
down any building or subdivision project due to 
non-payment of capacity fees.

2. All City ordinances including subdivision 
ordinances, public works ordinances, building and 
standards ordinances, as well as any development 
agreements and water rights purchases duly enacted 
with each subdivision developer, shall remain in   
effect and shall apply in addition to the new 
capacity fees.

3. City building permit fees, planning and zoning fees, 
and all other fees pertinent to building and 
developing in the City of Ririe shall remain in 
force.

4. To further the growth of the City, and only as 
deemed necessary and reasonable such as during an 
economic downfall or circumstances that would 
necessitate the accentuating of commercial growth, 
the Mayor and Council through a development 
agreement with a builder/developer may apply 
proportionally or in whole the City water hook up 
fee as a credit toward the Ririe Water System

Capacity Buy-In Fee, or proportionally or in whole the 
City sewer hookup fee as a credit toward the Ririe 
Sewer Capacity Buy-In Fee.

5. The Mayor and Council may through a development 
agreement with a developer/buyer proportionally or in 
whole reasonably offset the value of water rights 
against the Ririe Water System Capacity Buy-In Fees.

6. SEVERABILITY: This ordinance is severable, and in the 
event any portion of this ordinance is deemed 
unenforceable under the law for any reason the 
remaining portions of this ordinance not so affected 
shall remain in full force and effect.

This Ordinance shall take effect and be in force from and after its passage, approval and publication.

Dated this 3rd day of August, 2021

(Ord. A-2021-01, 8-3-2021)

7-4-1:	DEFINITIONS:

As used in this chapter, the following words and terms shall have the meanings ascribed to them in this section:

APARTMENT HOUSE:	Any building or portion thereof which is 
                    designed to be occupied or which is 
                    occupied as a home of three or more families 
				living independently of each other.

BASE RATE:		The fixed portion of the monthly service fee  
				charged to cover depreciation, amortization 
                    and debt service of the system, and 
                    generally the fixed costs of Operations and 
				Maintenance. This fee is based on an 
                    assessed number of user equivalents.

CAPACITY  		An equity-based fee calculation designed to

BUY-IN FEE:		place new utility users on an equal equity                                  
				share basis with existing users as allowed by 
				State law.

COMMERCIAL AND      Shall include any property, building or INDUSTRIAL 		structure used for any business activity, BUILDINGS:		trade, or profession; and shall include, but 
				not be limited to, garages, service 
                    stations, wholesale and retail stores, 
                    laundries, dry cleaners, restaurants, bars, 
                    barbershops, beauty shops, schools, 
                    churches, public buildings, and all other 
                    not specifically defined as residential 
                    buildings.

CONDOMINIUM:		Any building or complex of buildings which 
                    is designed to be owned and occupied as a 
                    home of four or more families living 
                    independently of each other where the 
                    condominium complex has a water service line 
                    and is organized as a condominium regime 
                    pursuant to Idaho Code title 55, Chapter 15. 
                    Each condominium unit shall be designed with 
                    a separate water service line to be 
                    connected to the City water mainline.

DRINKING WATER:	Water meeting the EPA Safe Drinking Water 
                    Act and State Department of Environmental 
                    Quality regulations that is distributed to 
                    City users and is deemed fit for human 
                    consumption by said regulatory entities.

DOPL, PLUMBING 	Division of Occupational Professional PROGRAM:			Licensing, Plumbing Program formerly the 
				Division of Building Safety, Plumbing 
                    Program

DUPLEX:			A building designed to be, or which is 
				occupied by two (2) independent family 
                    units. This includes single family 
                    residences with a separate apartment.

HOOKUP FEE:		The hookup fee is a fee made by the City to 
				permit the connection to the City drinking 
				water system via a water service line and 
				meter.

MOBILE HOME 		Any area or site of land upon which two or PARK:			more mobile homes are placed and maintained 
				for dwelling purposes only, either on a 
				permanent or semi-permanent basis.

MONTHLY SERVICE 	A fee made by the City to owners of

FEE:				residential, commercial, or industrial 
				buildings, for water service to said 
				buildings. This fee is the sum of the base 
				rate and use rate.

MOTEL:			A building or group of buildings on the same 
				premises, either detached or in connected 
				rows, containing sleeping or dwelling units, 
				but without kitchen facilities in the 
				individual units, and designed for or 
				occupied by travelers. A motel with kitchen 
				facilities in the individual units will be 
				considered as an apartment house. A motel 
				with a commercial building on the same 
				premises, both of which are served through 
				one service connection, will be considered 
                    as a commercial building.

RESIDENTIAL 		Shall include single-family residences BUILDING:			and duplexes for the purpose of 
				establishing water service charges.

SATISFACTORY 		A. A person who has held a water account in

WATER ACCOUNT		his/her name with the City for the past six 
CREDIT RATING:		consecutive months within which the water

associated charges for each month were 
				paid in full on or before the tenth 
				day of the following month, or

B. That a person who closed a water 
				account with the City within the past 
				six months did, at the time of closing 
				the account, possess the same good 
				record of paying on time as described 
				in subsection A of this definition.

SERVICE 			The physical connection of a water service CONNECTION:		line to the water mainline. A service 
				connection is usually located within the 
				public right of way or easement. A service 
				connection may be on private property when 
				the property owner has more than one service 
				connection and the service connections 
                    attach to a private mainline which then 
                    enters the public street right-of-way and 
                    connects to a City water mainline.

SINGLE-FAMILY 		Any building designed to be utilized as a RESIDENCE:		residence for one family. This definition 
				shall include a single manufactured or 
                    mobile home which is not located on a rented 
                    space in a mobile home park.

USER EQUIVALENT:	One residential user equivalent is 
                    designated as the service to a typical 
                    single-family dwelling. All other 
                    connections are prorated in relation to 
                    residential user equivalents based on the 
                    estimated usage of or benefit derived from 
                    the service. The user equivalent shall be 
                    based on the volume of drinking water 
                    consumed by the average home in year.

An assessment of user equivalent is 
				made at the time of application for 
				service is made and is based upon 
				the user equivalent schedule herein.

USE RATE:			The variable cost portion of the monthly 
				service fee which consists generally of the 
				variable portion of operations and 
				maintenance costs.

WATER			Any water line that delivers water to a MAINLINE:			service line. These may be City owned or 
				privately owned if on private property.

WATER SERVICE 		The section of piping that delivers water LINE:			from the main distribution line to the 
                    meter, then into the building of the user.

WATER SYSTEM:		The drinking water system of the City. It is 
				a Class I community drinking water system as 
				defined by the USEPA and Idaho DEQ.

7-4-2:	OWNERSHIP AND MANAGEMENT OF WATER SYSTEM:

Through the enactment of ordinances and council resolutions, the City council shall exercise policy making control over the City's municipal water system. The mayor shall administer the day-to-day operation of said system within the policy guidelines established by the council by ordinance and resolution. Regardless of who shall have borne the cost of installing any part of the City water system, that part, once installed and approved by and accepted by the City council, shall become the property of the City, subject to any rights of guarantee accompanying such decision.

7-4-3:	CONNECTION REQUIRED:

A. Connection to System Required: Every parcel of  
    land or premises within the boundaries of the 
    City, improved for occupancy and occupied or  
    used by any person or persons, or as a 
    commercial business, shall be connected to the 
    public water system. The owner or person in 
    charge of such land shall make or cause to be 
    made such connection within one hundred twenty 
    (120) days after receiving official notice from 
    the City to connect.

B. Dwelling Without Water Service Declared Health 
    Hazard and Public Nuisance: The human habitation 
    of any residence or use of any other building or 
    structure for purposes other than warehousing or 
    storage of nonperishable goods or commodities 
    where no workers regularly labor without a ready 
    supply of running potable water, whether from the 
    City system or other authorized source, inside of 
    such dwelling or other structure shall constitute 
    a health hazard and a public nuisance. In such 
    circumstances, the City may case proceedings to 
    be brought for the abatement of the occupancy or 
    use of such a place.

C.  Habitation of Dwelling Without Water Service Unlawful: 
    The human habitation of any residential dwelling or 
    other building or structure without a required source 
    of running potable water as required by this chapter 
    shall be unlawful and subject to penalty as provided 
    in Section 7-4-15 of this chapter.

7-4-4:  WATER SYSTEM FEES ESTABLISHED:

A.  Establishment: It is necessary to protect the 
	health, safety and welfare of the City inhabitants 
	and to provide equitable fees upon all lots, lands, 
	property and premises served by the drinking water 
	system. Said system and facilities consist 
	generally of water mainlines, valves, hydrants, 
	service lines and water meters.

B.   Availability: The City will make the service of the 
     system available within its capacity to all persons 
     in the City service area without discrimination as 
     to race, color, religion or national origin.

The City water service area is currently confined   
     to those areas within the City corporate limits 
     with limited service allowed outside of the 
     corporate limit. Should service be extended 
     elsewhere within or outside of the City limits, 
     fees for well pumping, storage, booster pumping, 	   
     maintenance, debt service, and equipment 		
     replacement will be charged on the same basis 	
     as City residents with service.

C.  Open Account Required Prior to Drawing Water: 
     Whenever a water hookup service has been installed 
     to provide City water service to a property, no 
     water shall be drawn through that hookup by an 
     owner, tenant or guest of either if there is not 
     currently on file with the City clerk-treasurer an 
     open water account for that service. The fact that 
     water service may be on at the time an owner, 
     tenant or guest of either should move onto a 
     property shall not relieve such person from the 
     requirements of this section. The City may, without 
     further notice, shut off any service which is used 
     without establishing an account. Any person using 
     water from the City system without establishing an 
     account shall be in violation of this Chapter.

D.  System Fees: A system of fees for access to and 
     the use of the drinking water supply and 
     distribution system is necessary. Those fees will 
     differentiate among the different types of users of 
     the system to ensure equitable fees for the various 
     categories. The fees from each user shall be used 
     for the operation and maintenance of the system and 
     for capital improvements to replace assets that 
     reach the end of their useful lives. Fees shall be 
     proportional to the estimated or measured flow 
     contribution of each user of the system. Similar 
     uses shall be treated in a like manner. Fees 
     assessed by the City shall be reviewed periodically 
     and may be revised as needed by resolution of the 
     City council to address operation and maintenance 
     costs and capital improvements needs.

E.  Each User Equivalent to Pay Fees: Fees will be 
     assessed to each user equivalent served by the 
     water service line and mainline connection. Partial 
     user equivalents are allowed if the assessment of 
     user equivalents is not a whole number.

F.   Fees Paid in Full at Building Permit Issuance: The 
     hookup fee and the water capacity buy­ in fee shall 
     be paid at the time of the issuance of a building 
     permit. No building permit shall be issued until 
     such fees are paid in full.

G.   Turn Off and Turn On Fees: Any time that the water 
     is either turned on or shut off at the meter by the 
     City staff due to a request or actions of the 
     customer, the customer shall pay a turn off or turn 
     on fee in an amount set by resolution of the 
     council.

7-4-5:	WATER HOOKUP FEE:

A.  Hookup Fee: The hookup fee for all residential, 
     public, commercial and industrial buildings shall 
     be established by resolution and subject to 
     adjustments from time to time by the City council. 
     Hookup fees are to be deposited into the water 
     fund.

B.  The purpose of a hookup fee is to compensate the 
     City for administrative and clerical effort to 
     setup new account, inspect the physical connection 
     of the water service line to the City water 
     mainline, and reimburse the City for the meter and 
     any materials provided to the account applicant.

C.   Deposit May be Required:

1. All new customers and any customer who does not 
     currently possess a satisfactory water credit 
     rating with the City shall, prior to a water 
     account being established or continued in his/her 
     name, pay a deposit in an amount to be set by 
     resolution of the City council.

1. Applied to Account Balance: Any customer having a water 
     deposit on file with the City shall, upon acquiring a 
     satisfactory water credit rating, be entitled to have the 
     deposit applied to payment on the account. However, if a 
     satisfactory water credit rating should subsequently be 
     lost because of delinquencies of an account, another 
     deposit, accordance with Subsection A.1 of this Section, 
     shall due and payable to the City immediately. If said

deposit not paid within ten days, the account shall be 
     considered delinquent and shall be processed in accordance 
     with Subsection 7-4-7.A.4 of this Chapter.

7-4-6:	WATER CAPACITY BUY-IN FEE

A. The City Council adopted Ordinance A-2021-01 
     implementing a capacity buy-in fee. The fee is 
     designed to achieve a fair equity position between 
     each new user equivalent of the water system and 
     existing users. This fee is assessed to each new 
     residential user equivalent.

B.  The collected water capacity buy-in fees are kept  
     in a reserve account known as the water 
     capitalization reinvestment account. The revenue 
     shall be used for the purpose of funding water   
     supply wells, storage tanks, booster pumping and 
     distribution improvements.

7-4-7:	MONTHLY SERVICE FEE:

A. Minimum Monthly Service Fee: The City 
     clerk-treasurer shall furnish to each property 
     owner, or agent, a monthly statement of the amount 
     due for water service for that month. The failure 
     of any user to receive notice or statement will in 
     no way affect the obligation of the user to remit 
     monthly as required.

1. Basis: Monthly service fees will be computed using a base rate plus a variable use rate. The base rate shall be determined using the residential equivalent user system initially set up by determining the estimated number of user equivalents according to the schedule herein. The base rate and declining block variable use rate determined by reading the water meters has been established by resolution and will be subject to periodic change by the City council.

2. Purpose: The purpose of the monthly service fee    
is to pay for system maintenance and operation, 
including depreciation (for asset replacement), 
debt service and loan reserves of the water 
system.

3. Monthly Billing and Payment: Meters shall be read, 
and bills shall be rendered, as nearly as 
practicable, at monthly intervals for all service 
supplied during the preceding month. The monthly 
water billing will be included in conjunction with 
the monthly sewer bill. Irrespective of whether a 
billing statement has been received, all City 
utility accounts are due the tenth day of each 
month. Any water associated charges accrued during 
any month shall be deemed past due after the tenth 
day of the succeeding month.

4. Winter Months: During winter months, water will

be charged based on the last reading before snowfall.  Any adjustments will be made when the meters are read the following spring.

B.   Vacation or Shutdown Rates: When a person, firm, or 
     business, determines that his home, business, etc., 
     is going to be vacant, not used, or shut down for an 
     extended period of time which exceeds one month, he 
     (or they) may notify the City clerk-treasurer, who 
     is empowered to decrease the monthly water rate for 
     said home, business, etc., to one-half (½) its base 
     rate for the time that said home, business, etc., 
     is actually not being used or operated. This will 
     be done only after the City official has shut off 
     the service. A shutoff fee will be charged for this 
     permanent or semi-permanent discontinuance of water 
     service. A turn on fee will subsequently be charged 
     when it is requested that water service be 
     reestablished to said home, business, etc.

C.   Delinquent Accounts:

- Delinquent Account: An account becomes delinquent 
when it has gone into the subsequent billing cycle 
without full payment. Delinquent accounts shall be 
assessed a late charge, per unit, in an amount to  
be set by resolution of the City council. Whenever an account becomes delinquent, the City clerk-
treasurer shall notify any tenant and landowner of 
the property concerned by U.S. mail or physical 
residential tagging, that unless a payment in the 
amount determined by the most recent bill is 
received by the City within seven days after the 
issuance of said notice, water service to the 
property may be discontinued. In determining the 
amount due, the cost of preparation of the 
delinquent notice, as prescribed by resolution of 
the City council, shall first be added to the 
account balance.

2. Shut Off Delinquent Account: In the event that the 
notice issued in accordance with Subsection C.1 of 
this Section does not result in payment of the 
bill or the filing of an appeal, then, on the day 
following the expiration of the time limit for the 
City's receipt of such payment, the City may 
disconnect the service by closing the valve at the 
meter. When the delinquent amount reaches a fixed 
amount set by resolution, the meter will be 
removed, the shut-off valve closed and the service 
will be considered disconnected.

3. Appeal: Appeals shall be heard by the mayor or the 
mayor's designee and the decision shall be final.

D.        D.  Reinstallation Fee: The property owner is required 
     to pay the City a reinstallation fee plus the cost 
     of the meter, parts and any labor associated with 
     the installation before acceptance of payment of a 
     hookup fee to reinitiate service. All back charges 
     and fees shall be paid in full before 
     reinstallation occurs. The reinstallation fee will 
     be set forth by resolution.

E.   Property Owner Ultimately Liable for Bills and Damage:

1. All water service accounts shall be opened and 
maintained by the owner of record or contract 
purchaser of record of the property to be served. 
Such owner or purchaser shall agree, as a 
condition of opening any water service account, to 
be liable for any service or other charges imposed 
because of such water services, whether such 
charges are the consequence of use of the water or 
provision of service by the owner or by tenants 
who have agreed to pay utility charges.

2. The record owner may allow a tenant to pay utility 
charges while the tenant is in possession of 
rented premises, but such action shall not relieve 
the owner of the ultimate responsibility for the 
charges accrued to the account.

3. Unpaid water bills and water associated services 
shall be the ultimate obligation of the owner of 
the property served. Any property receiving 
service from the City water system may be 
disconnected if said services are not paid for as 
required herein. An owner may allow a tenant to 
pay utility charges while the tenant is in 
possession of rented premises, but such action 
shall not relieve the owner of the ultimate 
responsibility for the charges accruing to the 
account.

4. The City's cost of repairing or replacing any 
meter, shutoff valve or other related part of the 
water system having been damaged by any person 
other than a City officer or employee shall be a 
legal and financial obligation of the owner of the 
property. Owners are responsible for the acts of 
their tenants, contractors or permissive users.

F.     F.  Multiple Users: Where a residence, apartment(s), 
     retail store or other commercial establishment is 
     provided water service in conjunction with another 
     establishment in the same building, the minimum user 
     equivalent rating for that building shall be 
     determined based on the number of establishments 
     contained therein. (For example, a retail store with 
     a residence attached would be rated at a minimum of 
     2 residential user equivalents. A commercial/office 
     building with 3 apartments would be rated at 4 user 
     equivalents, minimum.)

G.         G.  Schedule Of User Equivalents/Charges: The following 
     schedule lists a number of users and the number of  
     user equivalents assigned to each user. All other 
     users shall require special appraisal by the council 
     and will be computed on an individual basis:

SCHEDULE OF USER EQUIVALENTS/CHARGES

Number of User

Classifications                                                                                                                Equivalents

| Apartments (see Multiple living unit) | - |
| --- | --- |
| Auto Garage (per 3,000 square feet) | 1 |
| Bar (per 10 seats or fraction thereof) | 1 |
| Barbershop, per chair (minimum 1.00) | .50 |
| Beauty salon, per operator station (minimum 1.00) | 1 |
| Car wash (per bay minimum or based on water usage) | 2 |
| Church | 1 |
| Condominium (see Multiple living unit) | - |
| Dining Hall (Senior Citizens Center per 2 persons) | 1 |
| Doctor or Dentist (per practitioner) | 2 |
| Duplex (see Multiple living unit) | - |
| Laundromats: Self-service (up to and including 12 washing machines) (Each machine in excess of 12) | 5 .40 |
| Commercial (to be computed on an individual basis) | TBD |
| Lodge, per 3,000 square feet | 2 |
| Mobile Home Court/Park, per space | 1 |
| Mobile Home or Trailer House on Own Premises | 1 |
| Mobile Home or Trailer House on Own Premises | 1 |
| Motel, Hotel, Rooming House, etc.: First Unit Each Additional Bed Space | 1 .25 |
| Multiple Living Unit: Each Unit | 1 |
| Office Building (for each 2,500 square feet of gross floor space or fraction thereof) | 1 |
| Recreational Vehicle Hookup in RV Park | .50 |
| Restaurant (10 seats or fraction thereof) | 2 |
| Retail or Business Establishment (for each 3,000 square feet of gross floor space or fraction thereof) | 1 |
| Schools: With Cafeteria & Shower (students & staff) With Cafeteria Only (students & staff) Without Cafeteria & Showers | .10 .09 .08 |
| Service Station (with restroom service) | 2 |
| Single-Family Residence | 1 |
| Theater (50 seats) | 1 |

The above user fees are based upon typical monthly sewer usage as contemplated by standards set forth herein. Such standards shall be subject to adjustment from time to time by the city council. Any such change shall be communicated to the sewer user before it is imposed. And such user shall be provided an opportunity to be heard before the city council if said user contests the additional user fee.

H.  Voluntary Discontinuance: When water services are  
     voluntarily discontinued by request of the owner, 
     the base rate portion of the monthly service fee 
     shall continue to be paid. Full water service fees 
     will resume immediately upon water service turn back 
     on.

I.  Industrial Users Not on Schedule: Industrial users 
     or other businesses not categorized above or not 
     clearly defined as being within one or more of the 
     above classifications shall be charged at a rate to 
     be determined by the mayor, subject to appeal to 
     the City council, upon application of the property 
     owner, after considering all relevant evidence 
     pertaining thereto at a public hearing held for such 
     purpose. The rate shall be established based upon 
     consideration of the nature and intensity of the 
     proposed use and total impact to the City drinking 
     water system. The fee shall be proportionately 
     consistent with the schedule set forth herein. The 
     rationale for charging any such fee shall be 
     explained, in writing, upon request by the user.

J.   Aggrieved User; Installation of Meter:

1. Should any user consider himself to be aggrieved by the foregoing schedule, or by the determination of the mayor and City council, such user may, at his own expense, verify the water usage, in which event the charge shall be determined based upon verified results. Such meter shall be calibrated and monitored to the satisfaction of the contract City engineer. Reliable technical data shall support any rate determination.

2. The City may at its own digression from 
     time-to-time conduct a flow/revenue study wherein 
     the amount of monthly fees paid by the average 
     residential user for the previous full fiscal year 
     for the associated water volume be compared to the 
     revenue and flow of each commercial/institutional 
     users revenues and flows to verify the accuracy of 
     assigned user equivalents. Future fees billed and 
     paid	will be adjusted to calculated flow-based 
     user equivalents. The City nor the users have the 
     right to require the other to retroactively correct 
     past fees billed and paid to be in conformance with 
     study findings of actual user equivalents.

7-4-8:  CROSS-CONNECTION CONTROL PROGRAM

A.  The City is responsible to take reasonable and 
     prudent measures to protect the water system 
     against contamination and pollution from cross 
     connections through premises isolation, internal or 
     in-plant isolation, fixture protection, or some 
     combination of premises isolation, internal 
     isolation, and fixture protection in accordance 
     with the Idaho Drinking Water Rules (IDAPA 
     58.01.08.552.06).

B.   Inspection Program:

1. The property owner remains responsible to   
maintain their own backflow prevention devises.

2. The City, at its own discretion, may locate 
     cross connections and determine required suitable 
     protection for a property owner. The City will 
     inspect and may even test a property owner device 
     for compliance. This will typically be done 
     through random sampling or whenever there is 
     suspicion of a cross-connection.

C.   New Connection: For new connections, including lawn 
     sprinkler system, suitable protection must be 
     installed prior to providing water service.

1.The property owner shall place the backflow   
     prevention device in a utility easement 
     accessible to the City or provide access via 
     implied easement to enable City to implement this     
     Cross-Connection Control Program.

2. Lawn Sprinkler Systems: Property owner shall 
     have all lawn sprinkler systems installed with a 
     backflow prevention device to protect against 
     contamination of the City water supply. All systems 
     shall be inspected by the state DOPL, Plumbing 
     Program inspector.

D.  Property Owner to Pay Costs of Materials and 
     Installation: Property owner is responsible for the 
     costs of backflow prevention materials and 
     installation and for state DOPL, Plumbing Program 
     permitting fees.

E. Appropriate and adequate backflow prevention 
     assembly types for various facilities, fixtures, 
     equipment, and uses of water should be selected 
     from the AWWA Pacific Northwest Section Cross 
     Connection Control Manual, the Uniform Plumbing 
     Code, the AWWA Recommended Practice for Backflow 
     Prevention and Cross Connection Control (M14), the 
     USC Foundation Manual of Cross Connection Control, 
     or other sources deemed acceptable by the 
     Department. The assemblies must meet the 
     requirements of Section 543 of the Idaho Drinking 
     Water Rules.

F.  Inspections and testing of all installed backflow 
     prevention assemblies is to be done by a tester 
     licensed by a licensing authority recognized by the 
     Idaho Department of Environmental Quality. Testing 
     shall be done in accordance with the test 
     procedures published by the University of Southern 
     California Foundation for Cross-Connection Control 
     and Hydraulic Research. See the USC Foundation 
     Manual of Cross-Connection Control referenced in 
     Subsection 002.02.

G.  Assemblies that fail testing or those found to be 
     defective shall be repaired, replaced, or isolated 
     within ten business days by the property owner at 
     his/her own expense. If the failed assembly cannot 
     be repaired, replaced, or isolated within ten 
     business days, water service to the failed assembly 
     shall be discontinued.

H.  The City will terminate service to any structure, 
     facility, or premises where suitable backflow 
     protection has not been provided for a cross 
     connection. Violators will be subject to penalty as 
     described in 7-4-17 of these regulations.

7-4-9:  WATER SERVICE LINES, METER BOXES and METERS:

A. Compliance Required: All water service line 
     construction shall conform to City standard 
     drawings and specifications, and the requirements 
     of the current edition of plumbing laws, rules and 
     regulations of the State DOPL, Plumbing Program.

B.  Meters: Each service line shall have a meter box 
     and meter. The City will own and maintain all water 
     meters and boxes where located along City streets 
     or in platted easements. The City retains an 
     implied unwritten easement to access these meters 
     and boxes near City Street rights-of-way yet lie 
     within private property. When water meters and 
     boxes are located within private property (as in 
     the case of apartments, townhouses or multi-family 
     housing) the City will then only own and maintain 
     the meter. The City will not pay rent or any other 
     charge for meters or other water facilities, 
     including meter boxes and connections on a 
     customer's premises.

  - Single-Family Residence: Each standard service connection is a three-fourths inch (3/4") or a one-inch (1") meter and shall service one residence only.

  - Other Buildings or Structures: Whenever a service connection larger than a three-fourths inch (3/4") or a one-inch (1") meter is required or the connection is to any building or for any use that is not a permanent family residence, it is considered a commercial service, except as provided in Subsection B of this Section. A three-fourths inch (3/4") meter that services a building used for commercial activity shall also be considered a commercial service. The hookup fees shall be set by resolution of the City council.

  - Installation: Installation of water meters shall be performed only by authorized employees of the City. All meters shall be sealed by the City at the time of installation and no seal shall be altered or broken except by one of its authorized employees.

  - Payment of Meters: All meters shall be paid for by the customer requesting or receiving City water service. Other City provided materials (service connection saddle, corporation stop, water service pipe and meter box) are also to be paid for by the customer at the same time as the meter. These materials shall be paid for as part of the hookup fee.

5.  Location of Meters: Meters (and the meter box) shall normally be placed at the property line. The meter will be installed wherever the applicant desires, within reason. Driveways or other locations surrounded by pavement are allowed with traffic-rated rings and covers.

6.  Joint Use of Meters: The joining of several customers to take advantage of the single minimum charges and large quantity rates shall be prohibited.

7.  Multiple Meters: Where an individual consumer is supplied with water through more than one metered service, charges shall be computed separately for each individual meter.

8.  Meters with Remote Readout: Whenever necessary to assure ready access to meter supplied information, the City may require the installation of a water meter with a remote, electrically powered readout, said readout to be placed outside of a structure in a manner and location readily visible to a meter reader. Whenever such installation shall be required, the costs thereof shall be borne by the landowner.

9. Meters for New Subdivision Lots: The developer will provide an idler in lieu of a meter in the developer installed meter box. The owner of the property will then purchase the water meter at the time service is requested and payment of the hookup fee is made. The City will then replace the idler with the meter once the billing account is set up. The City may install a lock on the shut-of valve inside the meter box after installation to prevent unauthorized use of water.

C. Contractor Installation Qualifications: The 
     installation of the service connection to the water 
     mainline is to be done by one holding an 
     appropriate state public works license. The 
     installation of the water service line is to be 
     done by a state licensed plumber authorized to 
     perform the work. The jurisdictional division of 
     the City and State DOPL, Plumbing Program is 
     outlined in the Memorandum of Understanding between 
     the Idaho Division of Building Safety (now DOPL), 
     Plumbing Program and the Idaho Department of 
     Environmental Quality.

D.   Permits and Inspections Required:

1. Water Service Connection Permit: A property owner desiring to obtain a water service connection to a City owned water mainline shall file a request with the City. Such request shall be in sufficient detail as to adequately inform the City as to the proposed location of the desired water service line and connection point and the nature of the anticipated use. At the time of approval of the permit request, but before the building permit is issued, the property owner shall pay to the City the hookup fees and capacity buy-in fees which shall be established and adjusted from time to time by council resolution. The property owner shall also obtain a date for inspection by the City of the construction and physical connection to the City water mainline.

2. Right-of-Way Encroachment Permit: No water service line installation shall be done in the City right-of-way without first obtaining a City Right-of-Way Encroachment Permit and the state on-call utility locate service has been notified and field locates made.

3. Extension Prohibited Without Permission: It shall be unlawful for any person to extend any water service line beyond the limits of the building for which a permit has been given without obtaining permission for the desired extension and appropriate hookup fees and capacity buy-in fees are paid.

4. If the water meter box is found to have been installed within the property boundary of the landowner, but without a documented easement, the property owner shall be deemed to have granted an easement across his/her property to allow the City access to the water meter. If this implied easement is not satisfactory to the landowner, it will be the landowner's financial responsibility to have the service connection and water meter moved to a location where the City has documented access rights to the water meter. In addition to such costs, the property owner is responsible for all costs of reinstallation of any private water service line made necessary by the relocation of any meter as provided by this Section. If such implied easement is not acknowledged and accepted by the property owner, or if the property refuses to pay for the relocation of a meter so affected, water service to the premises may be terminated after notice to the property owner until such permission of water meter movement has been accomplished. No person shall receive water from the City water system without providing ready access to the water meter to allow reading and maintenance thereof.

  - Inspection at Time of Connection: In accordance with permit requirements, no backfill of the water service line connection to the mainline shall occur without City inspection and approval of the materials and workmanship of the connection. Should a property owner or his duly authorized agent connect water service line to the City mainline without said inspection and approval, the property owner will be required, at his sole expense, to uncover the newly connected water service line works so that proper inspection can occur.

D.   Trench Excavation, Backfill and Surface Restoration: 
     Any trench excavation, backfill and surface 
     restoration required shall conform to the City 
     standard drawings and specifications, or the county 
     jurisdiction, or the State of Idaho Transportation 
     Department as applicable.

E.  Cost of Installation Borne by Private Property 
     Owner: The water service line, meter, meter box,

connective joints, shut-off valves and the 
     connection to the City water mainline shall be 
     furnished and installed by and at the sole expense 
     of the property owner.

F.   Service Lines and Connections Requirements:

  - Location: Service lines shall meet all minimum vertical and horizontal separation distances required from sewer service lines, sewer mainline, pressure sewer lines and any non-potable fluid containing line in accordance with the Idaho Drinking Water Rules (IDAPA 58.01.08) as administered by the Idaho Department of Environmental Quality.

  - Sleeves are required on water service lines laid within 18 inches over/under sewer mainlines, sewer service lines or any non-potable fluid containing line up to the building.

  - Service Connections to Mainline: Service to two or more buildings through one water service line is prohibited. All service line connections must be made directly into a water mainline.

  - Trench Excavation, Backfill and Surface Restoration: Any trench excavation, backfill and surface restoration required shall conform to the City standard drawings and specifications, or the county jurisdiction, or the State of Idaho Transportation Department as applicable.

G.  Maintenance: All water users shall keep their 
     individual water service lines in good repair and 
     good condition. Water users shall also be 
     responsible for all costs associated with 
     maintaining the entire service line from the 
     building up to the City water meter, unless the 
     water meter is located within private property. In 
     this case the City is responsible for only that 
     part of the service line within the City street 
     right-of-way.

7-4-10:  PRIVATE WATER MAINLINE EXTENSIONS:

A.   Water Design, Plans and Specifications:

1. Compliance with Standards: All construction or reconstruction of private water mainlines to City owned mainlines shall be in accordance with the City standard drawings and specifications, and subject to inspection by the City.

2. Plan and Profile Drawings: Plan and profile drawings shall be prepared for all new water mains (whether public or private) and for all water mainline extensions, reconstructions, or renovations; and all such plan and profile drawings must be reviewed by the contract City engineer and approved by the City council and the Idaho Department of Environmental Quality before any construction work on said water lines is started per Idaho Code 39-118 or its successor.

3. Private Water Mainline Extension: A private mainline extension shall be sized to meet water supply needs, including fire sprinkler flow and outside irrigation. The property owner requesting a water mainline extension shall submit engineered drawings for City review and approval. The property owner shall be responsible for the City engineering review fees incurred to review the water line drawings as billed by the City.

4. Extension Prohibited Without Permission: It shall be unlawful for any person to extend any water mainline beyond the limits of the building for which a permit has been given without obtaining permission for the desired extension.

B. Contractor Installation Qualifications: The 
     installation of the mainline connection to the City 
     owned water mainline is to be done by one holding 
     an appropriate state public works license. The 
     installation of the water service line is to be 
     done by a state licensed plumber authorized to 
     perform the work. The jurisdictional division of 
     the City and State DOPL, Plumbing Program is 
     outlined in the Memorandum of Understanding between 
     the Idaho Division of Building Safety (now DOPL), 
     Plumbing Program and the Idaho Department of 
     Environmental Quality.

C.   Permits and Inspections Required:

1. Mainline Connection Permit: A property owner desiring to attach a private water mainline to the City owned mainline shall file a request with the City. Such request shall be in sufficient detail as to adequately inform the City as to the proposed location of the desired water mainline and mainline connection point. The property owner shall also obtain a date for inspection by the City of the construction. City review fees of plans and specifications must be paid at the time permit application is made.

2. Right-of-Way Encroachment Permit: No private mainline improvement work shall be done in the City right-of-way without first obtaining a City Right-of-Way Encroachment Permit and the state one-call utility locate service has been notified and field locates made.

3. Inspection at Time of Connection: In accordance with permit requirements, no backfill of the private water mainline and its connection to the City mainline should occur without City inspection and approval of the materials and workmanship of the connection. Should a property owner or his duly authorized agent connect private mainlines to the City mainline without said inspection and approval, the property owner will be required, at his sole expense, to uncover the newly connected water line works so that proper inspection can occur.

D. Trench Excavation, Backfill and Surface 
     Restoration: Any trench excavation, backfill and 
     surface restoration required shall conform to the 
     City standard drawings and specifications, or the 
     county jurisdiction, or the State of Idaho 
     Transportation Department as applicable.

E.   Cost of Installation Borne by Property Owner: The 
     private mainline connection to the City water 
     mainline and private mainlines installed on private 
     property shall be furnished and installed by and at 
     the sole expense of the property owner.

F.   Private Water Mainline Maintenance: Once the water 
     mainline connection to City owned mainline has been 
     completed and properly inspected to the City's 
     satisfaction, the City will own, operate and 
     maintain the portion of the private mainline 
     installed in the City street right­ of-way after the 
     contractors one year warranty period has expired. 
     The property owner shall own, operate and maintain 
     all water mainlines on private property installed 
     for his/her exclusive use.

7-4-11:  EXTENSIONS OF CITY WATER MAINS:

A.   Areas Within City: The City may construct, or cause 
     to be constructed, extensions to the water system 
     if one of the following conditions exists:

1. The annual anticipated revenue made available to the City from the customers to be immediately served by such an extension is not less than one fiftieth (1/50) of the actual cost to the City for the construction of said extension.

2. The City declares that the water main extension is needed for the overall benefit and improvement of the entire water distribution system.

3. The City council approves a contract for the construction of the extension. However, subdividers or developers within or adjacent to the City shall construct, at their own expense, all water mains within their subdivisions or developments.

A.    Areas Contiguous to City: In areas contiguous to the City that may be annexed to the City, the City 
may require such areas to be annexed before water 
service is provided. Prior to annexation, the City 
shall require the execution and approval of an 
annexation agreement which shall detail the 
conditions for water service.

C.   Areas Outside City: Water lines shall not, without 
     the City council's approval, be extended outside 
     the corporate limits of the City; however, when, in 
     the opinion of the City, there will be special 
     benefit and improvement to the City by reason of an 
     extension outside the City, special arrangements 
     may be made with the council to allow said 
     extension; provided, however, that the areas to be 
     serviced outside of the City shall not benefit to a 
     greater extent than like areas provided with water 
     service within the City; and provided further, that 
     no such arrangement shall be made unless adequate 
     water supply and water line capacity is available 
     for such outside the City water services.

7-4-12:  WATER SYSTEM DESIGN AND MATERIALS:

A.   Materials And Installations:

  - All water mainline pipe, valves, hydrants, service connections, service lines, meter boxes and their installation shall be in accordance with City standard drawings and specifications for the construction of water distribution system infrastructure.

  - All water service line materials and their installation shall be in accordance with the City standard drawings and specifications for construction of water distribution improvements.

B.   Design and Approvals:

1. The plans for all water mainlines, valves,   
hydrants, service connections, service lines and 
meter boxes shall be designed, signed, and sealed 
by a registered professional engineer licensed in 
the state of Idaho and reviewed by the City contract engineer and approved in writing by the City and the Idaho Department of Environmental Quality in accordance with Idaho Code 39-118 or its successor.

2. All water lines, valves, hydrants, service 
  connections, service lines and meter boxes shall be 
  designed in accordance with the Idaho Drinking 
  Water Rules (IDAPA 58.01.08).

3. Minimum water main size shall be six-inch diameter. Larger sizes shall be as required by the City's overall water system master plan or as are needed to provide adequate water distribution capacity for the area (and surrounding developable area) being developed and/or annexed to the City.

C.   Standard Drawings and Specifications - Adoption of 
     ISPWC: The City of Ririe Standard Drawings and 
     Specifications is the 2020 edition of the Idaho 
     Standards for Public Works Construction (ISPWC) and 
     the supplemental modifications attached at the end 
     of This ordinance pertaining to water 
     infrastructure labeled: Division 400 Supplement to 
     the 2020 Edition of the Idaho Standards for Public 
     Works Construction. In the event of a conflict 
     between the ISPWC and the City of Ririe Division 
     400 Supplement to the 2020 ISPWC, the City of Ririe 
     Division 400 Supplement to the 2020 ISPWC shall 
     govern unless the contrary is approved in writing 
     by the City for a specific circumstance as 
     recommended by the City's contract engineer. The 
     terms "Engineer" and "City" in the ISPWC and the 
     City of Ririe Drawing and Specifications shall 
     refer to the City contract engineer and the City of 
     Ririe.

7-4-13:  SUBDIVISION DEVELOPER’S OR PROPERTY OWNER’S   
           RESPONSIBILITY:

A.    Water Lines:

  - The owner or developer of any piece of property or tract of land is responsible for the construction of the water line (or for the water line costs) along any of the property sides or frontage of the property along which a water line is needed for the overall completeness and continuity of the City water distribution system. If it is a large tract of land, the owner or developer shall also be responsible for the necessary and required system of water lines within the interior of the tract of land.

  - The owner or developer of any piece of property shall construct all needed water lines (of the approved size) within and along all sides or frontages of any piece of property prior to the time of any building constructed thereon is occupied or utilized unless phased construction of the water main system is specifically allowed or provided for in the annexation or development agreement. In lieu of actually constructing said required water line or water lines, the property owner or developer may deposit adequate monies with the City to fully cover the cost(s) of said required water line construction.

3. The owner or developer of any property or subdivision will normally be responsible only for the cost of up to an eight-inch diameter water line. The additional cost of providing a water line of a size greater than eight inches will normally be a City responsibility. The City will reimburse the developer for the extra cost of the additional water line size (over the 8 inch diameter size referenced above) as set forth in the annexation or development agreement between the developer and the City. Provided further, however, that if any proposed subdivision or development is of such size or magnitude that it requires or necessitates a water size larger than an eight inch diameter line or if the type of development is such as warrant line capacity in excess of the capacity of an eight inch diameter water line, then the owner or developer of said subdivision or development shall be responsible for the entire cost of whatever size of water line is needed to accommodate his development.

B.   Ownership and Maintenance of Water Lines in Private 
     Property: Private property owners with water 
     mainlines and service lines with meters (accessible 
     to the City via an easement) constructed on private 
     property that terminates on private property 
     serving the exclusive interests of the private 
     property owner or owners will retain ownership of 
     water mainlines on said property. City ownership of 
     water lines will begin at the property line or 
     public right-of-way. Private property owner 
     retains all operations and maintenance 
     responsibility of said mains on their private 
     property. Water meters will remain property of the 
     City. Service lines and meter boxes exclusively on 
     private property remains private property and 
     maintained by the property owner.

7-4-14:  FIRE HYDRANTS:

A.   Access by Authorized Persons Only: All public fire 
     hydrants shall be kept in repair by the City 
     maintenance. Members of the fire department and 
     such other person as may be authorized shall have 
     free access to such hydrants. No other person shall 
     draw or attempt to draw any water from a fire 
     hydrant unless he is an authorized employee of the 
     City or has the written permission of the City 
     maintenance supervisor to do so. The City council 
     may assess an equitable charge for water drawn from 
     a fire hydrant and designate what system or 
     arrangement of valves, etc., are equipped in 
     conjunction with the use of said hydrants(s).

B.  Prohibited Acts: No person shall willfully or 
     carelessly drive or run any vehicle against any 
     fire hydrant or park any vehicle within fifteen 
     feet (15') of any fire hydrant. Any person who 
     shall injure or damage any fire hydrant by 
     accident, or by carelessness, or otherwise, shall 
     immediately report such injury or damage to the 
     City maintenance supervisor and such person so 
     injuring or damaging said hydrant shall be liable 
     for any damage caused thereby.

C.  Fire flow requirements, fire hydrant spacing and 
     related waterline size(s) shall be as required in 
     the International Fire Code for each zones or 
     Development.

7-4-15:  MISCELLANEOUS PROVISIONS:

A.   Every part of the City water system up to, and 
     including, any shutoff valve and/or meter, which may 
     be installed at or near the property line of any 
     lot, is the property of the City. It shall be 
     unlawful for any person to connect to, interfere or 
     tamper with, turn on or off, permit connection or 
     delivery of water to third persons for use within a 
     residence or other building not otherwise provided   
     with water service in accordance with this Chapter, 
     or to operate or introduce any substance into any 
     part of the City water system unless that person is 
     acting under the direct supervision of a qualified 
     employee of the City or first obtains express 
     written permission from the mayor.

B. Right Of Entry: The officers and any duly 
     authorized employees or representatives of the City, 
     establishing his position as an authorized 
     representative of the City, shall be permitted to 
     enter onto private properties for the purpose of 
     inspection, reinspection, observation, measurement, 
     sampling, testing or otherwise performing such 
     duties as may be necessary in the enforcement of the 
     provisions of this chapter, and related rules and 
     regulations of the City.

C.  Right To Revoke Permissions: Permission given to 
     connect water mains and services shall be upon the 
     express condition that the City may, for good 
     cause, revoke the same and the person making such 
     connection, or his successor in interest, shall 
     have no right to claim any damage in consequence of 
     such permission being revoked.

7-4-16:  LIABILITY FOR DAMAGES:

A.  Interruptions in Water Supply: The City shall not 
     be liable for damages caused by scarcity of 
     water caused by the interruptions of water supply 
     due to accidents to the water works or mains or 
     interruptions during the time of alterations, 
     additions, or repairs to the water system or for 
     any other unavoidable causes.

B. Insufficient Supply: In times of, or in 
     anticipation of, scarcity of water or when the 
     water system is unable to furnish a sufficient 
     supply of City water for all purposes, the mayor 
     may, by proclamation, limit the use of water to 
     such extent as may be necessary for the public 
     good. Such proclamation shall be published in two 
     (2) consecutive issues of the official newspaper, 
     and after such publication, the proclamation shall 
     have the same force and effect as a City ordinance.

C.  Liability for Damage: Whenever an agent or person 
     other than the City work crew performs any 
     excavation in the public right-of-way that results 
     in damage to the City utility system, the cost of 
     repairing any such damage to City utilities shall 
     be borne by the agent or person causing such 
     damage, or by the landowner who employed him/her. 
     No excavation shall begin until response has been 
     received from the state mandated one call service.

D.  Hauling of Water: Drawing water from the City 
     water system for the purpose of hauling or 
     transporting it away shall be done only under the 
     supervision of an officer or employee of the City, 
     and then only after the applicable fees and 
     deposits have been paid to the City 
     clerk-treasurer. Said fee may be set by resolution 
     of the City.

7-4-17: ENFORCEMENT:

A.   Violations: No person shall violate the requirements 
     of this chapter. All persons shall be held strictly 
     responsible for any and all acts of agents or 
     employees done under the provisions of this or any 
     other ordinance, rule or regulation of the City. 
     Upon being notified by the City of any defect 
     arising in any water works or of any violation of 
     this chapter, the person or persons having charge of 
     said work shall immediately cease any violation and 
     correct the same. Violations of this chapter may be 
     sanctioned by imposition of criminal misdemeanor 
     penalties and upon conviction thereof, shall be 
     subject to penalty as provided in Section 1- 4-1 of 
     this Code and by civil remedy from a court of 
     competent jurisdiction, by fees and charges 
     calculated to recapture inordinate costs prompted by 
     noncompliance and/or by disconnection from the water 
     system. Each and every connection or occupancy in 
     violation and each and every day or part of a day a 
     violation continues may be deemed a separate offense 
     here under and punishable as such. The mayor or any 
     designated agent of the City may initiate 
     enforcement action.

B.   Liability For Expense, Loss or Damage: Any person 
     violating any of the provisions of this chapter, 
     rules or regulations of the City, shall be liable to 
     the City for any expense, loss or damage occasioned 
     by the City by reason of such violation.

DIVISION 400 SUPPLEMENT TO THE 2020 EDITION OF THE IDAHO STANDARDS FOR PUBLIC WORKS CONSTRUCTION

## SECTION 401 -WATER PIPE AND FITTINGS

Subpart 401.2.2.A

All City water main pipe is to be AWWA C-900 DR18 (Pressure Class 235).

Subpart 401.2.2.C

Water main pipe fittings are to be ductile iron compact fittings per AWWA C153 or grey iron per AWWA C110.

Subpart 401.3.7

Replace B & C with the following:

B.   Install insulated copper wire over the top of all 
     mainline pipe and service lines. Wire is to be 
     accessible at all mainline gate valves, hydrant 
     valves, and water service meter boxes. Locator wire 
     in meter boxes is to be left with a three foot 
     pigtail of additional wire wrapped around meter 
     setter support pipe for accessibility. Install

periodic locate wire boxes per SD-514 when distance 
     between service lines, valves and hydrants warrant 
     them.

C.   Test for and ensure continuity prior to City acceptance.

Subpart 401.3.9

Flushing, pressure tests, disinfection and chlorine residual testing will be in strict conformance with ISPWC Standards and performed by the contractor. Pressure test service lines simultaneous with the mainlines to which the service lines are attached. The contractor shall pressure test service lines and meter components ten feet downstream of the meter box as a minimum or to the end of the contractor-installed service pipe. Place a cap at the end of the service line so that testing of meter box components and service lines can be performed simultaneously with the mainline.

## SECTION 403 - HYDRANTS

Subpart 403.2.2

Accepted hydrants are Mueller Centurion, Clow Medallion and Waterous Pacer. Waterous Pacer hydrants must be supplied with a 24-inch traffic section to be of comparable height with other accepted hydrants.

Add Subpart 403.2.9

2.9   Flush Hydrant

A. Kupferle Manguard Post Hydrant (as per Ririe-405). This detail 
   replaces SD-405.

B.  Or approved equal.

Add Subpart 403.3.2.H

H. A flush hydrant is required at the end of all 
   permanent dead-end mainlines, such as cul-de-sacs at 
   locations where fire hydrants are not necessary. 
   Place the flush hydrant within one foot of the back 
   of the sidewalk within the utility easement or in 
   street right-of-way one foot from the property line 
   in non-curb and guttered areas. The locations of 
   each flush hydrant are to be plainly shown on each 
   set of drawings submitted for review.

## SECTION 404-WATER SERVICE LINE AND METERS

Subpart 404.2.2.A

Service pipe for ¾-inch to two-inch services shall be SIDR 7 (250 PSI) rated HOPE service pipe.

Subpart 404.2.2.D

- Service pipe for apartments with multiple services shall be treated as a mainline pipe from the City owned mainline. All services will then stem from this privately owned main and comply with Paragraph A above.

- The sizing of the mainline for a building's fire sprinkling system is the responsibility of the     Applicant. Multi-residential buildings serving multiple units will require a special  privately owned mainline pipe and meter manifold design so that each unit may be separately metered. The detail design must be stamped by an Idaho                Professional Engineer and submitted to the City for review and approval prior to            installation. Industrial, commercial and institutional meters will be ¾ inch to            two-inch or larger as approved by the City. Line sizing for fire sprinkling systems is               the responsibility of the Applicant.

Subpart 404.2.4.G

Delete 1.a and 1.b and add the following to the end of paragraph one after the colon: Mueller coil box type per detail Ririe-2.

Delete 2.a and add the following to the end of paragraph two after the colon: Mueller Coil box type per detail Ririe-3.

Subpart 404.2.4.1

Add Paragraph I as follows:

I. Curb Stops

  - All service lines are to have curb stops before the meter.

  - Provide Ford B66 Series with no lead or equal.

  - Curb boxes are to be the arch base type suitable for six-foot bury depth. "Erie Pattern" lid and extension rod. Provide Ford EA1 Series or equal. Provide also larger curb box base for 1 ½ inch and two-inch lines.

CHAPTER 4

DRINKING WATER REGULATIONS

ARTICLE A.  INDIVIDUAL WATER AND WASTEWATER SYSTEMS

SECTION:

7-4A-1: 	Water and Wastewater Systems

7-4A-2: 	Exceptions

7-4A-3: 	Procedures for Sandpoint Wells Not to Exceed Twenty 			       Feet

7-4A-4: 	Violations and Penalties

7-4A-1: 	WATER AND WASTEWATER SYSTEMS:

A. 	Compliance: No person shall initiate construction of any 		dwelling or other occupiable structure within the corporate 	limits of the city except in full compliance with the 			provisions of this article. All proposed dwellings and 
     other occupiable structures shall henceforth be served by 			municipal water and sewer.

B. 	Applicability: Any sand point well providing non potable 
     water only shall be subject to the procedures in this 
     article. (2025 Code)

7-4A-2: 	EXCEPTIONS:  None. (2025 Code)

7-4A-3: 	PROCEDURES FOR SANDPOINT WELLS NOT TO EXCEED TWENTY 			     FEET:

A. 	Conditional Use Permit Required: Any person who hereafter 		desires to place an individual well on his own property 		shall require a conditional use permit and shall submit to 		the planning and zoning commission a completed application 		form provided by the commission representative, accompanied 	by a non-refundable filing fee and mailing fee as 
     established by resolution of the city council. Each 
     individual well shall require a separate conditional use 
     permit.

B. 	Application; Checklist:

1. The name, address, and telephone number of the applicant 	or at least one owner of the property.

2. The size and legal description of the entire area to 		scale, general location, proof of ownership, location of 
     the proposed well site, site plan showing setbacks and lot 
     size.

3. A list of the names and addresses of all property owners 	within a three hundred foot (300') radius of the property 
     of the proposed site, prepared by an assigned city official 
     and two (2) sets of mailing labels for mailings.

4. Location of the nearest water main or sewer main, and 
     its distance from the property of the proposed site.

5. Proof of notification and response from other parties or 	agencies that may be affected by the proposed well, as 			directed by the commission.

6. A checklist of these proceedings shall be part of the 		application and in the applicant's file as a permanent 			record.

7. The application shall not be complete until all required 	written responses are received by the commission 				representative.

8. The application shall be completed with fees paid, and 		submitted for consideration to the commission or
     representative at least twenty-one (21) days prior to the 
     next regular meeting of the commission. The completed 
     application and written responses, when available, are to 
     be copied for each commissioner, the city clerk-treasurer 
     and the mayor.

9. The commission or designated staff member shall review 		the application for correct information and set a public 		hearing date.

C. 	Notice Of Public Hearing: Notice of public hearing before 		the planning and zoning commission and city council, if 		conducted, shall be as required by Idaho Code.

D. 	Commission Action: Following the public hearing, the 			commission shall consider all relevant evidence and 
     comments and determine whether to recommend that the 
     council approve or disapprove the conditional use permit 
     for an individual well. The conditional use permit for 
     individual wells may be determined by the council upon a 
     clear showing by the applicant that the following 
     circumstances exist:

1. The defined need for future continuity of municipal 			systems would not be adversely affected by installation of 		an individual system at the location requested.

2. The water supply of the city would not be adversely 			affected by installation of an individual system at the 		proposed location.

3. Adjoining landowners would not be adversely affected by 		the discontinuity of municipal systems, which might 			eventually serve lands owned by them.

E. 	Conditions: The following conditions may be placed upon 		conditional use permits issued pursuant to this article:

1. Limits on the duration of such conditional uses.

2. Installation of partial public utility improvements to 		enable provision of water and wastewater systems in the 		future.

3. Steps to mitigate the effects of installations on 			environmental quality.

4. The posting of bonds or other acceptable surety to 
     assure future compliance with conditional use or ordinance 			requirements.

5. Such other obligations as may be required to address 		potential requirements enabled to Idaho Code section 			67-6512.

F. 	Continuation Of Public Hearing: Any public hearing by the 		commission may be continued to the next regular meeting of 		the commission at the discretion of the commission.

G. 	Commission's Recommendation: The commission's 				recommendations for the approval or denial of the 				conditional use permit at the regular meeting shall be 			submitted to the council as soon as possible.

H. 	Council Action: The council shall review the commission's 		recommendations and all other relevant evidence and take 		action to approve or deny the conditional use permit; 
     reject the recommendations of the commission; return the 			application for modification; or the council may request a 		public hearing by the council for review. The public 
     hearing will follow the same format as for the commission.

I. 	Approval And Authorization: Upon the approval or 
     disapproval of the conditional use permit, the applicant 
     will be notified, in writing, of such decision. If an 
     application is disapproved, the council shall cite the 
     specific reasons for disapproval. If an application is 
     approved, the council shall issue a conditional use permit 
     for an individual well, stating all specific conditions, 
     time limits, and modifications as deemed appropriate 
     therein. (2025 Code)

7-4A-4: 	VIOLATIONS AND PENALTIES:

A. 	Penalty For Violations: Every person convicted of a 			violation of any provision of this article shall be guilty 		of a misdemeanor, and subject to penalty as provided in 		section 1-4-1 of this code.

B. 	Violations Of Continuing Nature: For any violation of a 		continuing nature, each day's violation shall be considered 	a separate offense and shall subject the offender to the 		penalties provided in subsection A of this section for each 	offense.

C. 	Action To Require Compliance: Notwithstanding the 
     imposition of any penalties hereunder, the city may 
     institute an appropriate civil action or proceeding to 
     require compliance with the provisions of this article. 
     (2025 Code)

CHAPTER 5

SEWER USE REGULATIONS

SECTION:

7-5-1:           Definitions

7-5-2:      Ownership and Management of Sewer System

7-5-3:      Connection Required

7-5-4:      Sewer System and Fees Established

7-5-5:      Hookup Fee

7-5-6:      Sewer Capacity Buy-in Fee

7-5-7:      Monthly Service Fees

7-5-8:      Use of Public Sewer

7-5-9:      Sewer Service Lines

7-5-10:     Private Sewer Mainline Extensions

7-5-11:     Extension of City Sewer Mains

7-5-12:     Sewer System Design and Materials

7-5-13:     Subdivision Developer's or Property Owner's 
              Responsibility

7-5-14:     Miscellaneous Provisions

7-5-15:     Enforcement

Attachment:	Division 500 Supplement to the 2020 Edition of the Idaho Standards for Public Works Construction

7-5-1:     DEFINITIONS:

As used in this chapter, the following words and terms shall have the meanings ascribed to them in this section:

APARTMENT HOUSE:     Any building or portion thereof which is 
                     designed to be occupied or which is 
                     occupied as a home of three or more 
                     families living independently of each 
                     other.

BOD (BIOCHEMICAL    The quantity of oxygen utilized in the OXYGEN DEMAND)      biochemical oxidation of organic matter 
                    under a standard laboratory procedure in 
                    five days at twenty degrees centigrade 
                    (2CT C) expressed in milligrams per liter.

BASE RATE:          The fixed portion of the monthly service 
				fee charged covers depreciation, 
				amortization and debt service of the system, 
				and generally the fixed costs of Operations 
				and Maintenance. This fee is based on an 
				assessed number of user equivalents.

BUILDING (HOUSE)	The system of sewer pipes within the walls of SEWER:			the building and extending five feet outside 
				the wall of the building.

CAPACITY			An equity-based fee calculation

BUY-IN FEE:		designed to place new utility users

on an equal equity share basis with existing 
     users as allowed by State

COMMERCIAL AND		Shall include any property, building or   
INDUSTRIAL	     structure used for any business activity,  
BUILDINGS:          trade, or profession; and shall include, but 
				not be limited to, garages, service 
                    stations, wholesale and retail stores, 
                    laundries, dry cleaners. restaurants, bars, 
                    barbershops, beauty shops, schools, 
                    churches, public buildings, and all other 
                    not specifically defined as residential 
                    buildings.

CONDOMINIUM:		Any building or complex of buildings which 
                    is designed to be owned and occupied as a 
                    home of four or more families living 
                    independently of each other where the 
                    condominium complex has a sewer line and is 
                    organized as a condominium regime pursuant 
                    to Idaho Code title 55, Chapter 15. Each 
                    condominium unit shall be designed with a 
                    separate sewer service line to be connected 
                    to the City sewer mainline.

DOPL, PLUMBING	Division of Occupational Professional 
PROGRAM:          Licensing, Plumbing Program (formerly the 
				Division of Building Safety, Plumbing 
				Program)

DUPLEX:			A building designed to be, or which is 
				occupied by two (2) independent family 
                  units. This includes single family 
                  residences with a separate apartment.

HOOKUP FEE:		The hookup fee is a fee made by the City to 
				permit the connection and discharge of 
				wastewater to the City sewer system via a 
				sewer service line.

INDUSTRIAL WASTES:	Any flow discharged to the wastewater 
				treatment system identified in the standard 
				industrial classification manual, 1972, 
				office of the management and budget, as 
				amended and supplemented under the following 
				divisions:

Division A.	Agriculture, forestry and fishing;

Division B.	Mining;

Division C.	Manufacturing;

Division D. 	Transportation, communications,

electric, gas, and sanitation

services;

Division E.	Services.

MOBILE HOME PARK:	Any area or site of land upon which 
				two or more mobile homes are placed 
				and maintained for dwelling 
				purposes only, either on a 
				permanent or semi-permanent basis.

MONTHLY SERVICE	A fee made by the City to owners of residential. 
FEE:			     commercial, or industrial buildings, for the right 
				to discharge sewage from said buildings to the 
				public sewer system. This fee is the sum of the 
				base rate and use rate.

MOTEL:			A building or group of buildings on the same 
				premises, either detached or in connected rows, 
				containing sleeping or dwelling units, but without 
				kitchen facilities in the individual units, and 
				designed for or occupied by travelers. A motel 
				with kitchen facilities in the individual units 
				will be considered as an apartment house. A motel 
				with a commercial building on the same premises, 
				both of which are served through one service 
				connection, will be considered as a commercial 
				building.

pH:			The base-10 logarithm of the reciprocal of the 
				concentration of hydronium ions in grams per liter 
				of solution.

RESIDENTIAL		Shall include single-family residences and    
BUILDING:	     duplexes for the purpose of establishing sewer 
				service charges.

SANITARY SEWER:	A pipe or conduit for carrying sewage from 
				developed land uses to which storm water, surface 
				water, and ground water are not intentionally 
				Admitted.

SEWAGE:			A combination of water carried waste from 
				residential buildings, commercial buildings and 
				industrial buildings.

SERVICE			The physical connection of a sewer service line to 
CONNECTION:       the sewer mainline. A service connection is 
				usually located within the public right of way or 
				easement. A service connection may be on private 
				property when the property owner has more than one 
				service connection and the service connections 
				attach to a private mainline which then enters the 
				public street right-of-way and connects to a City 
				owned manhole.

SEWER			A pipe or conduit for carrying sewage which may be MAINLINE:		owned and operated by the City or privately owned 
				within private property.

SEWER SERVICE		The sewer line from the building sewer to the City    
LINE:			sewer mainline in the street or alley or private 
				Mainline.

SINGLE-FAMILY		Any building designed to be utilized as a 
RESIDENCE:		residence for one family. This definition shall 
				include a single manufactured or mobile home which 
				is not located on a rented space in a mobile home 
				Park.

TOTAL SUSPENDED	Solids that either float on the surface or are in 
SOLIDS (TSS):		a suspension in water, sewage, or other liquid, 
				and which are removable by laboratory filtration, 
				reported in milligrams per liter.

USER EQUIVALENT:	One residential user equivalent is designated as 
				the service to a typical single-family dwelling. 
				All other connections are prorated in relation to 
				residential user equivalents based on the 
				estimated usage of or benefit derived from the

service. The user equivalent shall be based on the contribution to the system of the flow, BOD and suspended solids from a typical residential dwelling as given below:

Flow: 200 gallons/day (6,000 gallons/month)

BOD: 0.51 pounds/day

TSS: 0.51 pounds/day

An assessment of user equivalent is made at the time of application for service is made and is based upon the user equivalent schedule herein.

USE RATE:		The variable cost portion of the monthly 
				service fee which consists generally of the 
				variable portion of operations and 
				maintenance costs. (2025 Code)

7-5-2:	OWNERSHIP AND MANAGEMENT OF SEWER SYSTEM: Through the enactment of ordinances and council resolutions, the city council shall exercise policy making control over the City's municipal sewer system. The mayor shall administer the day to day operation of said system within the policy guidelines established by the council by ordinance and resolution. Regardless of who shall have borne the cost of installing any part of the City sewer system, that part, once installed and approved by and accepted by the City council, shall become the property of the City, subject to any rights of guarantee accompanying such decision. (2025 Code)

7-5-3:	CONNECTION REQUIRED:

A.	Connection to System Required: Every parcel of land or premises 
	within the boundaries of the City, improved for occupancy and 
	occupied or used by any person or persons, or as a commercial 
	business, shall be connected to the sewer system. The owner or 
	person in charge of such land shall make or cause to be made such 
	connection within one hundred twenty (120) days after receiving 
	official notice from the City to connect.

B.	Dwelling Without Water Service Declared Health Hazard and Public 
	Nuisance: The human habitation of any residence or use of any 
	other building or structure for purposes other than warehousing 
	or storage of nonperishable goods or commodities where no workers 
	regularly labor without a sewer connection service line and 
	building sewer inside of such dwelling or other structure shall 
	constitute a health hazard and a public nuisance. In such 
	circumstances, the City may cause proceedings to be brought for 
	the abatement of the occupancy or use of such a place.

C.	Habitation of Dwelling Without Sewer Service Unlawful: The human 
	habitation of any residential dwelling or other building or 
	structure without a required building sewer, sewer service line, 
	and a connection to mainline as required by this chapter shall be 
	unlawful and subject to penalty as provided in Section 7-5-15 of 
	this chapter. (2025 Code)

7-5-4:  SEWER SYSTEM AND FEES ESTABLISHED:

A.	Establishment: It is necessary to protect the health, safety and 
	welfare of the City inhabitants and to provide equitable fees 
	upon all lots, lands, property and premises served by the 
	sanitary sewer system. Said system and facilities consist 
	generally of manholes, sewer mains, interceptors, as well as the 
	provision for treatment and disposal of sewage.

B.	Availability: The City will make the service of the system 
	available within its capacity to all persons in the City service 
	area without discrimination as to race, color, religion or 
	national origin. The City sewer service area is currently 
	confined to those areas within the City corporate limits having 
	sewer service available. Should service be extended elsewhere 
	within or outside of the City limits, fees for sewage treatment, 
	plant operation, maintenance, debt service, and equipment 
	replacement will be charged on the same basis as City residents 
	with service.

C.	System Fees: A system of fees for access to and the use of the 
	sanitary sewer collection and treatment system is necessary. 
	Those fees will differentiate among the different types of users 
	of the system to ensure equitable fees for the various 
	categories. The fees from each user shall be used for the 
	operation and maintenance of the system and for capital 
	improvements to replace assets that reach the end of their useful 
	lives. Fees shall be proportional to the estimated or measured 
	flow contribution of each user of the system. Similar uses shall 
	be treated in a like manner. Fees assessed by the City shall be 
	reviewed periodically and may be revised as needed by resolution 
	of the City council to address operation and maintenance costs 
	and capital improvements needs.

D.	Each User Equivalent to Pay Fees: Fees will be assessed to each 
	user equivalent served by the sewer service line and mainline 
	connection. Partial user equivalents are allowed if the 
	assessment of user equivalents is not a whole number.

E.	Fees Paid in Full at Building Permit Issuance: The hookup fee and 
	sewer capacity buy-in fee shall be paid at the time of the 
	issuance of a building permit. No building permit shall be issued 
	until such fees are paid in full. (2025 Code)

7-5-5:	HOOKUP FEE:

A.	Hookup Fee: The hookup fee for all residential, public, 
	commercial and industrial buildings shall be established by 
	resolution and subject to adjustments from time to time by the 
	City council. Hookup fees are to be deposited into the sewer 
	Fund.

B.	The purpose of a hookup fee is to compensate the City for 
	administrative and clerical effort to setup new account, inspect 
	the physical connection of the sewer service line to the City 
	sewer mainline, and reimburse the City for any materials provided 
	to the account applicant. (2025 Code)

7-5-6:	SEWER CAPACITY BUY-IN FEE

A.	The City Council adopted Ordinance A-2021-01 implementing a sewer 
     capacity buy-in fee. The fee is designed to achieve a fair equity  
     position between each new user equivalent of the sewer system and 
     existing users. This fee is assessed to each new residential user 
     equivalent.

B.	The collected sewer capacity buy-in fees are kept in a reserve 
	account known as the sewer capitalization reinvestment account. 
	The revenue shall be used for the purpose of funding treatment 
	plant, sewage collection, pumping and transmission improvements. 
     (2025 Code)

7-5-7:	MONTHLY SERVICE FEES:

A.	Minimum Monthly Service Fee: The City clerk-treasurer shall 
	furnish to each property owner, or agent, a monthly statement of 
	the amount due for sewer service for that month. The failure of 
	any user to receive notice or statement will in no way affect the 
	obligation of the user to remit monthly as required.

1.	Basis: Monthly service fees will be computed using abase 
	rate plus a variable use rate. The base rate shall be determined 
	using the residential equivalent user system initially set up by 
	determining the estimated number of user equivalents according to 
	the schedule herein. The base rate and declining block variable 
	use rate has been established by resolution and will be subject 
	to periodic change by the City council.

2.	Purpose: The purpose of the monthly service fee is to pay for system maintenance and operation, including depreciation (for asset replacement), debt service and loan reserves of the public sewer system.

3.	Monthly Billing and Payment: The monthly sewer billing will be included in conjunction with the monthly water bill. Payment shall be due as for water service.

4.	Delinquencies: The City is hereby authorized to shut off City supplied water to a delinquent sewer customer, to physically disconnect the sewer service connection to any property, or to seek such other legal remedy as may be necessary to discontinue sewage discharge into the sewer system, if the monthly sewer service fee for that property is delinquent, or if use by the property is in violation of this chapter. The service connection may be disconnected or otherwise blocked if the delinquent charge or ordinance violation continues for more than ten days after the City has notified the property owner of the pending disconnection.

5.	Reconnection: The property owner is required to pay the City all expenses related to physically disconnecting the service connection before acceptance of payment of a hookup fee for reconnection. All back charges and fees shall be paid in full before reconnection will be allowed. An applicable fee as set forth by resolution will be charged in the event the water is turned on after a shutoff for a delinquent account or for nonpayment of sewer charges.

6.	Occupation or Use Prohibited: It shall be unlawful to occupy or use a structure to which sewer service has been disconnected.

B.   Property Owner Ultimately Liable for Bills and Damage:

1.	All sewer service accounts shall be opened and maintained by the owner of record or contract purchaser of record of the property to be served. Such owner or purchaser shall agree, as a condition of opening any sewer service account, to be liable for any service or other charges imposed because of such sewer services, whether such charges are the consequence of use of the sewer by the owner or by tenants who have agreed to pay utility charges.

2.	An owner may allow a tenant to pay utility charges while 
     the tenant is in possession of rented premises, but such   
     action shall not relieve the owner of the ultimate 
     responsibility for the charges accruing to the account.

C.   Additional User Fees for Commercial/Industrial Customer: 
     Existing and future nonresidential establishments may be 
     assessed additional user fees for treatment if the BOD and 
     TSS included in their sewage exceeds that of an average user 
     equivalent given volume by definition herein. The manner in 
     which this assessment is to be made is to weight flow, BOD 
     & TSS at 1/3 each. So, if the flow was equal to one EDU, the  
     BOD equal to two EDU's and TSS equal to three EDU's, then 
    The total EDU's the bill would be based on is to be adjusted 
     to compensate for the excess sewage strength as follows:

Flow: 1x1/3 EDU=

1/3 EDU BOD: 2x1/3

EDU = 2/3 EDU TSS:

3x1/3 EDU = 3/3 EDU

Total:  2 EDU's are to be billed.

D.	Multiple Users: Where a residence, apartment(s), retail store or 
     other commercial establishment is provided sewer service in

conjunction with another establishment in the same building, the

minimum user equivalent rating for that building shall be determined based on the number of establishments contained therein. (For example, a retail store with a residence attached would be rated at a minimum of 2 residential user equivalents. A commercial/office building with 3 apartments would be rated at 4 user equivalents, minimum.)

E.	Schedule Of User Equivalents/Charges: The following schedule 
	lists a number of users and the number of user equivalents 
	assigned to each user. All other users shall require special 
	appraisal by the council and will be computed on an individual 
	Basis:

SCHEDULE OF USER EQUIVALENTS/CHARGES

Number of User

Classifications                                          Equivalents

| Apartments (see Multiple Living Units) | - |
| --- | --- |
| Auto Garage (per 3,000 square feet) | 1 |
| Bar (per 10 seats or fraction thereof) | 1 |
| Barbershop, per chair operator station (minimum 1) | .50 |
| Beauty Salon, per operator station (minimum 1) | 1 |
| Carwash (per bay minimum or based on water usage) | 2 |
| Church | 1 |
| Condominium (see Multiple Living Unit) | - |
| Dining Hall (Senior Citizens Center per 2 persons) | 1 |
| Doctor or Dentist (per practitioner) | 2 |
| Duplex (see Multiple Living Unit) | - |
| Laundromats: Self-service (up to and including 12 washing machines) Each machine in excess of 12 | 5 .40 |
| Commercial (to be computed on an individual basis) | TBD |
| Lodge (per 3,000 square feet) | 2 |
| Mobile Home Court/Park (per space) | 1 |
| Mobile Home or Trailer House on Own Premises | 1 |
| Motel, Hotel, Rooming House, etc. First Unit Each Additional Bed Space | 1 .25 |
| Multiple Living Unit: Each Unit | 1 |
| Office Building (for each 2,500 square feet of gross floor space or Fraction thereof) | 1 |
| Recreational Vehicle Hookup in RV Park | .50 |
| Restaurant (10 seats or fraction thereof) | 2 |
| Retail or Business Establishment (for each 3,000 square feet of gross Floor space or fraction thereof) | 1 |
| Schools: With Cafeteria & Shower (students & staff) With Cafeteria Only (students & staff) Without Cafeteria & Showers | .10 .09 .08 |
| Service Station (with restroom service) | 2 |
| Single-family Residence | 1 |
| Theater (50 seats) | 1 |

The above user equivalents are based upon typical monthly sewer usage as contemplated by standards set forth herein. Such standards shall be subject to adjustment from time to time by the City council. Any such change shall be communicated to the sewer user before it is imposed. And such user shall be provided an opportunity to be heard before the City council if said user contests the additional user fee.

F.	Voluntary Discontinuance: When sewer services are voluntarily    
     discontinued by request of the owner, the base rate portion 
	of the monthly service fee shall continue to be paid. Full 
	sewer service fees will resume immediately upon water service 
	turn back on.

G.	Industrial Users Not on Schedule: Industrial users or other 
	businesses with industrial waste and uses not categorized    
     above or not clearly defined as being within one or more of 
	the above classifications shall be charged at a rate to be 
	determined by the mayor, subject to appeal to the City 
	council, upon application of the property owner, after 
	considering all relevant evidence pertaining thereto at a 
	public hearing held for such purpose. The rate shall be 
	established based upon consideration of the nature and 
	intensity of the proposed use and total impact to the City 
	sewer system. The fee shall be directly related to the cost 
	of providing sewage facilities for such use and shall be   
     proportionately consistent with the schedule set forth 
     herein. The rationale for charging any such fee shall be 
     explained, in writing, upon request by the user.

H.	Aggrieved User; Installation of Meter:

1.  Should any user consider himself to be aggrieved by the 
	foregoing schedule, or by the determination of the mayor and     
     City council, such user may, at his own expense, install a 
	meter, metering the strength and flow of his sewage, in which 
	event the charge shall be determined based upon verified 
	results. Such meter shall be calibrated and monitored to the 
	satisfaction of the contract City engineer. Reliable 
	technical data shall support any rate determination.

2. The City may, at its own discretion, from time-to-time conduct a flow/revenue study wherein the amount of monthly fees paid by the average residential user for the previous full fiscal year for the associated sewer volume (as determined by the wintertime use extrapolated for an entire year) be compared to the revenue and flow of each commercial/institutional users revenues and flows to verify the accuracy of assigned user equivalents. Future fees billed and paid will be adjusted to calculated flow-based user equivalents. The City nor the users have the right to require the other to retroactively correct past fees billed and paid to be in conformance with study findings of actual user equivalents. (2025 Code)

7-5-8:	USE OF PUBLIC SEWER:  In addition to those   
          limitations on the use of sewer set forth in the plumbing laws, rules and regulations of the State DOPL, Plumbing Program, the following additional requirements are established:

A.	Prohibited Discharges: No person shall discharge 
	or cause to be discharged any storm water, surface 
	water, runoff, subsurface drainage, cooling water, 
	heating water, water from swimming pools, or 
	industrial process water except as hereinafter 
	provided, to any sanitary sewer.

B.	Physical Barriers Against Sewer Gases: Physical barriers 
	against sewer gases must be maintained. "Physical barriers" 
	are defined as approved sink and toilet traps filled with 
	Liquid or airtight caps on all sewer inlet/outlets except 
	approved vents. If the property owner does not provide and 
	maintain barriers, the service connection may be 
	disconnected by the City and the property owner shall be 
	required to pay the City all expenses related to 
	disconnecting the service connection before issuance of a 
	reconnection permit.

C.	High Temperatures: No person shall discharge or cause to 
	be discharged any liquid or vapor having a temperature 
	higher than one hundred forty degrees Fahrenheit (140°F).

D.	pH: No person shall discharge or cause to be discharged 
	any waters or wastes having a Ph lower than 5.5 or higher 
	than 8.5 or having any other corrosive property capable of 
	causing damage or hazard to the collection system, lift 
	pump stations and sewage treatment works.

E.	Grease Traps: In all cases where a building is used as a 
	hotel, boarding house, restaurant, service station,

garage, etc., the owner or occupant shall provide a 
	properly constructed grease trap through which all wastes 
	of a greasy nature shall be drained. The City council 
	shall have authority and is hereby authorized and directed 
	to compel any person to provide and use a grease trap as 
	aforesaid whenever, in its judgment, the same is 
	necessary.

F.	Pretreatment Required on Industrial Wastes: The admission

into the public sewers of any commercial/industrial user

with sewage having above average strength measured largely 
	by higher-than-normal concentrations of BOD and TSS, shall

pre-treat the sewage to normal concentrations. BOD is to be

reduced to two hundred twenty milligrams per liter (220 
	mg/I). TSS is to be reduced to two hundred twenty 
	milligrams per liter (220 mg/I) by weight. Pretreatment 
	must also reduce objectionable characteristics or 
	constituents to within the maximum limits provided herein;

and control the quantities and rates of discharge of such

waters or waste.

  - Report Relating to Quantity and Characteristics of Waste: Each person or firm desiring to make a connection to the public sewer for the purpose of discharging industrial wastes thereto shall prepare and file with the City a report that shall include actual or predicated data relating to the quantity and characteristics of the waste to be discharged.

  - Plans, specifications and any other pertinent information relating to the proposed preliminary treatment facilities shall be submitted for the approval of the City council. The council may seek technical support from a contract City engineer and weigh his/her recommendations. No construction of such facilities shall be commenced until said approvals are obtained from the City council.

  - Pretreatment of all industrial wastes shall comply with all applicable federal, state, and local laws and regulations.

4. Charges: Industrial waste charges may be based upon the amount of flow, biochemical oxygen demand (BOD), and suspended solids or any combination thereof as may be recommended by the City's contract City engineer or this chapter.

5. Total Charge: The sewage collection and treatment charges for industrial waste shall completely pay for all collection and treatment costs associated with said waste.

6. Control Manhole or Sample Port: When required by the City, the owner of any industrial building shall install, at his expense, a suitable control manhole or sample port in the building sewer to enable observation, sampling and measurement of the wastes. Such manholes or ports, when required, shall be accessible at all times and not susceptible to freezing.

7. Maintenance of Preliminary Treatment 
	Facilities: Where preliminary treatment facilities 
	are required for any water or wastes, they shall 
	be maintained continuously in satisfactory and 
	effective operation by the building owner at his 
	Expense.

8. Measurements, Tests and Analyses: All measurements, tests, and analyses of the characteristics of water and waste to which reference is made shall be determined in accordance with standard methods for the examination of water and wastewater and shall be determined from samples taken at the control manhole or port.

9. Special Agreement Or Arrangement: No statement contained in this section shall be construed as preventing any special agreement or arrangement between the City and the owner of any industrial building whereby an industrial waste of unusual strength or character may be accepted by the City for treatment, subject to payment therefor by the owner of the industrial building of fair user equivalent charges consistent with the schedule set forth in this chapter and consistent with federal and state law. It shall be the responsibility of each discharging user to pay its fair share of the costs of discharge and treatment of its wastewater. (2025 Code)

7-5-9:  SEWER SERVICE LINES:

- Compliance Required: All sewer service line construction shall be accomplished using watertight materials and conform to City standard drawings and specifications, and the requirements of the current edition of plumbing laws, rules and regulations of the State DOPL, Plumbing Program.

- Cleanouts: Cleanouts in building sewers and sewer service lines shall be provided in accordance with the rules of the City and the aforementioned plumbing laws. All cleanouts shall be maintained watertight.

- Connection Point; Septic Tank: The connection point for a building sewer in those cases in which the present building sewer discharges to a septic tank and drain field shall be at a point between the building and the septic tank prior to entrance into septic tank. All existing septic tanks must be properly abandoned.

D. Contractor Installation Qualifications: The installation of the service connection to the sewer mainline is to be done by one holding an appropriate state public works license. The installation of the sewer service line is to be done by a state-licensed plumber authorized to perform the work. The jurisdictional division of the City and State DOPL, Plumbing Program is outlined in the Memorandum of Understanding between the Idaho Division of Building Safety (now DOPL), Plumbing Program and the Idaho Department of Environmental Quality.

- Lifting Required: For all buildings in which any building sewer is too low to permit gravity flow to the public sewer mainline, sanitary sewage carried by such building sewer shall be lifted by pumping or ejection as approved by the City and discharged to the public sewer in a small pressure pipe at the expense of the building owner.

- Permits and Inspections Required:

1. Sewer Service Connection Permit: A property owner desiring to obtain a sewer service connection to a City owned sewer mainline shall file a request with the City. Such a request shall be in sufficient detail as to adequately inform the City as to the proposed location of the desired sewer service line and connection point and the nature of the anticipated discharge. At the time of approval of the permit request, but before any construction may begin, the property owner shall also pay to the City all sewer hookup fees and capacity buy­ in fees which shall be established and adjusted from time to time by council resolution. The property owner shall also obtain a date for inspection by the City of the construction and physical connection to the City sewer mainline.

2. Right-of-Way Encroachment Permit: No sewer service line installation shall be done in the City right-of-way without first obtaining a City Right-of-Way Encroachment Permit and the state on-call utility locate service has been notified and field locates made.

3. Extension Prohibited Without Permission: It shall be unlawful for any person to extend any sewer service line beyond the limits of the building for which a permit has been given without obtaining permission for the desired extension and appropriate hookup fees and capacity buy-in fees are paid.

4. Inspection at Time of Connection: In accordance with permit requirements, no backfill of the sewer service line connection to the mainline shall occur without City inspection and approval of the materials and workmanship of the connection. Should a property owner or his duly authorized agent connect sewer service line to the City mainline without said inspection and approval, the property owner will be required, at his sole expense, to uncover the newly connected sewer line works so that proper inspection can occur.

- Trench Excavation, Backfill and Surface Restoration: Any trench excavation, backfill and surface restoration required shall conform to the City standard drawings and specifications, or the county jurisdiction, or the State of Idaho Transportation Department as applicable.

- Cost of Installation Borne by Private Property Owner: The sewer service line and connection to the City sewer mainline shall be furnished and installed by and at the sole expense of the property owner.

- Service Lines and Connections Requirements:

1. Location: Service lines shall meet all minimum 
vertical and horizontal separation distance 
required from potable drinking water facilities in 
accordance with the Idaho Drinking Water Rules 
(IDAPA 58.01.08) as administered by the Idaho 
Department of Environmental Quality.

2. Sleeves are required on sewer service lines laid 
   within 18 inches over/under water mainlines or water

service lines up to the building sewer line.

3. Connection to Manhole: A service connection may 
    not be connected to a manhole. A private sewer force 
    main or private sewer mainline may be connected to a  
    manhole.

4. Service Connections to Mainline: Service to two or 
   more buildings through one sewer service line is  
   prohibited. All service line connections must be made 
   directly into a sewer mainline.

5. Trench Excavation, Backfill and Surface 
   Restoration: Any trench excavation, backfill and 
   surface restoration required shall conform to the City 
   standard drawings and specifications, or the county 
   jurisdiction, or the State of Idaho Transportation 
   Department as applicable.

J. Maintenance: All sewer users shall keep their  
   individual sewer service lines in good repair and  
   shall keep them in such good condition that they 
   shall not allow ground water to enter into the  
   sewer system. Sewer users shall also be responsible 
   for all costs associated with maintaining the 
   entire service line from the building sewer line up 
   to the City sewer mainline. (2025 Code)

7-5-10:	PRIVATE SEWER MAINLINE EXTENSIONS:

A. Sewer Design, Plans and Specifications:

1. Compliance with Standards: All construction or    
reconstruction of private sewer mainlines to City 
owned mainlines and manholes shall be accomplished 
using watertight materials, shall be in accordance 
with the City standard drawings and specifications, 
and subject to inspection by the City.

2. Plan and Profile Drawings: Plan and profile 
    drawings shall be prepared for all new sewer mains 
    (whether public or private) and for all sewer   
    mainline extensions, reconstructions, or

renovations; and all such plan and profile drawings  
    must be reviewed by the contract City engineer and 
    approved by the City council and the Idaho z

Department of Environmental Quality before any 
     construction work on said sewer lines is started 
     per Idaho Code 39-118 or its successor.

3. Private Sewer Mainline Extension: A private 
    mainline extension shall not have an inside  
    diameter of less than eight inches. The property  
    owner requesting a sewer mainline extension shall 
    submit engineered drawings for City review and 
    approval. The Property owner shall be responsible 
    for the City engineering review fees incurred to 
    review the sewer line drawings as billed by the 
    City.

B. Contractor Installation Qualifications: The 
    installation of the mainline connection and

mainline to the City-owned manhole is to be done 
    by one holding an appropriate state public works  
    license. The installation of the sewer service 
    line is to be done by a state licensed plumber 
    authorized to perform the work. The jurisdictional 
    division of the City and State DOPL, Plumbing 
    Program is outlined in the Memorandum of 
    Understanding between the Idaho Division of 
    Building Safety (now DOPL), Plumbing Program and 
    the Idaho Department of Environmental Quality.

C.  Permits and Inspections Required:

1. Mainline Connection Permit: A property owner  
    desiring to attach a private sewer mainline to 
    the City owned mainline shall file a request with 
    the City. Such a request shall be in sufficient 
    detail as to adequately inform the City as to the 
    proposed location of the desired sewer mainline 
    and mainline connection point and the nature of 
    the anticipated discharge. The property owner 
    shall also obtain a date for inspection by the 
    City of the construction. City review fees of 
    plans and specifications must be paid at the time 
    permit application is made.

2. Right-of-Way Encroachment Permit: No private 
    mainline improvement work shall be done in the  
    City  right-of-way without first obtaining a 
    City Right-of-Way Encroachment Permit and the 
    state one-call utility locate service has been 
    notified and field locates made.

3. Inspection at Time of Connection: In    
   accordance with permit requirements, no backfill   
   of the private sewer mainline to a manhole 
   should occur without City inspection and 
   approval of the materials and workmanship of the 
   connection. Should a property owner or his duly 
   authorized agent connect private mainlines to 
   the City system without said inspection and 
   approval, the property owner will be required, 
   at his sole expense, to uncover the newly 
   connected sewer line works so that proper 
   inspection can occur.

4.Extension Prohibited Without Permission: It  
   shall be unlawful for any person to extend any 
   sewer mainline beyond the limits of the building 
   or buildings for which a permit has been given 
   without obtaining permission for the desired 
   extension.

D. Trench Excavation, Backfill and Surface   
   Restoration: Any trench excavation, backfill and 
   surface restoration required shall conform to the 
   City standard drawings and specifications, or the 
   county jurisdiction, or the State of Idaho 
   Transportation Department as applicable.

E. Cost of Installation Borne by Property Owner: The 
   private mainline connection to the City sewer 
   mainline and private mainlines installed on 
   private property shall be furnished and installed 
   by and at the sole expense of the property owner.

F. Private Sewer Mainline Maintenance: Once the 
   sewer mainline connection to City owned manhole 
   has been completed and properly inspected to the 
   City's satisfaction, the City will maintain the 
   improved manhole after the contractors one year 
   warranty period has expired. The property owner 
   shall own, operate and maintain all sewer 
   mainlines on private property installed for 
   his/her exclusive use. (2025 Code)

7-5-11:	EXTENSION OF CITY SEWER MAINS:

A. Areas Within City: The City may construct, or 
   cause to be constructed, extensions to the sewer 
   system if one of the following conditions exists:

1. The annual anticipated revenue made available 
   to the City from the customers to be immediately 
   served by such extension is not less than one 
   fiftieth (1/50) of the actual cost to the City for 
   the construction of said extension.

2. The City declares that the sewer main 
   extension is needed for the overall benefit and 
   improvement of the entire sewage collection 
   system.

3. The City council approves a contract for the  
   construction of the extension. However, 
   subdividers or developers within or adjacent to 
   the City shall construct, at their own expense,  
   all sewer mains within their subdivisions or 
   developments.

B. Areas Contiguous to City: In areas contiguous to  
   the City that may be annexed to the City, the 
   City may require such areas to be annexed before 
   sewer service is provided. Prior to annexation, 
   the City shall require the execution and approval 
   of an annexation agreement which shall detail the 
   conditions for sewer service.

C. Areas Outside City: Sewer lines shall not, 
   without the City council's approval, be extended 
   outside the corporate limits of the City; 
   however, when, in the opinion of the City, there 
   will be special benefit and improvement to the 
   City by reason of an extension outside the City, 
   special arrangements may be made with the council 
   to allow said extension; provided, however, that 
   the areas to be serviced outside of the City 
   shall not benefit to a greater extent than like 
   areas provided with sewer service within the 
   City; and provided further, that no such 
   arrangement shall be made unless adequate sewer 
   line capacity and sewage treatment capacity is 
   available for such outside the City sewer 
   services. (2025 Code)

7-5-12:	SEWER SYSTEM DESIGN AND MATERIALS:

A. Materials and Installations:

1. All sanitary sewer main line pipe, manholes, 
   lift stations, accessories, appurtenances, and 
   their installation shall be in accordance with 
   City standard drawings and specifications for the 
   construction of sanitary sewer infrastructure.

2. All sewer service line materials and their 
   installation shall be in accordance with the City 
   standard drawings and specifications for 
   construction of sanitary sewers.

B. Design and Approvals:

1. The plans for all sewer mainlines, sewage lift 
   stations, sewage treatment facilities, etc., shall be     
   designed, signed, and sealed by a registered  
   professional engineer licensed in the state of Idaho 
   and reviewed by the City contract engineer and   
   approved in writing by the City and the Idaho 
   Department of Environmental Quality in accordance 
   with Idaho Code 39- 118 or its successor.

2. All sewer lines, lift stations, sewage  
treatment facilities, etc., shall be designed in accordance with the Idaho Wastewater Rules (IDAPA 58.01.16).

3. Minimum sanitary sewer main size shall be eight inch diameter. Larger sizes shall be as required by the City's overall sewer system master plan or as are needed to provide adequate sewage carrying capacity for the area (and surrounding developable area) being developed and/or annexed to the City.

C. Standard Drawings and Specifications - Adoption of 
   ISPWC: The City of Ririe Standard Drawings and 
   Specifications is the 2020 edition of the Idaho 
   Standards for Public Works Construction (ISPWC) 
   and the supplemental modifications attached at the 
   end of this ordinance pertaining to sewer 
   infrastructure labeled: Division 500 Supplement to 
   the 2020 Edition of the Idaho Standards for Public 
   Works Construction. In the event of a conflict 
   between the ISPWC and the City of Ririe Division 
   500 Supplement to the 2020 ISPWC, the City of 
   Ririe Division 500 Supplement to the 2020 ISPWC 
   shall govern unless the contrary is approved in 
   writing by the City for a specific circumstance as 
   recommended by the City's contract engineer. The 
   terms "Engineer" and "City" in the ISPWC and the 
   City of Ririe Drawing and Specifications shall 
   refer to the City contract engineer and the City 
   of Ririe. (2025 Code)

7-5-13:  SUBDIVISION DEVELOPER'S OR PROPERTY OWNER'S  
         RESPONSIBILITY:

Sewer Lines:

1. The owner or developer of any piece of property or tract of land is responsible for the construction of the sewer line (or for the sewer line costs) along any of the property sides or frontage of the property along which a sewer line is needed for the overall completeness and continuity of the City sewer collection system. If it is a large tract of land, the owner or developer shall also be responsible for the necessary and required system of sewer lines within the interior of the tract of land.

2. The owner or developer of any piece of property shall construct all needed sewer lines (of the approved size) within and along all sides or frontages of any piece of property prior to the time of any building constructed thereon is occupied or utilized unless phased construction of the sewer main system is specifically allowed or provided for in the annexation or development agreement. In lieu of actually constructing said required sewer line or sewer lines, the property owner or developer may deposit adequate monies with the City to fully cover the cost(s) of said required sewer line construction.

3. The owner or developer of any property or subdivision will normally be responsible only for the cost of up to an eight-inch diameter sanitary sewer line. The additional cost of providing a sanitary sewer line of a size greater than eight inches will normally be a City responsibility. The City will reimburse the developer for the extra cost of the additional sanitary sewer line size (over the 8 inch diameter size referenced above) as set forth in the annexation or development agreement between the developer and the City. Provided further, however, that if any proposed subdivision or development is of such size or magnitude that it requires or necessitates a sewer size larger than an eight inch diameter line or if the type of development is such as to produce a wastewater flow in excess of the capacity of an eight inch diameter sewer line, then the owner or developer of said subdivision or development shall be responsible for the entire cost of whatever size of sewer line is needed to accommodate his development.

B. Sewage Lift Stations: Sewer mains shall be kept as  
   deep as practicable to keep sewage lift stations to 
   the smallest number possible. When a sewage lift 
   station and its related facilities is required, it 
   and the sewer main system directing sewage to it 
   shall generally be designed for an entire definable 
   area and not just to serve a single property owner or 
   developer. The cost of the sewage lift station shall 
   be equitably divided between the developer and the 
   City as set forth in the annexation or development 
   agreement.

C. Ownership and Maintenance of Sewer Lines in Private 
   Property: Private property owners with sewer lines 
   constructed on private property that terminates on 
   private property serving the exclusive interests of 
   the private property owner or owners will retain 
   ownership of sanitary sewer mainlines on said 
   property. City ownership of sewer lines will begin 
   at the property line or public right-of-way. 
   Private property owners retain all operations and 
   maintenance responsibility of said mains on their 
   private property. (2025 Code)

7-5-14:	MISCELLANEOUS PROVISIONS:

A.  Use Of System Refused: The City may prohibit the 
    discharge into the public sewer of sewage that does 
    not comply with the limitations set forth in this 
    chapter.

B.  Tampering With System Prohibited: No person shall 
    maliciously, willfully or negligently break, damage, 
    destroy, uncover, deface or tamper with any     
    structure, appurtenance or equipment which is part 
    of the City sewage works. No person shall discharge 
    any substance into the sewer without first 
    establishing an account. Any person violating these 
    provisions shall be subject to immediate restraint 
    and the penalties provided by this chapter.

C.  Right Of Entry: The officers and any duty authorized   
   employees or representatives of the City, establishing 
   his position as an authorized representative of the 
   City, shall be permitted to enter in and upon any and 
   all buildings, industrial facilities and properties for 
   the purpose of inspection, reinspection, observation, 
   measurement, sampling, testing or otherwise performing 
   such duties as may be necessary in the enforcement of 
   the provisions of this chapter, and related rules and 
   regulations of the City.

D.  Right To Revoke Permissions: Permission given to 
    connect sewer mains and services shall be upon the 
    express condition that the City may, for good 
    cause, revoke the same and the person making such 
    connection, or his successor in interest, shall 
    have no right to claim any damage in consequence of 
    such permission being revoked. (2025 Code)

7-5-15:	ENFORCEMENT:

A. Violations: No person shall violate the   
    requirements of this chapter. All persons shall be 
    held strictly responsible for any and all acts of 
    agents or employees done under the provisions of 
    this or any other ordinance, rule or regulation of 
    the City. Upon being notified by the City of any 
    defect arising in any sewer or of any violation of 
    this chapter, the person or persons having charge of 
    said work shall immediately cease any violation and 
    correct the same. Violations of this chapter may be 
    sanctioned by imposition of criminal misdemeanor 
    penalties, and upon conviction thereof, shall be 
    subject to penalty as provided in Section 1- 4-1 of 
    this code and by civil remedy from a court of 
    competent jurisdiction, by fees and charges 
    calculated to recapture inordinate costs prompted by 
    noncompliance and/or by disconnection from the sewer 
    system. Each and every connection or occupancy in 
    violation and each and every day or part of a day a 
    violation continues may be deemed a separate offense 
    here under and punishable as such. The mayor or any 
    designated agent of the City may initiate 
    enforcement action.

B.  Liability For Expense, Loss or Damage: Any person 
    violating any of the provisions of this chapter, 
    rules or regulations of the City, shall be liable 
    to the City for any expense, loss or damage 
    occasioned by the City by reason of such violation.

(2025 Code)

DIVISION 500 SUPPLEMENT TO THE 2020 EDITION OF THE IDAHO STANDARDS FOR PUBLIC WORKS CONSTRUCTION

# SECTION 501 - GRAVITY SEWERS

Subpart 501.2.2.A

All sewer mainline pipe shall be ASTM D3034 (SDR35) Polyvinyl Chloride (PVC) with integral bell and rubber gasketed joints. Gaskets shall conform to ASTM F477. Use fabricated PVC tees or wyes conforming to ASTM D3034 with watertight gaskets and joints for all service line connections.

Subpart 501.3.4

Low pressure air testing, deflection testing and CCTV inspection on all new sewer mains are required.

# SECTION 502 - MANHOLES

SD-501A

The standard manhole is to be as per SD-501A (precast base) with the following modifications:

  - A six-inch protruding concrete ring around each manhole base is mandatory to ensure that there is adequate resistance to uplift forces of the manhole when submerged in groundwater.

  - Provide manholes without steps.

  - A double flexible watertight rubber boot at the manhole pipe/sewer line connection in lieu of a single banded boot is required to minimize the risk of failure and infiltration.

Other Manholes

All other manholes that may be needed on a project such as SD-502A, SD-504 and SD-505A are to be amended in the same manner as SD-501A.

Subpart 502.3.5.B.2 Delete Paragraphs a & band replace with the following:

A cast-in-place manhole base may only be used when placing a new manhole over an existing mainline. The existing pipe is to remain in place from the spring line of the pipe downward, except to allow the new mainline to connect at a 0.1 foot higher grade. The new cast-in-place base and upper manhole sections are to be lined with an approved infiltration inhibitive liner to prevent infiltration.

Add Subpart 502.3.16 WARRANTY

Contractor/developer warrants to the City that any new manhole found to be infiltrating groundwater for any reason during the one-year warranty period shall be corrected by

lining the manhole with an infiltration inhibitive liner at no cost to the City. Liner application must be proceeded by structural repairs that will enable the liner to function properly.

# SECTION 504 - SEWER SERVICES

Subpart Section 504.2.2.A

Use ASTM D3034 SOR 35 sewer service pipe and fittings.

Subpart Section 504.2.2.E.1

Use sewer service tee or wye fittings for services on new mainline pipe.

Subpart 504.2.2.E.3

Use Inserta-Tee type sewer service connections on exiting mainline pipe.

# ADD NEW SECTION 513

SECTION 513

SANITARY SEWER LIFT STATIONS

PART1	GENERAL

  - SECTION INCLUDES

    - Sanitary sewer lift station materials and installation.

  - RELATED SECTIONS

    - Section 204 - Structural Excavation and Backfill

    - Section 305 - Pipe Bedding

    - Section 703 - Cast-in-Place Concrete

    - Section 802 - Crushed Aggregate

  - REFERENCES

    - ASTM C 478: Precast Reinforced Concrete Manhole Section

    - ASTM C 497: Standard Test Methods for Concrete Pipe, Manhole Sections, or Tile

    - Local City and State Electrical Codes and the National Electric Code (NEC), current edition.

  - SUBMITTALS

A. Submit show drawings for materials to be installed or furnished 
   under this section.

B. Submit manufacturer's certification that manholes, 
       pipes, pumps, panels, and appurtenances meet or exceed 
       specified requirements.

C. Submit manufacturer's installation instructions and maintain 
       copy at the jobsite.

  - PROJECT RECORD DOCUMENTS

- Accurately record actual locations of constructed lift 
  stations and other encountered utilities in relation to 
  existing permanent benchmarks.

- Provide a copy of record documents to Owner prior to 
  issuance of substantial completion.

  - DELIVERY, STORAGE AND HANDLING

- Handle and store manhole sections, covers, pipes, pumps, 
  panels and appurtenances in a manner which prevents 
  shock, damage, or detrimental exposure to weather.

- Project joint sealing material used for lift station structure 
  manholes from sunlight and contamination until ready for 
  installation in the manhole.

PART2	MATERIALS

  - LIFT STATION STRUCTURES

    - A typical submersible pumps wet well/dry well lift station is allowed.

    - Wet well and dry well structures are to be manufactured of standard precast concrete units conforming to Section 704 with a field applied spray on liner to eliminate the chance of groundwater infiltration. The bases of the wet well and dry well must be designed to resist buoyancy forces.

    - Concrete bases and lids are to be reinforced to support intended loads.

    - Accessories: Lids on both wells are to be flat. The wet well is to be fitted with a 30 inch by 48-inch hinged metal access cover with upper guide brackets, chain hooks, lid support, and horizontal bar for attachment of the liquid level sensor cables. Meet or exceed HS-25 loading requirements if situated where vehicles can drive over the lid. The dry well access may be a standard manhole frame and lid.

  - PRESSURE PIPE, VENT PIPE AND CONDUITS

    - Pressure pipe: Four-inch minimum diameter or as required by size of pumps. Ductile iron inside the lift station and over to and through the valve vault. Downstream of valve vault conform with Section 505 - Pressure Sewers. Provide ductile iron fittings in the force main per Subpart 505.2.4.

    - Ball Check Valves: Are required in the dry well to automatically prevent force main drainage back into the wet well.

    - Isolation Gate Valves: Per Section 402.2.2.

    - Emergency Bypass Pumping: Provide a ductile iron tee or wye fitting, gate valve and four-inch stainless steel female camlock fitting for emergency bypass pumping.

    - Vent Pipe: four-inch galvanized rigid steel pipe with threaded fittings. Threaded collar on the above ground end of the vent pipe, with a welded 1/8-inch screen covering the opening.

    - Electrical Conduits: Galvanized rigid steel meeting the requirements of NEC and of the size required for the conductor.

  - SUBMERSIBLE PUMPS

    - Pumps brand, model and type are to be submitted to City Public Works for review and approval. The City alternately may dictate the brand and model. The City's decision will be final.

    - Equipped with the required size electric motor connected for operation on a 208/240- volt, 3-phase, 60-hertz, 4-wire service with a minimum of 35 feet of Type SPC cable suitable for submersible pump applications or as approved by the Engineer.

C. Power Cable: Sized according to NEC and ICEA Standards 
       with a P-MSHA approval. Connect from the pumps 
       directly to the control panel.

D. Furnish each pump with a minimum of 25 feet of stainless   
   steel lifting chain of adequate strength to safely raise 
   and lower the pump with two galvanized or stainless 
   steel lifting rails and with mounting hardware and 
   stainless steel bolts to fit the pumps and bases to the 
   lift station as shown on the Standard Drawings.

  - PUMP CONTROLS

    - Pump Control Panel: Minimum 36" x 30" x 12" NEMA Type 4 gasketed, watertight, dust tight, lockable enclosure. 208/240-volts, 3-phase, 60-hertz, 4-wire service or as approved by the Engineer. All electrical equipment U.L. listed.

    - Panel Requirements

      - Intrinsically safe solid-state alternator for two or more pumps which provides alternating operation of pumps under normal conditions and provides simultaneous operation of both pumps in case of high level conditions.

      - Condensation heater.

      - Lightning arrestor.

      - Supply a manual transfer switch and receptacle consistent with existing City lift stations for connection to the City's portable generator.

      - Pump motor controls, to include:

        - Running time meter

        - Pump run light

        - Combination circuit breaker/overload with manual reset for protection against current overloads, short-circuit protection and disconnect for all phases.

        - Across-the-line magnetic contactor.

        - Hand/off/auto pump operations selector switch.

        - Amperage meters with interior panel door display, sized appropriately based on the individual pump motor.

    - Liquid pressure transducer mounted in the lift station manhole, communication cable, and control unit(s) for pump control.

    - Equipment to be compatible with the lift station pump control and designed for the site conditions.

    - Control unit installed in a lockable weather resistant enclosure mounted on the lift station control pedestal a minimum of 30 inches above the lift station lid.

    - Design the control system for a duplex pump system capable of ensuring that the lead pump changes with each "pump on" event.

    - Design the control unit to allow the operator to control pump operation from the unit mounted on the lift station pedestal with a liquid level selection range between O and 30 feet. Control unit to control pump and alarm operation at the following operator-selected liquid level elevations that occur in the Lift Station Manhole (with elevations supplied by the City Public Works):

      - Pump off (minimum liquid level}.

      - Lead pump on.

      - Lag pump on.

      - Alarm on.

H. All electrical conduit openings that penetrate the lift station 
       wall to have pliable removable seal-offs, NEMA-approved for 
       Class 1 Division 1 locations.

I. Electrical package shall be equipped with a SCADA 
    termination panel compatible with the City SCADA 
    provider to install a SCADA link. City will direct 
    the SCADA link installation. Developer will 
    reimburse City for all the SCADA link costs.

J.  An emergency generator is required as part of new lift 
  station for any subdivision that will support more than 
  10 homes.

PART3	WORKMANSHIP

  - SUBMERSIBLE PUMPS AND CONTROLS

    - Install all equipment in a neat, plumb and workmanlike manner in accordance with the manufacturer's recommendations.

    - Mount the level transducer on the lift station wall in accordance with the manufacturer recommendations and in a location that is accessible at access hatch and minimizes false liquid level readings.

  - ELECTRICAL WORK

A.  Complete all electrical work in accordance with the approved 
       plans, these specifications, all local and State electrical 
       codes and the NEC, current edition, by licensed electricians.

B.  Obtain a state electrical installation permit prior to 
       starting construction.

  - ELECTRICAL SERVICE

A.  Install conduit and wire from the meter base at the 
        lift station control panel to the power supply as 
        shown on the plans or as directed by Rocky Mountain 
        Power representatives.

B.   Arrange for inspection by the state electrical 
         inspection upon completion of the lift station 
         installation. Pay any inspection fees.

  - ALARM SYSTEMS

A.   Sanitary Sewer Lift Stations: Alarms to be accommodated 
        through the telemetry system.

B.  Install an alarm light on the outside of the control 
         panel. Connect the high-level alarm sensor cable from 
         the lift station to the control panel. Notify the City 
         Public works when the alarm system work is completed 
         to verify that the alarm system is functioning 
         properly.

  - PRESSURE DISCHARGE PIPE

A.  Install pressure discharge pipe to discharge manhole if 
         possible. Install the pipe at an elevation so the top of the 
         pressure discharge pipe and the top of the gravity pipe in 
         the discharge manhole are level. Direct the outflow of the 
         pressure discharge pipe at the gravity pipe invert.

B.  Locate ball valves and isolation valves and 
         emergency bypass pumping connection in the dry well.

  - CONDUITS

A.  Install conduits in sand bedding.

B.  Blow conduits free of any loose debris or moisture 
         prior to pulling any wire and seal conduits 
         thereafter.

  - START-UP AND TRAINING

A.  Upon completion of construction, notify Ririe Public 
    Works of the date and time for initial start-up of 
    the lift station.

B.  Complete a minimum four-hour test period and provide 
         operation training for City Public Works staff. 
         Provide two paper copies and one electronic pdf copy 
         of the approved operation and maintenance manual to 
         the City prior to acceptance.

C.   Develop final completion punchlist with City Public    
         Works. Punchlist must be completed for final 
         acceptance.

PART4	MEASUREMENT AND PAYMENT

  - Sanitary sewer lift station to be measured on a   
lump sum basis complete, in place, and fully operational as stated in these Specifications. Payment includes full compensation for all labor, materials, equipment and tools necessary to furnish, install, test and make ready for service the lift station complete and in place. If not specifically indicated otherwise on the Plans and specifically included in the Bid Schedule, all items required to perform the work, including structure excavation and structure backfill, precast concrete manhole, lift station wet well, pipe, discharge pipe and fittings, trench excavation and backfill, submersible pumps and controls, and any other required items, are incidental to the Bid Item.

- Sanitary Sewer Lift Station: On lump sum basis for  
 the construction of a fully operational lift 
 station.

1.  Bid Schedule Payment Reference: 513.1.1.A.1.

2.  Bid Schedule Description: Sanitary Sewer Lift 
      Station... lump sum (LS).

CHAPTER 6

CITY PARK

SECTION:

7-6-1: 	Park Rules And Regulations

7-6-1: 	PARK RULES AND REGULATIONS:

A. 	Vandalism: There will be no vandalism or malicious damage to 	
     any trees, buildings, playground equipment, fencing and 		
     anything which is considered to be part of the city park 		
     system.

B. 	Camping:

1.  Overnight Camping: There will be no overnight camping in 	
     the city park.

2. Group Meetings; Permission Required: There will be no 		
     camping or special group meetings permitted without prior 		
     approval and a permit issued by city hall.

C. 	Animals and Vehicles: There will be no unauthorized animals or 
     vehicles allowed in the city park.

D. 	Penalty: Any person who violates the provisions of this 		
     section shall be subject to penalty as provided in section 		
     1-4-1 of this code. (2025 Code)

CHAPTER 7

SUBSEQUENT CONNECTION UTILITY FEES

SECTION:

7-7-1: 	Purpose

7-7-2: 	Establishment Of Fees

7-7-3: 	Basis For Calculation

7-7-4: 	Payment Required Before Connection

7-7-5: 	Fees Established by Resolution; Analysis Required

7-7-6: 	Administrative Officials Enabled

7-7-1: 	PURPOSE:  The purpose of subsequent connection utility 			fees is to promote equity and fairness when facilities are installed through the efforts of specially benefited private parties through local improvement districts, through private development undertakings, and through other efforts where systems are extended or expanded through the contribution of specific parties rather than the public at large. (2025 Code)

7-7-2: 	ESTABLISHMENT OF FEES:  Subsequent connection water and 		     sewer fees (hereinafter utility fees) are hereby created and established. Subsequent connection utility fees are additional to the connection and capacity charges which each new system user is obligated to pay prior to connecting to the city utility systems. Subsequent connection utility fees are charges based upon the cost of constructing or extending water distribution lines, constructing additional water storage capacity, constructing or extending sewer collection lines, and otherwise constructing or installing additional features for the water distribution or sewer collection systems which will enable service to be provided to new user who will benefit from the improvements, but did not share in the cost of construction either by virtue of providing new capacity or by extending service into areas not previously served. (2025 Code)

7-7-3: 	BASIS FOR CALCULATION:  The subsequent connection fee 			  
          shall generally reflect the proportionate or marginal cost of engineering, construction, and right of way or easement procurement, etc., incurred to extend the water or sewer system which will enable providing capacity to previously unserved property, including costs incurred through a local improvement district or by a private developer and/or property owner. (2025 Code)

7-7-4: 	PAYMENT REQUIRED BEFORE CONNECTION:  No connection to a 		     newly constructed public water or wastewater facility governed by this chapter, other than by one who contributed proportionately to its construction or costs of installation, shall be allowed until a subsequent connection fee analysis has been completed and accepted by resolution of the city council or until a preliminary subsequent connection fee has been paid as estimated by the director of public works, subject to final determination by resolution of the city council. (2025 Code)

7-7-5: 	FEES ESTABLISHED BY RESOLUTION; ANALYSIS REQUIRED:  All 		     subsequent connection fees shall be fixed by resolution after submission by the director of public works, of a proposed subsequent connection fee analysis. Such analysis shall include a compilation of costs which have been incurred, the basis of their calculation, the manner of their distribution and source of their payment, and the proposed distribution of proceeds from any subsequent connection fees collected which may include payment to private parties. The costs of construction, installation, and extension shall be ascertained by the director of public works of the improvement district or by costs provided and certified by the developer and/or property owner. Such costs shall be limited to the direct expenses of engineering design, construction, and acquisition of easements and rights of way, excluding therefrom rights of way dedicated and improvements installed pursuant to subdivision or other ordinance requirements. Costs do not include administration or overhead of a private developer or property owner. Each resolution of the city council establishing a subsequent connection fee shall establish the fees to be charged and their manner of calculation, the physical area subject to payment of the subsequent connection fees, the disposition of the proceeds from such fees, and the duration of such charges, with such duration not to exceed five (5) years from the date of the resolution, subject to subsequent action by the city council in the final year of the initial period, extending the initial duration by not more than the duration which was originally approved. Except in the case of a local improvement district, a copy of the subsequent user fee analysis shall be provided to the private party(ies) who paid for installation of the specific facilities which are the subject of the analysis no less than ten (10) days prior to the city council meeting at which the analysis will be considered. Such party(ies) may be heard at said meeting. (2025 Code)

7-7-6: 	ADMINISTRATIVE OFFICIALS ENABLED:  The mayor, city  
          maintenance supervisor and city attorney are hereby authorized to take such actions as may be necessary, including entering into contracts with those who have initially paid for the installation of utility system improvements, to carry out the provisions of any resolution of the city council establishing a subsequent connection fee. (2025 Code)

CHAPTER 8

ALLEYS

SECTION:

7-8-1: 	Purpose And Intent

7-8-2: 	Dumping In Alleys, Restriction Prohibited

7-8-3: 	Placement For Pick Up

7-8-4: 	Alleys To Be Kept Clear of Vehicles

7-8-5: 	Violation; Penalty

7-8-1: 	PURPOSE AND INTENT:  The purpose and intent of this 			    
          chapter is to establish the alleys as rights of way and eliminate all forms of refuse being dumped in the alleys. (2025 Code)

7-8-2: 	DUMPING IN ALLEYS, RESTRICTION PROHIBITED:  No person 			  
          shall allow, suffer, cause or permit the dumping of refuse in the alleys including tree trimmings, grass clippings, leaves, old household furniture and fixtures and garbage. Buildings and fences now restricting the sixteen-foot (16') width of the alley cannot be replaced. Any shrubs that restrict the sixteen-foot (16') width must be trimmed back. (2025 Code)

7-8-3: 	PLACEMENT FOR PICK UP:  All garbage will be placed on 			 
          the curb in front of each home for pick up. Any refuse that is not acceptable for garbage hauling must be handled by each household. All containers must be set twenty feet (20') back from the street within forty-eight (48) hours of pick up. (2025 Code)

7-8-4: 	ALLEYS TO BE KEPT CLEAR OF VEHICLES:  All alleys will 			 
          be kept clear of vehicles allowing clear access for all utility trucks. (2025 Code)

7-8-5: 	VIOLATION; PENALTY:  Any person violating any of the 			 
          provisions of this chapter or willfully refusing to comply with any proper requirements shall be deemed guilty of a misdemeanor. Each day a violation of the provisions of this chapter continues shall constitute a separate offense. (2025 Code)

1. See subsection 6-1-4E for parking restriction in city parks.