TITLE 8

BUILDING REGULATIONS

Building Codes                	8-1

8-1-1: International Building Codes			    pg. 2

8-1-2: Uniform Code for Abatement of Dangerous Buildings

8-1-3: International Fire Code

8-1-4: International Mechanical Code

8-1-5: Uniform Housing Code

8-1-6: Idaho Public Works Standards for Public Works 	               
		    Construction

8-1-7: Sanitation Requirements

8-1-8: Certificate Of Occupancy

Flood Damage Protection       	8-2

8-2-1: Statutory Authorization, Finding of Fact,      pg. 5
		    Purpose and Objectives

8-2-2: Definitions

8-2-3: General Provisions

8-2-4: Administration

8-2-5: Provisions For Flood Hazard Reduction

8-2-6: Violations; Penalty

Storm Water Management      	 8-3

8-3-1: Definitions							   pg. 15

8-3-2: Purpose

8-3-3: Applicability

8-3-4: General Requirements

8-3-5: Performance Standards

8-3-6: Guarantee Of Installation

8-3-7: Adoption Of Supporting Documentation

8-3-8: Prohibited Conduct

8-3-9: Variance

8-3-10: Penalties

Manufactured Homes	8-4

8-4-1: Short Title							   pg. 25

8-4-2: Authority

8-4-3: Purpose

8-4-4: Jurisdiction

8-4-5: Interpretation

8-4-6: Definitions

8-4-7: Requirements

8-4-8: Building Permit Fees

8-4-9: Enforcement; Penalty

CHAPTER 8-1

BUILDING CODES

SECTION:

8-1-1: 	International Building Codes

8-1-2: 	Uniform Code for Abatement of Dangerous Buildings

8-1-3: 	International Fire Code

8-1-4: 	International Mechanical Code

8-1-5: 	Uniform Housing Code

8-1-6: 	Idaho Public Works Standards for Public Works 				       Construction

8-1-7: 	Sanitation Requirements

8-1-8: 	Certificate Of Occupancy

8-1-1: 	INTERNATIONAL BUILDING CODES:

A. 	Adopted: The approved editions of the following nationally 		recognized codes, as adopted by the state of Idaho or the 		Idaho building code board, are adopted as the official 			building codes of the city:

1. International building code, except for section 103.3.

2. International residential code, parts I-IV and IX,    
        except for sections 103.2 and 103.3.

3. International energy conservation code.

The adopted versions of the foregoing codes shall be deemed superseded by successive versions of such codes as they are adopted or approved by the Idaho building code board effective on the date any such codes are made effective by the Idaho building code board.

B. 	Fees Prescribed: The city council shall be and is hereby 		authorized from time to time to promulgate and prescribe 		building permit fees by resolution.

C. 	Barbed Wire and Electric Fences: It shall be unlawful for 		any person to erect or maintain any electric fence, or any 		fence constructed in whole or in part of barbed wire or to 		use barbed wire as a guard to any parking lot or parcel of 		land. (2025 Code)

8-1-2: 	UNIFORM CODE FOR ABATEMENT OF DANGEROUS BUILDINGS:  
          The uniform code for the abatement of dangerous buildings, 1997 edition, as amended, prepared by the International Conference of Building Officials (ICBO), providing a just, equitable, and practicable method whereby buildings or structures which, from any cause, may endanger the life, limb, health, morals, property, safety or welfare of the general public or their occupants, may be required to be repaired, vacated or demolished, is hereby adopted and shall be the uniform code for the abatement of dangerous buildings of the city. (2025 Code)

8-1-3: 	INTERNATIONAL FIRE CODE:  The approved edition of the 			     international fire code, as adopted by the state of Idaho or the state fire marshal, is adopted as the official fire code of the city. The adopted version of the foregoing code shall be deemed superseded by successive versions of such codes as they are adopted or approved by the state fire marshal effective on the date any such codes are made effective by the state fire marshal. (2025 Code)

8-1-4: 	INTERNATIONAL MECHANICAL CODE:  The latest approved 			     edition of the international mechanical code, published by the International Conference of Building Officials, as adopted by the state of Idaho, except for sections 103.2 and 103.3, is hereby adopted as the uniform mechanical code of the city. The adopted version of the foregoing code shall be deemed superseded by successive versions of such code as it is adopted by the state of Idaho effective on the date any such codes are made effective by the state of Idaho. (2025 Code)

8-1-5: 	UNIFORM HOUSING CODE:  The uniform housing code, 1997 			     edition, as amended, prepared by the International Conference of Building Officials (ICBO), providing minimum standards for the protection of life, limb, health, property, safety and welfare of the general public and the owner and occupants of residential buildings, is hereby adopted and shall be the uniform housing code of the city. (2025 Code)

8-1-6: 	IDAHO PUBLIC WORKS STANDARDS FOR PUBLIC WORKS 				     CONSTRUCTION:  The Idaho public works standards for public works construction is hereby adopted and shall be the public works standards for the city. (2025 Code)

8-1-7: 	SANITATION REQUIREMENTS:  At the time of a building 			     permit application, unless adequate sewer facilities are available, the applicant shall secure a sanitary outhouse to be placed on the location where the construction would take place as a condition of securing a building permit. (2025 Code)

8-1-8: 	CERTIFICATE OF OCCUPANCY:

A. 	Certificate Of Occupancy Required: It is unlawful for any 		person to occupy a dwelling or building without an approved 	certificate of occupancy. Each day such violation continues 	shall be considered a separate offense.

B. 	Violation: A violation of this section shall constitute a 		misdemeanor and, upon conviction thereof, shall be subject 		to penalty as provided in section 1-4-1 of this code. (2025 	Code)

CHAPTER 8-2

FLOOD DAMAGE PROTECTION

SECTION:

8-2-1: 	Statutory Authorization, Finding of Fact, Purpose, And 		  Objectives

8-2-2: 	Definitions

8-2-3: 	General Provisions

8-2-4: 	Administration

8-2-5: 	Provisions For Flood Hazard Reduction

8-2-6: 	Violations; Penalty

8-2-1: 	STATUTORY AUTHORIZATION, FINDING OF FACT, PURPOSE, AND 			OBJECTIVES:

A. 	Statutory Authorization: The legislature of the state of  
     Idaho has by statute delegated the responsibility to local 		governmental units to adopt regulations designed to promote 	the public health, safety, and general welfare of its 			citizenry to avoid the harm caused by flooding. Therefore, 		the city of Ririe, Jefferson and Bonneville Counties, 
     Idaho, does ordain as follows.

B. 	Statement Of Purpose: It is the purpose of this chapter to 		promote the public health, safety, and general welfare, and 	to minimize public and private losses due to flood 			conditions in specific areas by provisions designed:

1. To protect human life and health;

2. To minimize expenditures of public money and costly 
     flood control projects;

3. To minimize the need for rescue and relief efforts 			associated with flooding and generally undertaken at the 		expense of the general public;

4. To minimize prolonged business interruptions;

5. To minimize damage to public facilities and utilities 		such as water and gas mains, electric, telephone and sewer 		lines, streets, and bridges located in areas of special 		flood hazard;

6. To help maintain a stable tax base by providing for the 		sound use and development of areas of special flood hazard 		to minimize future flood blight areas;

7. To ensure that potential buyers are notified that 			property is in an area of special flood hazard; and

8. To ensure that those who occupy the areas of special 		flood hazard assume responsibility for their actions. (2025 	Code)

8-2-2: 	DEFINITIONS:  Unless specifically defined below, words 		or phrases used in this chapter shall be interpreted to give them the meanings they have in common usage and to give this chapter its most reasonable application:

AREA OF SPECIAL     The land in the floodplain within the city

FLOOD HAZARD:		to a one percent (1%) or greater chance of    	               flooding in any given year. Such lands are 					     designated on the flood 	hazard maps for     
                    Ririe accompanied by code A or V.

BASE FLOOD: 	     The flood having a one percent (1%) chance 
                    of being equaled or exceeded in any given  
                    year. Also referred to as the 100-year 
                    flood. Designation on Ririe flood hazard 
                    maps always include the letter A or V.

DEVELOPMENT: 	     Any manmade change to improved or unimproved  
                    real estate, including, but not limited to, 
                    buildings or other structures, mining, 
                    dredging, filling, grading, paving, 
                    excavation or drilling operations located 
                    within the area of special flood hazard.

FLOOD INSURANCE	The official map on which the federal 
RATE MAP (FIRM):    insurance administration has delineated both 
                    the areas of special flood hazard and the 
                    risk premium zones applicable to the city of 
                    Ririe.

FLOOD INSURANCE	The official report provided by the federal 		
STUDY:              insurance administration that includes 
                    flood profiles, the flood boundary/floodway 
                    map, and the water surface elevation of the 
                    base flood.

FLOOD OR 		     A general and temporary condition of partial or FLOODING:		complete inundation of normally dry land areas 			from:

A. The overflow of inland waters, and/or

B. The unusual and rapid accumulation of   
        runoff of surface waters from any source.

FLOODWAY: 		The channel of a river or other watercourse 
                    and the adjacent land areas that must be 
                    reserved to discharge the base flood without 
                    cumulatively increasing the water surface 
                    elevation more than one foot 1').

LOWEST FLOOR: 	     The lowest floor of the lowest enclosed area 				(including basement). An unfinished or flood 				resistant enclosure, usable solely for 
                    parking of vehicles, building access or 
                    storage, in an area other than a basement 
                    area, is not considered a building's lowest 
                    floor; provided, that such enclosure is not 
                    built so as to render the structure in 
                    violation of the applicable nonelevation 
                    design requirements of this chapter 				               found at subsection 8-2-5B1b of this 
                    chapter.

MANUFACTURED 	     A structure, transportable in one or more     HOME:			sections, which is built on a permanent 
                    chassis and is designed for use with or 
                    without a permanent foundation when 
                    connected to the required utilities. For 
                    floodplain management purposes the term 
                    manufactured home also includes park 
                    trailers, travel trailers, and other similar 				vehicles placed on a site for greater than 
                    one hundred eighty (180) consecutive days. 
                    For insurance purposes the term manufactured 
                    home does not include park trailers, travel 
                    trailers and other similar vehicles.

NEW 			     Structures for which the start of  CONSTRUCTION:       construction commenced on or after the 
                    effective date hereof.

START OF 		     Includes substantial improvements and means  CONSTRUCTION:  	the date the building permit was issued, 
                    provided the actual start of construction, 
                    repair, reconstruction, placement or other 
                    improvement was within one hundred eighty 
                    (180) days of the permit date. The actual 
                    start means either the first placement of 
                    permanent construction of a structure on a 
                    site, such as the pouring of slab or 					
                    footings, the installation of poles, the 					     construction of columns or any work beyond 
                    the stage of excavation; or the placement of 
                    a manufactured home on a foundation or other 
                    method of anchoring. Permanent construction 
                    does not include land preparation, such as 
                    clearing, grading and filling; nor does it 
                    include the installation of streets and/or 
                    walkways; nor does it include excavation for 
                    a basement, footings, piers, or foundation 
                    or the erection of temporary forms; nor does 
                    it include the installation on the property 
                    of accessory buildings, such as garages or 
                    sheds not occupied as dwelling units or not 				          part of the main structure.

STRUCTURE: 	     A walled and roofed building including a gas 
                    or liquid storage tank that is principally 						aboveground.

SUBSTANTIAL 	     Any repair, reconstruction, or improvement  IMPROVEMENT:	     of a structure, the cost of which equals or 
                    exceeds fifty percent (50%) of the market 
                    value of the structure either:

A. Before the improvement or repair is 
        started, or

B. If the structure has been damaged and is 
        being restored, before the damage 
        occurred. For the purposes of this

definition "substantial improvement" is 
        considered to occur when the first 
        alteration of any wall, ceiling, floor, 
        or other structural part of the building 
        commences, whether that alteration 
        affects the external dimensions of the 
        structure. (2025 Code)

8-2-3: 	GENERAL PROVISIONS:

A. 	Lands To Which Chapter Applies: This chapter shall apply to 	all areas of special flood hazard depicted on the federal 		flood hazard map within the jurisdiction of the city of 		Ririe.

B. 	Basis For Establishing Areas of Special Flood Hazard: The 		areas of special flood hazard identified by the federal 		insurance administration in a scientific and engineering 		report entitled "The Flood Insurance Study for The City of 		Ririe", dated February 17, 1988, with accompanying flood 		insurance maps, is hereby adopted by reference and declared 	to be a part of this chapter. The flood insurance study is 		on file at city hall, Ririe, Idaho. (2025 Code)

8-2-4: 	ADMINISTRATION:

A. 	Establishment Of Development Permit:

1. Development Permit Required: A development or building 		permit must be obtained before construction or development 		begins within any area of special flood hazard established 		by the flood insurance study referenced in subsection 8-2-
     3B of this chapter. The development, building or placement 		permit shall be required for all structures including 			manufactured homes, as set forth in section 8-2-2 of this 		chapter, and for all development including fill and other 		activities, also as set forth in section 8-2-2 of this 		chapter.

B. 	Designation Of Local Administrator: The city building 			inspector and/or the city planner is hereby appointed to  
     administer and implement	this chapter by granting or

denying development permit applications in accordance with 
     its provisions.

C. 	Duties And Responsibilities of Administrator: Duties and 		responsibilities of the local administrator shall include, 		but not be limited to:

1. Permit Review:

a. Review all development permits to determine that 
             the requirements of this chapter have been 
             satisfied.

b. Review all development permits to determine that 
             all necessary permits have been obtained from those 
             federal, state, or local governmental agencies from 
             which prior approval is required.

c. Review all development permits to determine if the 		
             proposed development is in the floodway. If 
             located in the floodway, assure that the 
             encroachment provisions of subsection 8-2-5C1 of 
             this chapter are met.

2. Use Of Other Base Flood Data: When base flood elevation 		data has not been provided by the study referenced in 			subsection 8-2-3B of this chapter, the city administrator 		shall obtain, review, and reasonably utilize any base flood 	elevation and floodway data available from a federal, 
     state or other reliable source, in order to administer 
     subsection 8-2-5B, "Specific Standards", of this chapter 
     and subsection 8-2-5C "Floodways", of this chapter.

3. Information To Be Obtained and Maintained:

a. Where base flood elevation data is provided through 		the flood insurance study or required as in subsection 
          C2 of this section, the city administrator shall 
          obtain and record the actual elevation (in relation to 
          mean sea level) of the lowest floor (including 
          basement) of all new or substantially improved 
          structures, and whether or not the structure contains 
          a basement.

b. For all new or substantially improved floodproofed 		
             structures:

(1) Verify and record the actual elevation (in 				         relation to mean sea level), and

(2) Maintain the floodproofing certifications 				         required by this chapter.

c. Maintain for public inspection all records 			      
             pertaining to the provisions of this chapter.

4. Alteration Of Watercourses: While administering this 
     chapter the city administrator shall endeavor to:

a. Notify adjacent communities and the state of Idaho, 		   department of water resources prior to any 
             alteration or relocation of a watercourse, and 
             submit evidence of such notification to the federal 
             insurance administration.

b. Require that maintenance is provided within the 		   
             altered or relocated portion of said watercourse so 
             that the flood carrying capacity is not diminished.

5. Interpretation Of FIRM Boundaries: While administering   
     this chapter the city administrator shall make    
     interpretations where needed, as to the exact location of 
     the areas of special flood hazard (for example, where there 	appears to be a conflict between a mapped boundary and 			actual field conditions). The person contesting the 
     location 	of the boundary shall be given a reasonable 
     opportunity to appeal the interpretation. Such appeals 
     shall be granted consistent with the standards of section 
     60.6 of the rules and regulations of the national flood 
     insurance program (44 CFR 59-76). (2025 Code)

8-2-5: 	PROVISIONS FOR FLOOD HAZARD REDUCTION:

A. 	General Standards: In all areas of special flood hazard, 
     the following standards are required:

1. Anchoring:

- All construction and substantial improvements 		   
shall be anchored to prevent flotation, collapse, 
or lateral movement of the structure.

b. All manufactured homes must likewise be anchored to 		   prevent flotation, collapse or lateral movement, 
             and shall be installed using methods and practices 
             that minimize flood damage. Anchoring methods may 
             include, but are not limited to, use of over the 
             top or frame ties to ground anchors. (Reference 
             FEMA's "Manufactured Home Installation In Flood 		
             Hazard Areas" guidebook for additional techniques.)

2. Construction Materials and Methods:

a. All new construction and substantial improvements 		  
             shall be constructed with materials and utility 
             equipment resistant to flood damage.

b. All new construction and substantial improvements 		
             shall be constructed using methods and practices 
             that minimize flood damage.

c. Electrical, heating, ventilation, plumbing, and air 		   conditioning equipment and other service facilities  
             shall be designed and/or otherwise elevated or 
             located to prevent water from entering or 
             accumulating within the components during 
             conditions of flooding.

3. Utilities:

a. All new and replacement water supply systems shall 		 
             be designated to minimize or eliminate infiltration 
             of floodwater into the system.

b. New and replacement sanitary sewage systems shall 
             be designed to minimize or eliminate infiltration 
             of floodwaters into the systems and discharge from 
             the systems into floodwaters; and

c. On site waste disposal systems shall be located to 		 
             avoid impairment to them or contamination from them 
             during flooding.

4. Subdivision Proposals:

a. All subdivision proposals shall be consistent with 		
             the need to minimize flood damage;

b. All subdivision proposals shall have public 			
             utilities and facilities such as sewer, gas, 
             electrical, and 	water systems located and 
             constructed to minimize flood damage;

c. All subdivision proposals shall have adequate 			  
             drainage provided to reduce exposure to flood 
             damage; and

d. Where base flood elevation data has not been 			
             provided or is not available from another 
             authoritative source, it shall be generated for 
             subdivision proposals and other proposed 
             developments which contain at least fifty 		
             (50) lots or five (5) acres (whichever is less).

5. Review Of Building Permits: Where elevation data is not 		available either through the flood insurance study or from 		another authoritative source (subsection 8-2-4C2 of this 		chapter), application for building permits shall be 
     reviewed 	to assure that proposed construction will be 
     reasonably safe from flooding. The test of reasonableness 
     is a local judgment and includes use of historical data, 
     high water marks, photographs of past flooding, etc., where 
     available. Failure to elevate at least two feet (2') above 
     grade in these zones may result in higher insurance rates.

B. 	Specific Standards: In all areas of special flood hazard 		where base flood elevation data has been provided as set 		forth in subsection 8-2-3B, "Basis for Establishing the 		Areas Of Special Flood Hazard", of this chapter or 			subsection 8-4-2C2, "Use Of Other Base Flood Data", of this 	chapter, the following provisions are required:

1. Residential Construction:

a. New construction and substantial improvement of any 		   residential structure shall have the lowest floor, 
             including basement, elevated to or above base flood 
             elevation.

b. Fully enclosed areas below the lowest floor that 
             are subject to flooding are prohibited or shall be 
             designed to automatically equalize hydrostatic 
             flood forces on exterior walls by allowing for the 
             entry and exit of floodwaters. Designs for meeting 
             this requirement must either be certified by a 
             registered professional engineer or architect or 
             must meet or exceed the following minimum criteria:

(1) A minimum of two (2) openings having a total 				         net area of not less than one square inch for 			    every square foot of enclosed area subject 
                   to flooding shall be provided.

(2) The bottom of all such openings shall be no 				         higher than one foot (1') above grade.

(3) Openings may be equipped with screens, 					         louvers, or other coverings or devices; 
                   provided, that they permit the automatic 
                   entry and exit of floodwaters.

2. Nonresidential Construction: New construction and 			substantial improvements of any commercial, industrial or 		other nonresidential structure shall either have the lowest 	floor, including basement, elevated to the level of the 
     base flood elevation; or together with attendant utility 
     and sanitary facilities, shall:

a. Be floodproofed so that below the base flood level 		  
             the structure is watertight with walls 
             substantially impermeable to the passage of water;

b. Have structural components capable of resisting 		 
             hydrostatic and hydrodynamic loads and effects of 
             buoyancy;

c. Be certified by a registered professional engineer 		  
             or architect that the design and methods of 
             construction are in accordance with accepted 
             standards of practice for meeting provisions of 
             this subsection based on their development and/or 
             review of the structural design, specifications and 
             plans. Such certifications shall be provided to the 
             official as set forth herein;

d. Nonresidential structures that are elevated, not 		  
             floodproofed, must meet the same standards for 
             space below the lowest floor as described in 
             subsection B1b of this section;

e. Applicants floodproofing nonresidential buildings 		    
             shall be notified that flood insurance premiums 
             will be based on rates that are one foot (1') below 
             the floodproofed level (e.g., a building 
             constructed to the base flood level will be rated 
             as 1 foot below that level).

3. Manufactured Homes: All manufactured homes to be placed 		or substantially improved within flood zones A1-30, AH, and 	AE shall be elevated on a permanent foundation such that 
     the lowest floor of the manufactured home is at or above 
     the base flood elevation and be securely anchored to an 			adequately anchored foundation system in accordance with 
     the provisions of subsection A1b of this section.

C. 	Floodways: Located within areas of special flood hazard 		established in subsection 8-2-3B of this chapter are areas 		designated as floodways. Since the floodway is an extremely 	hazardous area due to the velocity of floodwaters which 		carry debris, potential projectiles, and erosion potential, 	the following provisions apply:

1. Encroachments, including fill, new construction, 			substantial improvements, and other development are 			expressly prohibited unless certification is provided by a 		registered professional engineer or architect demonstrating 	that encroachment shall not result in any increase in flood 	levels during the occurrence of the base flood discharge.

2. If subsection C1 of this section is satisfied, all new 		construction and substantial improvements shall comply with 	all applicable flood hazard reduction provisions of this 		section. (2025 Code)

8-2-6: 	VIOLATIONS; PENALTY:  Penalties for a violation of 
          this chapter may include denial of permit, civil enforcement, and/or criminal misdemeanor charges. (2024 Code)

CHAPTER 8-3

STORM WATER MANAGEMENT

SECTION:

8-3-1: 	Definitions

8-3-2: 	Purpose

8-3-3: 	Applicability

8-3-4: 	General Requirements

8-3-5: 	Performance Standards

8-3-6: 	Guarantee Of Installation

8-3-7: 	Adoption Of Supporting Documentation

8-3-8: 	Prohibited Conduct

8-3-9: 	Variance

8-3-10: 	Penalties

8-3-1: 	DEFINITIONS:  As used in this chapter, the following 			     words and terms shall have the meanings ascribed to them in this section:

AS BUILT 		Design plans which have been revised to reflect  DRAWINGS:		all changes to the plans which occurred during 				     construction.

(BMP)BEST 	Physical, structural and/or managerial practices  MANAGEMENT     that, when used singly or in combination, prevent PRACTICES:	or reduce pollution of water.

BOND: 		A surety bond, cash deposit or escrow account, 				     assignment of savings, or other means acceptable 				     to or required by the permit authority to 					     guarantee that work is completed in compliance 				     with all local government requirements.

CLEARING: 	The destruction and removal of vegetation by 					manual, mechanical or chemical methods.

CONVEYANCE: 	A mechanism for transporting water from one point 			to another, including pipes, ditches and 
               channels.

CONVEYANCE 	The drainage facilities, both natural and  SYSTEM:	     manmade, which collect, contain, and provide for 
               the flow of surface and storm water from the 
               highest points on the land down to a receiving 
               water. The natural elements of the conveyance 
               system include swales and small drainage courses, 
               streams, rivers, lakes and wetlands. The human 
               made elements of the conveyance system include 
               gutters, ditches, pipes, channels and most 
               retention/detention facilities.

DESIGN 	 	A rainfall event of specific return frequency and STORM:		duration that is used to calculate the runoff 				     volume and peak discharge rate.

DETENTION: 	The temporary storage of storm runoff in a BMP, 				     which is used to control the peak discharge  
               rates, and which provides gravity settling of 
               pollutants.

EROSION: 		The wearing away of the land surface by running 				     water, wind, ice or other geological agents, 					including such processes as gravitational creep, 				     detachment and movement of soil or rock fragments 			by water, wind, ice or gravity.

EROSION/		Any temporary or permanent measures taken to SEDIMENTATION	reduce erosion; control siltation and      CONTROL:		sedimentation; and ensure that sediment laden 				     water does not leave the site.

GROUND 	 	Water in a saturated zone or stratum beneath the WATER:		land surface or a surface water body.

IMPERVIOUS 	A hard surface area which either prevents or SURFACE:		retards the entry of water into the soil mantel 
               as under natural conditions prior to development, 			and/or a hard surface area which causes water to 				     run off the surface in greater quantities or at 
               an increased rate of flow from the flow present 
               under natural conditions prior to development. 
               Common impervious surfaces include, but are not 
               limited to, roof tops, walkways, patios, 
               driveways, parking lots or storage areas, 
               concrete or asphalt paving, gravel roads, packed 
               earthen materials, and oiled, macadam or other 
               surfaces which similarly impede the natural 
               filtration of storm water. Open, uncovered 
               retention/detention facilities shall not be 
               considered as impervious surfaces.

INFILTRATION: 	The downward movement of water from the surface 
               to the subsoil. The infiltration capacity is 					expressed in terms of inches/hour.

INTERMITTENT 	A stream or portion of a stream that flows only  STREAM:	     in	direct response to precipitation. It 
               receives little or no water from springs and no 
               long continued supply from melting snow or other 					sources. It is dry for a large part of the year, 				     ordinarily more than three (3) months.

LAND 		Any activity that results in a change in the DISTURBING	existing soil cover (both vegetative and  ACTIVITY:		nonvegetative) and/or the existing soil 						topography. Land disturbing activities include, 				     but are not limited to, demolition, construction, 			clearing, grading, filling and excavation.

NUTRIENTS: 	Essential chemicals needed by plants or animals 				     for growth. Excessive amounts of nutrients can 				     lead to degradation of water quality and algal 				     blooms. Some nutrients can be toxic at high 					     concentrations.

QUALIFIED, 	Licensed engineer, landscape architect, soil LICENSED		scientist or other individual who has  PROFESSIONALS:	demonstrated a professional ability to design 
               storm water systems and certify their 
               effectiveness with a registered stamp.

RECONSTRUCTION: Road reconstruction shall be considered any 					modification of the cross section or subgrade. 				     Paving or repaving shall not be considered 					     reconstruction.

RETENTION: 	The holding of runoff in a basin without release 				     except by means of evaporation, infiltration or 				     emergency bypass.

RUNOFF: 		Water originating from rainfall and other 					     precipitation that is found in drainage 						facilities, rivers, streams, springs, seeps, 					ponds, lakes, and wetlands, as well as shallow 				     ground water.

SEDIMENT: 	Fragmented material that originates from 					     weathering and erosion of rocks or unconsolidated 			deposits, and is transported by, suspended in, or 			deposited by water.

SEDIMENTATION: The deposition of sediment on ground surfaces and 			in watercourses.

STORM 		The time interval between major storms of FREQUENCY:	predetermined intensity and volumes of runoff for 			which storm sewers and other structures are 					     designed and constructed to handle hydraulically 				     without surcharging and backflooding, e.g., a 				     2-year, 25-year or 100-year storm.

STORM WATER: 	That portion of precipitation that does not 					     naturally percolate into the ground or evaporate, 			but flows via overland flow, interflow, channels 				     or pipes into a defined surface water channel, or 			a constructed infiltration facility.

SWALE: 		A shallow drainage conveyance or infiltration 
               area with relatively gentle side slopes, 
               generally with flow depths less than one-half 
               foot (1/2').

TREATMENT BMP: A BMP that is intended to remove pollutants from 				     storm water. A few examples of treatment BMPs are 			detention ponds, oil/water separators, 						     biofiltration swales and constructed wetlands.

UNIQUE SITE 	Soil, slope or depth to bedrock conditions that CHARACTERISTICS:  will not accommodate on site treatment through 			   infiltration to the standards specified in 
                  this chapter. (2024 Code)

8-3-2: 	PURPOSE:  The purpose of this chapter shall be to 				require implementation of surface water management techniques which rely upon natural on-site treatment and recycling of storm water as opposed to collection and conveyance of untreated storm water into ground water sources or into surface bodies of water. The underlying purposes to be achieved by implementation of such regulations are the protection of ground water quality through pretreatment of storm water prior to infiltration and protection of surface water resources from the effects of contaminants, sedimentation and erosion. (2024 Code)

8-3-3: 	APPLICABILITY:  The achievement of standards, 				     employment of methods, and techniques required by this chapter shall be applicable in the following circumstances:

A. 	New Development: Development of every new subdivision of 		land within the jurisdiction of the city.

B. 	Commercial; Industrial Sites: All commercial and industrial 	site development for which grading, site development, 			construction, or building permits may be required pursuant 		to the land use regulations of the city.

C. 	Public Projects: All public projects within the   
     jurisdiction of the city including, but not limited to, 
     street construction or reconstruction (modification of the 
     cross section, not to include resurfacing), park 
     development, building development, or other public works 
     construction or development. Except as necessary to   
     maintain the continuity of existing storm water handling 
     systems, all new construction as outlined in this 
     subsection shall employ the methods for storm water   
     management required by this chapter and shall otherwise be 
     designed to minimize adverse impacts upon surface and 
     ground water quality.

D. 	Slope: Every building or land development permit where land 	disturbing activity is to occur on any part of sites with a 	slope greater than fifteen percent (15%) or on sites less 		than five hundred feet (500') distant horizontally from, 
     and located vertically above, any surface water in the form 
     of a perennial or intermittent stream, river, pond, lake, 			wetland, or similar surface feature which can be determined 	from USGS 7.5-minute quadrangle topographic maps. Storm 		water management plans may not be necessary for individual 		building sites if runoff from the site has been 
     accommodated by an approved storm water management plan for 
     the subdivision in which the site is located. However, 
     detailed erosion control plans may still be required. The 			administrator of this chapter may allow a building permit 
     to be issued for minor additions to existing improvements 			without the preparation of a storm water management plan 
     if, in using their professional judgment, the requirements 
     of this chapter can be met by existing site conditions.       	(2025 Code)

8-3-4: 	GENERAL REQUIREMENTS:  Unless otherwise lawfully 				excused from compliance with the standards set forth in this chapter, all development to which this chapter is applicable shall comply with the following requirements and methods for storm water management control:

A. 	Storm Water: Storm water, in accordance with the 
     performance standards set forth herein, shall be directed 
     by nonerosive means to grass infiltration areas when 
     appropriate. Expected contaminant removals are as follows:

Total suspended solids                                                            95%

Total dissolved solids                                                               50%

Nutrients:           Nitrate                                                    20%-50%

Total nitrogen	80%

Phosphorus	90%

Metals	80%+

Organic chemicals	60%+

Bacteria	99%

If the proposed development exceeds site limitations as defined in Appendix D of the "Storm Water Management Plan Criteria and Engineering Standards", for grass infiltration methods, then an acceptable alternative storm water collection, treatment, and disposal system shall be implemented in accordance with an approved storm water management plan, subject to review by the city. Said grass infiltration areas or other approved alternative on site storm water collection and treatment systems may be approved in either nodal or dispersed form, subject to specific approval by the city during the development review process.

B. 	Storm Water Runoff: All development subject to this chapter 	shall be carried out such that the runoff of storm or other 	surface waters shall not be accelerated, concentrated, or 		otherwise conveyed beyond the exterior property lines or 		project boundaries of the project in question except in 		compliance with the provisions of BMPs adopted pursuant to 		this chapter or as allowed through joint management of 
     storm water with adjoining property owners pursuant to 
     agreement approved by the city. The quality of surface 
     runoff shall be protected by strict compliance with the 
     design standards and BMPs adopted pursuant to this chapter 
     or by implementation of measures shown by a qualified, 
     licensed professional, as defined in section 8-3-1 of this     
     chapter, to have an effective design capability which 
     exceeds the BMPs adopted hereby.

C. 	Storm Water Management Plan: Each landowner developing real 	property subject to the terms of this chapter shall develop 	a comprehensive storm water management plan which addresses 	and complies with the requirements and standards 
     established by this chapter and the plan criteria, design 
     standards and BMPs adopted by resolution pursuant to this 
     chapter. Each plan for storm water management shall be 
     approved by a qualified, licensed professional, who shall 
     submit both construction quality design drawings and 
     supporting calculations to the city council who may refer 
     for engineering review prior to start of construction. Each 	storm water management plan shall also identify necessary 		maintenance requirements.

D. 	Funding: Each developer who proposes and implements a storm 	water management system for collection or treatment of 
     storm water in accordance with this chapter shall also 
     establish, including assurance of adequate funding, the 
     necessary maintenance system including an acceptable plan 
     for sustained functioning of the collection and treatment 			system, and, when requested, grant the necessary easements, 	to the local government which may choose to provide 			continued maintenance of the system by public authorities 
     or may require the owner to accomplish same. The city is 
     hereby authorized to establish a department of city 
     government to perform maintenance service for storm water 
     management system components or to contract for such 
     maintenance in order that drainage system components can be    
     maintained. Such services shall be financed by charging 
     fees to owners and/or occupants of real property benefitted 
     by such service. Fees shall be set by resolution of the 
     city council.

For purposes of application of the design standards and other related documents and standards, the city shall be designated as the permit authority. (2024 Code)

8-3-5: 	PERFORMANCE STANDARDS:  The following performance 				standards shall be applicable to all design, construction, implementation, and maintenance of storm water management systems pursuant to the jurisdiction exercised through this chapter.

A. 	Runoff Rate: There shall be no increase in the peak rate of 	runoff from this site when compared with the dissipation of 	storm water in the undeveloped state for a 25-year storm. 		Within project boundaries, sufficient retention capacity 		shall be constructed to detain surface flow to meet the 		performance standard established by this section. For 			purposes of this chapter, "undeveloped state" shall mean 
     the natural soils and vegetation in place prior to the 
     start of any construction or clearing activity on a site.

B. 	Storm Water Collection: No storm water shall be collected 
     or concentrated except within a channel protected against 			erosion and containing energy dissipation measures to 			prevent further erosion on adjoining lands. Existing 			unprotected channels shall be protected against further 		erosion during site development. Any site 			development or construction shall preserve installed 			components of a storm water management plan. All disturbed 		soils shall be protected during construction to prevent 
     exposure to storm water and to contain eroded materials  
     should contact be made.

C. 	Grass Infiltration Areas: Any and all collected storm water 	shall be directed to grass infiltration areas which shall 
     be established with grass and other approved plant 
     materials or 	to an acceptable alternative storm water 
     management design. Grass infiltration areas or their 
     acceptable alternatives shall be sized to hold and treat 
     the first one-half inch (1/2") of storm water runoff from 
     all impervious surfaces. The overall storm water disposal 
     system shall have a capacity to handle a 25-year storm 
     based upon the appropriate Idaho transportation department 
     IDF curve, without damage to the storm water management 
     system or adjacent land and improvements. (2024 Code)

8-3-6: 	GUARANTEE OF INSTALLATION:  No final plat shall be 			     recorded until the storm water management facilities are in place and functioning as designed or until a guarantee of financial surety is provided to and accepted by the city. No certificate of occupancy will be issued until the storm water management system has been installed and is functioning as designed. If, in the judgment of the city council, project occupancy can be achieved without harm to the environment or potential occupants, occupancy may proceed upon receipt of an acceptable guarantee of financial surety to complete installation when weather conditions or other variables allow. In no case shall such guarantee be allowed if the incomplete improvements would result in increased erosion, sedimentation, or other damage to the development, public improvements, subsurface or surface waters, or the proposed storm water management system. At any time, the city is authorized to stop work on the installation of subdivision improvements, to embargo further issuance of building permits on a development, to stop work on any individual building or development of any individual building site, or to otherwise take steps necessary to protect the waters of the city from damage as a result of development. (2025 Code)

8-3-7: 	ADOPTION OF SUPPORTING DOCUMENTATION:  The city may 			     adopt, by resolution, design standards, definitions of terminology, administrative procedures, etc., intended to implement the general requirements and performance standards set forth in this chapter. Changes in the design standards may be accomplished by subsequently adopted resolutions. Such design standards may be complied within alternative ways that will contribute to rational achievement of the general requirements and performance standards set forth in this chapter; provided, that any such changes must be proposed by a qualified, licensed professional, subject to review and approval by the city. (2025 Code)

8-3-8: 	PROHIBITED CONDUCT:  No person shall damage, harm,  		    
          fail to install or complete or otherwise impair the grass infiltration areas, approved methods of transmission of storm water to grass infiltration areas or any portion of a storm water management system installed pursuant to this chapter. Unless other provisions are made in the process of development review and approval, responsibility for maintenance of storm water system elements remains with the property owner and violation of these maintenance requirements shall constitute a violation of this chapter. (2025 Code)

8-3-9: 	VARIANCE:  A variance from the requirements of this 			     chapter or from the design standards adopted pursuant to this chapter granting a variance due to unique site characteristics may only be granted in such circumstances if the approval of the variance would not otherwise impair achievements of the standards of purpose of this chapter, would not impose additional burden upon adjoining or downstream lands or landowners, or otherwise disrupt the scheme of storm water management in the community. It shall be incumbent upon anyone requesting a variance to provide data showing that alternative methods of storm water handling proposed will produce comparable efficacy of the storm water management measures required by this chapter. No variance shall be issued unless all elements of this section are met. Any request for variance from the terms of this chapter shall be first submitted to the city council who shall conduct a hearing in accord with the provisions of Idaho Code section 67-6516 and make the final decision concerning the variance. (2025 Code)

8-3-10: 	PENALTIES:  Provision of this chapter may be enforced 			     in the following manner:

A. 	Misdemeanor: Violation of the terms of this chapter may be 		considered a criminal misdemeanor and shall be punishable  	as provided in section 1-4-1 of this code. Each day of 			violation shall constitute a separate offense.

B. 	Additional Remedies: May be enforced by civil action,  	compel performance and completion of, or maintenance of, 
     facilities installed pursuant to this chapter.

C. 	Revocation Of Permit or Certificate: May have building 			permits or certificates of occupancy denied or revoked as 		the case may be. Occupancy of a dwelling or building 
     without an approved certificate of occupancy shall 
     constitute a violation of this chapter in addition to any 
     building or zoning ordinance from which the occupancy 
     requirement derives. (2025 Code)

CHAPTER 8-4

MANUFACTURED HOMES

SECTION:

8-4-1: 	Short Title

8-4-2: 	Authority

8-4-3: 	Purpose

8-4-4: 	Jurisdiction

8-4-5: 	Interpretation

8-4-6: 	Definitions

8-4-7: 	Requirements

8-4-8: 	Building Permit Fees

8-4-9: 	Enforcement; Penalty

8-4-1: 	SHORT TITLE:  This chapter shall be known and may be 			     cited as the MANUFACTURED HOME ORDINANCE OF THE CITY OF RIRIE. (2025 Code)

8-4-2: 	AUTHORITY:  These regulations are authorized by Idaho 			     Code title 50, chapter 13 and title 67, chapter 65 and the Idaho constitution article 12, section 2, as amended or subsequently codified. (20205 Code)

8-4-3: 	PURPOSE:  The purpose of this chapter is to regulate 			     the types of structures and placement of those structures within residential areas in the city. (2025 Code)

8-4-4: 	JURISDICTION:  These regulations shall apply to the 			     land within the corporate limits of the city as defined by the city and Jefferson and Bonneville Counties as well as any future legally annexed land added to the city corporate limits. (2025 Code)

8-4-5: 	INTERPRETATION:  The placement of all "manufactured 			     homes" as herein defined shall be submitted for approval to the planning and zoning committee and shall comply with the provisions of these regulations. These regulations shall supplement all other regulations, and where at variance with other laws, regulations, ordinances or resolutions, the more restrictive requirements shall apply. (2025 Code)

8-4-6: 	DEFINITIONS:  Terms or words used herein shall have 			     their common meaning with added defined terms in the following:

CURRENT LOCAL, 	Building code as established by the most STATE AND	FEDERAL	current copy of the international building BUILDING CODES:	code.

MANUFACTURED 		A. Any manufactured, relocatable home or HOME:			   dwelling unit, not designed for 
                       unrestricted highway use, built in a 
                       location removed from the home site and 
                       designed to be moved over public motor 
                       routes as a standard means of delivery. 
                       This would include, but is not limited 
                       to, homes designed for transport 					             after fabrication, on either their own 						   incorporated undercarriages with 
                       removable suspensions and wheels, or on 
                       flatbed or other trailers; or

B. A "manufactured home" or "mobile home" as 	   defined in Idaho Code section 39-4105.

MOBILE HOME PARK: 	   A tract of land subdivided according to  
                       the rules and regulations of the 
                       subdivision ordinance of the city, to 
                       provide for the sale, lease or rent of 
                       individual lots for the express purpose 
                       of placement of mobile or manufactured 
                       homes. (2025 Code)

8-4-7: 	REQUIREMENTS:  Manufactured homes are permitted within 		the city limits, outside of established mobile home parks, only under the following conditions:

A. 	Compliance Required: The manufactured home and its 			installation shall meet all local, state, and federal 			building codes and building permit requirements. The 			installation of the home shall comply with Idaho Code 			sections 44-2201 and 44-2202.

B. 	Siding: The home shall have exterior siding that is 			residential in character including, but not limited to, 		clapboards, simulated clapboards such as conventional metal 	or vinyl siding, or similar material. The siding shall not 		be composed of smooth, ribbed or corrugated metal, plastic, 	fiberglass, or composite panels.

C. 	Roof: The home shall have a wood shake, asphalt or nonglare 	roof with a minimum slope of 3/12 and eaves of at least six 	inches (6").

D. 	Hitches, Wheels and Axles: Any hitch(es), wheels, and 			axle(s) must be removed. (2025 Code)

E. 	Foundation:

1. The home shall be placed on a permanent foundation, 			matching the home's exterior dimensions, that complies with 	the international building code for residential structures. 	(2025 Code)

2. The foundation and foundation fascia must be similar in 		appearance and durability to the foundation of site built 		residential structures. (2025 Code)

8-4-8: 	BUILDING PERMIT FEES:

A. 	Fees: Rates for building permits for manufactured homes 
     will be equivalent to site-built homes.

B. 	Power To Establish and Change: Rates may be established and 	changed by resolution of the city council. (2025 Code)

8-4-9: 	ENFORCEMENT; PENALTY:

A. 	Issuance Of Permit: No permits for the placement of any 		manufactured home will be issued until the planning and 		zoning committee, or their duly assigned inspector, certify 	that all requirements of this chapter are met.

B. 	Civil Action: The city attorney may, in addition to taking 		whatever criminal action is deemed necessary, take steps to 	civilly enjoin any violation of this chapter.

C. 	Penalty: Violation of any of the provisions of this chapter 	or failure to comply with any of its requirements shall 		constitute a misdemeanor and be punishable as such. Each 		day such violation continues shall be considered a separate 	offense. The landowner, tenant, subdivider, builder, public 	official or any other person who commits, participates in, 		assists in or maintains such violation may each be found 		guilty of a separate offense. Nothing herein contained 
     shall prevent the city council or any other public official 
     or private citizen from taking such lawful action as is 			necessary to restrain or prevent any violation of this 			chapter or of the Idaho Code. (2025 Code)