TITLE 9

SUBDIVISION REGULATIONS

General Provisions 	9-1

9-1-1: Short Title							    pg. 3

9-1-2: Authority

9-1-3: Purpose

9-1-4: Jurisdiction

9-1-5: Interpretation

Definitions                                                  9-2

9-2-1: Interpretation Of Terms and Words		    pg. 5

9-2-2: General Definitions

Plats And Procedures 	9-3

9-3-1: Application For Subdivision Approval          pg. 14     
              Generally

9-3-2: Preliminary Plat

9-3-3: Final Plat

9-3-4: Record Of Lot; Plat Approval

9-3-5: Sale Of Lot; Plat to Be Recorded

9-3-6: Lot Splits/Minor Subdivisions

9-3-7: Fees

9-3-8: Appeals Process

Design Standards 	9-4

9-4-1: Minimum Standards Required				   pg. 26

9-4-2: Location

9-4-3: Street Widths

9-4-4: Intersections

9-4-5: Street Names

9-4-6: Pedestrian Walkways

9-4-7: Blocks

9-4-8: Lots

9-4-9: Easements

9-4-10: Reserve Strips

9-4-11: Public Sites and Open Spaces

Required Improvements	9-5

9-5-1: Improvements Required					   pg. 31

9-5-2: Monuments

9-5-3: Streets And Alleys

9-5-4: Curbs And Gutters

9-5-5: Street And Public Safety Signs

9-5-6: Streetlights

9-5-7: Sidewalks And Pedestrian Walkways

9-5-8: Underground Utilities

9-5-9: Water Supply, Sewer Systems and Solid Waste Disposal

9-5-10: Storm Drainage, Flood Controls

9-5-11: Fire Hydrants and Water Mains

9-5-12: Guarantee Of Improvements

9-5-13: Inspections

9-5-14: Inspector On Development Site

9-5-15: Failure To Complete Public Improvement Construction

9-5-16: Warranty Requirement

Administration And Enforcement	9-6

9-5-1: Improvements Required					   pg. 37

9-5-2: Monuments

9-5-3: Streets And Alleys

9-5-4: Curbs And Gutters

9-5-5: Street And Public Safety Signs

9-5-6: Streetlights

9-5-7: Sidewalks And Pedestrian Walkways

9-5-8: Underground Utilities

9-5-9: Water Supply, Sewer Systems and Solid Waste Disposal

9-5-10: Storm Drainage, Flood Controls

9-5-11: Fire Hydrants and Water Mains

9-5-12: Guarantee Of Improvements

9-5-13: Inspections

9-5-14: Inspector On Development Site

9-5-15: Failure To Complete Public Improvement Construction

9-5-16: Warranty Requirement

CHAPTER 1

GENERAL PROVISIONS

SECTION:

9-1-1: 	Short Title

9-1-2: 	Authority

9-1-3: 	Purpose

9-1-4: 	Jurisdiction

9-1-5: 	Interpretation

9-1-1: 	SHORT TITLE:  This title shall be known and may be 			     cited as the SUBDIVISION ORDINANCE OF THE CITY OF RIRIE. (2025 Code)

9-1-2: 	AUTHORITY:  These regulations are authorized by Idaho 			     Code title 50, chapter 13; title 67, chapter 65; and the Idaho constitution article 12, section 2, as amended or subsequently codified. (2025 Code)

9-1-3: 	PURPOSE:  The purpose of these regulations is to 				promote public health, safety and general welfare, and to provide for:

A. 	The harmonious development of the city and its area of 			impact.

B. 	The coordination of streets and roads within a subdivision 		with other existing or planned streets and roads.

C. 	Adequate open space for travel, light, air and recreation.

D. 	Adequate transportation, water drainage and sanitary 			facilities.

E. 	The avoidance of the scattered subdivision of land that 		would result in either of the following:

1. The lack of water supply, sewer service, drainage, 			transportation or other public services; or

2. The unnecessary imposition of an excessive expenditure 
     of public funds for the supply of such services;

F. 	Requirements as to the extent and the manner in which:

1. Roads shall be created, improved and maintained; and

2. Water and sewer and other utility mains, piping 			connections, or other facilities shall be installed.

G. 	The manner and form of making and filing of any plats; and

H. 	The administration of these regulations by defining the 		powers and duties of approving authorities. 					(2025 Code)

9-1-4: 	JURISDICTION:  These regulations shall apply to the 			     subdivision of land within the corporate limits of the city.(2025 Code)

9-1-5: 	INTERPRETATION:  All "subdivisions" as herein defined 			     shall be submitted for approval to the planning and zoning commission and shall comply with the provisions of these regulations. These regulations shall supplement all other regulations, and where at variance with other laws, regulations, ordinances or resolutions, the more restrictive requirements shall apply. (2025 Code)

CHAPTER 2

DEFINITIONS

SECTION:

9-2-1: 	Interpretation Of Terms and Words

9-2-2: 	General Definitions

9-2-1: 	INTERPRETATION OF TERMS AND WORDS:  Terms or words 
          used herein shall be interpreted as follows:

A. 	The present tense includes the past or future tense, the 		singular includes the plural, and the plural includes the 		singular.

B. 	The word "shall" is mandatory; the word "may" is 
     permissive; and the word "should" is preferred.

C. 	The masculine shall include the feminine. 					(2025 Code)

9-2-2: 	GENERAL DEFINITIONS:  As used in this title, the 				     following words and terms shall have the meanings ascribed to them in this section:

ADMINISTRATOR: 	The administrator or official of this title 					     shall be an individual designated by the 
                    city council or the city of Ririe Planning 
                    and Zoning Commission.

ALLEY: 			A minor street providing secondary access at 				the back or side of a property otherwise 					     abutting a street. Minimum alley width to be 				sixteen feet (16') and maximum to be twenty 				     	feet (20') with uniform alleys throughout  
                    the subdivision.

ARTERIAL STREET: 	A street designated for the purpose of 						     carrying fast and/or heavy traffic.  
                    Depending on the location, arterial streets 
                    will comply with the standards of city of 
                    Ririe or Jefferson or Bonneville County.

BLOCK: 			A group of lots, tracts, or parcels within 					     well defined boundaries, usually streets. 					     Blocks conform to a standard five hundred 				          twenty-five feet by two hundred fifty feet 					
                    (525' x 250') excluding alleys.

BUILDING: 		A structure having a roof supported by 						     columns and/or walls designed or used as the 				living quarters for one or more families, or 				a structure designed for use as an accessory 				to a structure used for living quarters or a 				structure designed for commercial or 						     industrial uses.

BUILDING SETBACK: 	The shortest distance between the property 					     line and the foundation, wall or frame of a 					     permanent building or accessory structure.

BUILDING SITE: 	An area proposed or provided and improved by 				grading, filling, excavation or other means 					     for erecting pads for buildings.

CEMETERY: 		A lot that has been platted for the selling 					     of sites for the burial of animal or human 					     remains.

CITY: 			City of Ririe, Idaho.

CITY COUNCIL: 		The city council of Ririe, Idaho.

COLLECTOR STREET: 	A street designated for the purpose of 						     carrying traffic from minor streets to other 				collector streets and/or arterial streets 
                    and shall be in accordance with existing 
                    streets and specifications of the city of 
                    Ririe.

COMPREHENSIVE 		An adopted document that herein may be   PLAN:			referred to as a comprehensive plan.

CONDOMINIUM: 		An estate consisting of an undivided 
                    interest in common in real property, in an 
                    interest or interests in real property, or 
                    in any combination thereof; together with a 
                    separate interest in real property, in an 
                    interest or interests in real property or in 
                    any combination thereof.

COUNTY RECORDER: 	The office of the Jefferson and/or 
                    Bonneville County recorder.

COVENANT: 		A written promise or pledge.

CUL-DE-SAC: 		A street connected to another street at one 					     end only and provided with a turnaround  
                    space at its terminus.

CULVERT: 			A drain that channels water under a bridge, 					     street, road or driveway.

DEAD END STREET: 	A street connecting to another street at one 				end only and not having provision for 						     vehicular turn-around at its terminus.

DEDICATION: 		The setting apart of land or interests in 					     land for use by the public by ordinance, 					     resolution or entry in the official minutes 					     as by the recording of a plat. Dedicated        
                    land becomes public upon the acceptance by 
                    the city.

DEVELOPER: 		Authorized agent(s) of a subdivider or the 					     subdivider himself.

DWELLING UNIT: 	Any building or other structure proposed or 					     built for occupancy by people.

EASEMENT: 		A grant by a property owner to specific 						persons or to the public to use land for 					     specific purposes. Also, a right acquired by 				prescription.

ENGINEER: 		Any person who is licensed in the state to 					     practice professional engineering.

FLOODPLAIN: 		The relatively flat area or lowland 
                    adjoining the channel of a river, stream, 
                    lake or other body of water which has been 
                    or may be covered by water of a flood of 
                    100-year frequency. The floodplain includes 
                    the channel, floodway or floodway fringe, as 				established pursuant to engineering 
                    practices of the U.S. army corps of 
                    engineers, as follows:

Channel: 	  	A natural or artificial watercourse of 						     perceptible extent, with definite bends and 					     banks to confine and conduct continuously or 				periodically flowing water.

Flood: 		The temporary inundation of land adjacent to 				and inundated by overflow from a river, 						stream, lake or other body of water.

Flood Of 		A flood magnitude that has a one percent 
   100-Year         (1%) chance of being equaled or exceeded in 
   Frequency:		given year.

Floodway: 		The channel of a river or other watercourse 					     and the adjacent land areas that must be 					     reserved to discharge the base flood without 
                    cumulatively increasing the water surface 
                    elevation more than one foot (1').

Floodway 		That part of the floodplain that is beyond

Fringe:		the floodway. Such areas include those 						     portions of the floodplain that will be 						inundated by a flood of 100-year frequency.

FRONTAGE STREET: 	A minor street, parallel to and adjacent to 					     an arterial street, to provide access to 					     abutting properties and shall be in 						     accordance with existing streets and 						     specifications of the city of Ririe.

GOVERNING BODY: 	The city council of the city of Ririe, 
                    Idaho.

HIGHWAY: 			The street designated as highway by an 						     appropriate state or federal agency.

IMPROVEMENT: 		Any alteration to the land or other physical 				construction associated with subdivision and 				building site developments.

LARGE SCALE 		A development consisting of more than twenty DEVELOPMENT:		(20) homes or an area larger than five 						     hundred twenty five feet by two hundred 
                    fifty feet (525' x 250').

LOT: 			A parcel, plot, tract or other land area of 					     suitable size as required in these 							regulations and any existing zoning 						     ordinance; and created by subdivision for 					     sale, transfer or lease. Lot size to conform 				to seventy-five feet by one hundred twenty-
                    five feet (75' x 125').

LOT AREA: 		The area of any lot shall be determined 						exclusive of street, highway, alley, road or 				rights of way of record.

LOT DEPTH: 		The horizontal distance between the front 
                    and rear lot lines.

LOT TYPES: 		As used in these regulations, lot types are 					     as follows:

Corner Lot: 	A lot located at the intersection of two (2) 				or more streets.

Interior Lot: 	A lot other than a corner lot with frontage 					     on only one street other than a corner lot.

Through Lot: 	A lot with frontage on more than one street 					     other than a corner lot.

MAJOR 		 	Any subdivision not able to qualify as a SUBDIVISION:		minor subdivision including, but not limited 				to, a subdivision of ten (10) or more lots, 					     or any size subdivision requiring any new 					     street or extension of the local government 					     facilities, or the creation of any public 					     improvements.

MINOR STREET: 		A street, which has the primary purpose of 					     providing access to abutting properties and 				          shall be in accordance with existing streets 				and specifications of the city of Ririe.

MINOR 			Any subdivision containing not more than ten SUBDIVISION:		(10) lots fronting on an existing street; not 				involving any new street or road, or the 					     extension of municipal facilities, or the 					     creation of any public improvements; and not 				adversely affecting the remainder of the 					     parcel or adjoining property; and not in 					     conflict with any provision or portion of 
                    any city comprehensive plan, zoning 
                    ordinance, or this title.

MOBILE HOME: 		A manufactured, relocatable dwelling unit, 					     not designed for unrestricted highway use, 					     which may or may not meet the minimum 						     requirements of the international building 					     code.

A. Designed for long-term occupancy and 			containing sleeping accommodations, a flush 		toilet, a tub or shower, bath and kitchen 		facilities with plumbing and electrical 			connections provided for attachment to 			outside systems.

B. Designed to be transported after 			fabrication on its own wheels, or on flatbed 	or other trailers, or detached wheels; and

C. Arrived at the site where it is to be 		occupied as a dwelling unit, prepared for 		occupancy except for minor and incidental 		unpacking and assembly operations, location 		on foundation supports, a connection to 			utilities and the like.

MOBILE HOME 		A tract of land subdivided according to the SUBDIVISION:		rules and regulations of this title to 						     provide for the sale, lease or rent of 						     individual lots for the express purpose of 					     placement of mobile homes.

MONUMENT: 		Any permanent marker either on concrete, 					     galvanized iron pipe or iron or steel rods, 					     used to identify any tract, parcel, lot or 					     street lines, as specified in Idaho Code 					     section 50-1303.

OPEN SPACE: 		An area open to the sky for outdoor 						     recreation activity exclusive of streets, 					     buildings or other covered structures.

ORIGINAL PARCEL 	An original lot or tract as recorded on any OF LAND:			plat of record on file in the office of the 					     county recorder or any unplatted contiguous 					     parcel of land held in one ownership as of 					     the date of adoption hereof.

OWNER: 			An individual, firm, association, syndicate, 				partnership or corporation having any 						     interest, legal or equitable, in the land.

PARTIAL 			A dedicated right of way providing only a RIGHT OF WAY:		portion of the required street width, 
                    usually along the edge of a subdivision or 
                    tract of land.

PLANNED UNIT: 		A subdivision designed as a combination of 					     residential, commercial and industrial uses 					     or any combination thereof planned for a 					     tract of land to be developed as a unit 
                    under single ownership or control, which is 						developed for the purpose of selling 						     individual lots or estates, whether fronting 				on private or dedicated streets, which may 					     include two (2) or more principal buildings.

PLANNING AND 		A group of individuals appointed by the city ZONING COMMISSION:	council to administer planning and zoning 					     ordinances for the city of Ririe and the 					     impact area.

PLAT: 			The drawing, mapping or planning of a 						     subdivision, cemetery, town site or other 					     tract of land or a replatting of such 						     including certifications, descriptions and 					     approvals including the following:

Final Plat: 	The final and formal presentation by drawing 				of an approved subdivision development, the 				          original and one copy of which is filed with 				the county clerk and recorder.

Preliminary 	The first formal presentation by drawings of

Plat:			a proposed subdivision.

PRIVATE STREET: 	A street that is not accepted for public use 				or maintenance, which provides vehicular and 				pedestrian access and shall be in accordance 				with existing streets and specifications of 					     the city of Ririe.

RESERVE STRIP: 	A strip of land between a partial street and 				adjacent property, which is reserved or held 				in public ownership for future street 						     extension or widening.

RIGHT OF WAY: 		A strip of land dedicated or reserved for 
                    use as a public way, which normally included 				streets, sidewalks and other public 
                    utilities or service areas.

STANDARD 			The specifications as specified in this  SPECIFICATIONS:	title or as officially adopted by the city.

STATE: 			The state of Idaho.

STREET: 			A right of way which provides access to 						adjacent properties, the dedication of which 				has been officially accepted. The term 						     "street" also includes the terms highway, 					     thoroughfare, parkway, road, avenue, 						     boulevard, land, place and other such terms. 				Streets shall conform to established 						     ordinances of the city of Ririe.

SUBDIVIDER: 		The individual, firm, corporation, 							partnership, association, syndicate, trust 
                    or other legal entity that executes the 						application and initiates proceedings for 
                    the subdivision of land in accordance with 
                    the provisions of this title. The subdivider 
                    need not be the owner of the property; 
                    however, he shall be the agent of the owner 
                    or have sufficient proprietary rights in the 
                    property to represent the owner.

SUBDIVISION: 		The result of an act of dividing an original 				lot, tract or parcel of land into more than 					     two (2) parts for the purpose of transfer of 				ownership or development, which shall also 					     include the dedication of a public street 
                    and the addition to, or creation of, a 							"cemetery", subdivisions and large scale 					     developments, as those terms are defined in 					     this section. However, this title shall not 					     apply to any of the following:

A. An adjustment of lot lines as shown on a 		recorded plat which does not reduce the    
     area, frontage, width, depth or building    
     setback lines of each building site below 
     the minimum zoning requirements, and does 
     not change the original number of lots in 
     any block of the recorded plat;

B. An allocation of land in the settlement 
     of an estate of a decedent or a court decree 
     for the distribution of property;

C. The unwilling sale of land as a result of 	"legal condemnation" as defined and allowed 		in the Idaho Code;

D. Widening of existing streets to conform 
     to the comprehensive plan;

E. Acquisition of street rights of way by a 		public agency in conformance with the 			comprehensive plan; and

F. The exchange of land for the purpose of 		straightening property boundaries which does 	not result in the change of present land 		usage.

SURVEYOR: 		Any person who is licensed in the state as a 				public land surveyor to do professional 						surveying.

UTILITIES: 		Installations for conducting water, sewage, 					     gas, electricity, television, storm water  
                    and similar facilities providing service to 
                    and used by the public.

VARIANCE: 		A modification of the requirements of this 					     title as to lot size; lot coverage; width; 					     depth; front yard, side yard, or rear yard 					     setbacks; parking spaces; height of 						     buildings; or other ordinance provisions 					     affecting the size or shape of a structure   
                    or the placement of the structure upon lots, 
                    or the size of lots. A variance shall not be 				considered a right or special privilege but 				          may be granted to an applicant only upon a 					     showing of undue hardship because of 						     characteristics of the site and the variance 				is not in conflict with the public interest.

VICINITY MAP:		A small-scale map showing the location of a 					     tract of land in relation to a larger area. 					     (2025 Code)

CHAPTER 3

PLATS AND PROCEDURES

SECTION:

9-3-1: 	Application For Subdivision Approval Generally

9-3-2: 	Preliminary Plat

9-3-3: 	Final Plat

9-3-4: 	Record Of Lot; Plat Approval

9-3-5: 	Sale Of Lot; Plat to Be Recorded

9-3-6: 	Lot Splits/Minor Subdivisions

9-3-7: 	Fees

9-3-8: 	Appeals Process

9-3-1: 	APPLICATION FOR SUBDIVISION APPROVAL GENERALLY:  Any 			     person desiring to create a "subdivision" as herein defined shall submit all necessary applications to the city planning and zoning commission. (2025 Code)

9-3-2: 	PRELIMINARY PLAT:

A. 	Filing Of Preliminary Plat Application and Data: The 			subdivider shall file with the administrator a complete 		subdivision application form and preliminary plat data as 		required in this title.

B. 	Combining Preliminary and Final Plat In Minor Subdivisions: 	The applicant may request that the subdivision be processed 	as both a preliminary and final plat if all the following 		exist:

1. The proposed subdivision does not exceed four (4) lots;

2. No new street dedication or street widening is involved;

3. No major special development considerations are involved 	such as, development in a floodplain, hillside development 		or the like;

4. All required information for both preliminary and final 		plat is complete and in an acceptable form; and

5. The proposed subdivision is not in conflict with any 		comprehensive plan or zoning ordinance of the city.

A request to combine both preliminary plat and final plat into one application shall be acted upon by the council upon recommendation of the zoning administrator.

C. 	Required Information and Data:

1. The contents of the preliminary plat and related 			information shall be in such form as stipulated by the 			planning and zoning commission; however, any additional   
     maps or data deemed necessary by the planning and zoning 
     commission may also be required.

2. The subdivider shall submit to the administrator at 
     least the following:

a. One copy of the preliminary plat of the proposed 		  
          subdivision, drawn in accordance with the requirements 		hereinafter stated. Each copy of the preliminary plat 
          shall be on good quality paper, shall have the 
          dimensions of not less than twenty-four inches by 
          thirty-six inches (24" x 36"), shall be drawn to a 
          scale of not less than one inch to one hundred feet 
          (1" = 100'), shall show the drafting date, and shall 
          indicate thereon, by arrow, the general northerly 
          direction;

b. One set of preliminary engineering plans (not meant 		to be cross sections or detailed designs) for streets, 		water, sewers, sidewalks and other required public 		   	     improvements; however, such engineering plans shall   
          contain sufficient information and detail to make a 
          determination as to conformance of the proposed 
          improvements to applicable regulations, ordinances and 
          standards;

c. A written application requesting approval of the 		   
          preliminary plan; and

d. Appropriate information that sufficiently details 		  
          the proposed development within any special 
          development area such as planned unit development, 
          floodplain, cemetery, mobile home, large scale 
          development, and hazardous and unique areas of 
          development.

3. The following shall be submitted separately:

a. The name of the proposed subdivision;

b. The name, address and telephone numbers of the 			     subdivider or subdividers and the engineer or surveyor 
          who prepared the plat;

c. The name and address of all adjoining owners of 		   
          property and residents within three hundred feet 
          (300') of the external boundaries of the land being 
          considered, whether or not bisected by a public right 
          of way, as shown on record in the county assessor's 
          office;

d. The legal description of the subdivision;

e. The statement of the intended use of the proposed 		   
          subdivision, such as residential single-family and 
          multiple housing, commercial, industrial, recreational 
          or agricultural and a designation of any sites 
          proposed for parks, playgrounds, schools, churches or 
          other public uses;

f. A map of the entire area scheduled for development 		  
          if the proposed subdivision is a portion of a larger 
          holding intended for subsequent development. A map 
          shall be submitted showing the location of existing 
          buildings, water bodies or courses and the location of 
          currently dedicated streets at the point where they 
          adjoin and/or are immediately adjacent; provided, that 
          actual measured distances shall not be required;

g. A vicinity map showing the relationship of the 			     proposed plat to the surrounding area (1/2 mile radius, 		scale option);

h. The land use and existing zoning of the proposed 		   
          subdivision and the adjacent land;

i. Streets, street names, rights of way and roadway 		 
          widths, including adjoining streets or roadways;

j. Lot lines and blocks showing the dimensions and 		
          numbers of each;

k. Contour lines, shown at five-foot (5') intervals 		
          where land slope is greater than ten percent (10%) and 
          at twenty-foot (20') intervals where land slope is ten 
          percent (10%) or less, referenced to an established 
          bench mark, including location, elevation and 
          topography;

l. A site report as required by the appropriate health 		district where individual wells or septic tanks are 			     proposed;

m. Any proposed or existing utilities, including, but 		   
          not limited to, storm and sanitary sewers, irrigation 			     laterals, ditches, drainages, bridges, streetlamps, 			     culverts, water mains, fire hydrants and their 
          respective profiles;

n. Any dedications to the public and/or easements, 		   
          together with a statement of location, dimensions and 			     purpose of such;

o. A statement as to whether or not a variance will be 		requested with respect to any provision of this title 			     describing the particular provision, the variance 
          requested and the reasons therefor;

p. Prior to the plat being approved, the following 		   
          agencies must respond, in writing, about any concerns 
          they have for the proposed development: fire 
          department, police, school board and city council.

4. The final plat shall clearly show the following:

a. Primary control points, approved by the planning   
          and zoning commission, or a description and ties to 
          such control points, to which all dimensions, angles, 
          bearings and similar data on the plat shall be 
          referenced. Such primary control points must include 
          either a section corner, quarter section corner, or 
          meander corner according to government survey.

b. Location and description of monuments.

c. Tract boundary lines, property lines, lot lines, 
          and right of way lines of streets, easements and other 
          rights of way, with accurate dimensions, bearings or 
          deflection angles and radii, arcs and center angles of 
          all curves.

d. Names and locations of adjoining subdivisions.

e. The length of all lot lines dimensioned in feet and 		decimals thereof, and value of all required true 
          bearings and angles dimensioned in degrees, minutes 
          and seconds.

f. The blocks numbered consecutively throughout the 		  
          entire subdivision and lots numbers consecutively 
          throughout each block, with areas to be excluded from 
          platting marked "Reserved" or "Not A Part".

g. The outline of any property, other than streets or 		
          alleys, by lengths and bearings, with the area marked 			
          "Public" and showing the proposed use.

h. A title which shall include the name of the 			  
          subdivision, name of the city, county and state, and 
          the location and description of the subdivision, 
          referenced to section, township and range.

i. Scale, north arrow and date.

j. Final plat shall be lettered with readable print 		
          whether it be CAD (computer aided design) system, 
          Leroy pen set or other similar mechanical lettering 
          set.

D. 	Procedure For Approval Of Preliminary Plat:

1. Administrative Review:

a. Certification by Administrator; Public Hearing: 
          Upon receipt of the preliminary plat and all other 
          required data as provided for herein, the 
          administrator shall certify the application as 
          complete and shall affix the date of application 
          acceptance thereon. The administrator shall schedule a 
          public hearing, which hearing shall be held within a 
          reasonable time from the date of certification of a 	
          complete application.

b. Review by Other Agencies: The administrator shall 		  
          refer the preliminary plat and application to as many 			     governmental agencies as deemed necessary. Such  
          agencies may include the following:

(1) Other governing bodies having joint 						jurisdiction;

(2) The appropriate utility companies, irrigation 			companies or districts and drainage districts;

(3) The school board of the school district; and

(4) Other agencies having an interest in the 					proposed subdivision.

c. Recommendation by Administrator: The administrator  
          shall provide that any transmittal as provided in 
          subsection D1b of this section will be returned within 
          fifteen (15) days. At the next regular scheduled 
           meeting as time permits, the administrator shall  
           review all responses to the planning and            
           zoning commission.

2. Public Notice: The administrator shall notify all

adjoining property owners whose names and addresses have 
     been provided by the subdivider. Such written notice shall  
     be mailed by first class mail at least fifteen (15) days 
     prior to the planning and zoning board meeting. The 
     planning and zoning commission’s failure to comply with the 
     notice provision shall not invalidate the commission’s 
     action, provided the spirit of the procedure is observed.

3. Action By Planning and Zoning Commission:

a. Planning and Zoning Commission’s Findings: In 
          determining the acceptance of a proposed subdivision, 
          the planning and zoning commission shall consider the 
          objectives of this title and at least the following:

(1) The conformance of the subdivision with the 				      comprehensive development plan;

(2) The availability of public services to 					     accommodate the proposed development;

(3) The continuity of the proposed development 				     with any capital improvement program;

(4) The public financial capability of supporting 			services for the proposed development; and

(5) The other health, safety and environmental 				     problems that may be brought to the planning and 				     zoning commission’s attention.

b. Action on Preliminary Plat: Upon granting or 
          denying a preliminary plat, the planning and zoning 
          commission shall specify:

(1) The ordinance and standards used in  
               evaluating the application;

(2) The reasons for recommending approval or 					denial; and

(3) The actions, if any, that the applicant could 			take to obtain approval of the preliminary plat.

c. Action on Combined Preliminary And Final Plat: If 		  
          the planning and zoning commission's conclusion is 
          favorable to the subdivider's request for the  
          subdivision to be considered as both a preliminary 
          plat and final plat, then action shall be taken in the 
          same manner as herein specified for the final plat. 
          The planning and zoning commission may approve, 
          approve conditionally or disapprove the combined 
          application.

E. 	Approval Period:

1. Failure to file with and obtain the certification of the 	acceptance of the final plat application by the coordinator 	within six (6) months after action by the planning and 			zoning commission shall cause all approvals of said 
     preliminary plat to be null and void, unless an extension  
     of time is applied for prior to its expiration by the 
     subdivider and granted by the planning and zoning 
     commission.

2. In the event that the development of the preliminary 
     plat is made in successive, contiguous segments in an  
     orderly and reasonable manner, and conforms such segments, 
     if submitted within successive intervals for one-half (1/2) 
     year, it may be considered for final approval without 
     resubmission for preliminary plat approval. (2025 Code)

9-3-3: 	FINAL PLAT:

A. 	Filing Of Final Plat: After the approval or conditional 		approval of the preliminary plat, the subdivider may cause 		the total parcel or any part thereof to be surveyed and a 		final plat prepared in accordance with the approved 			preliminary plat. The subdivider shall submit to the 			coordinator the following:

1. Three (3) copies of the final plat; and

2. Three (3) copies of the final engineering construction 		drawings for streets, water, sewers, sidewalks and other 		public improvements.

B. 	Contents: The final plat shall include and be in compliance 	with all items required under Idaho Code title 50, chapter 		13 and shall be prepared in accordance with the 				specifications set forth in Idaho Code section 50-1304. The 	final plat shall include at least the following:

1. A written application for approval of such final plat as 	stipulated by the planning and zoning commission;

2. Proof of current ownership of the real property included 	in the proposed final plat;

3. Such other information as the planning and zoning 
     commission may deem necessary to establish whether or not 
     all proper parties have signed and/or approved said final 
     plat;

4. Conformance with the approved preliminary plat and 			meeting all requirements or conditions thereof;

5. Conformance with all requirements and provisions of this 	title;

6. Conformance with acceptable engineering practices and 		local standards; and

7. Building sites on each lot adjusted to setback 				requirements.

C.   Procedure For Approval of Final Plat:

1. Review By Planning and Zoning Commission:

a. Acceptance: Upon receipt of the final plat, and 		 
          compliance with all other requirements as provided for 		herein, the administrator shall certify the 
          application as complete and shall affix the date of 
          acceptance thereon.

b. Submission of Final Plat: The planning and zoning 		
          commission shall review the final plat for compliance 
          with the approved or conditionally approved 
          preliminary plat. If the planning and zoning  
          commission determines that there is substantial 
          difference in the final plat than that which was 
          approved as a preliminary plat or conditions which 
          have not been met, the planning and zoning commission 
          may require that the final plat be submitted in the 
          same manner as required in the preliminary plat 
          process. Upon the determination that the final plat is 
          in compliance with the preliminary plat and all 
          conditional requirements have been met, the planning 
          and zoning commission shall place the final plat on 
          the planning and zoning commission agenda at the next 
          regular scheduled meeting as time permits.

2. Agency Review: The administrator shall transmit one copy 
     of the final plat, or other documents submitted, for review 
     and recommendation to the departments and agencies, as he 
     deems necessary to ensure compliance with the preliminary 
     approval and/or conditions of preliminary approval. Such 
     agency review should also include the construction 
     standards of improvements, compliance with health 
     standards, the cost estimate for all improvements and the 
     legal review of the performance bond.

3. Planning And Zoning Commission Action: The planning and    
     zoning commission, at its next meeting following receipt of 
     any agency report, shall consider any comments from 
     concerned persons and agencies to arrive at a decision on 
     the final plat. The planning and zoning commission 
     shall approve, conditionally approve, disapprove or table 
     the final plat for additional information within sixty (60) 
     days of the date of the first regular meeting at which the 
     plat is considered. Upon granting or denying the final 
     plat, the planning and zoning commission shall specify:

a. The ordinance and standards used in evaluating the 		 
          application;

b. The reasons for approval or denial; and

c. The actions, if any, that the applicant could take 		
          to obtain a permit.

D. 	Conditional Approval of Final Plat: With respect to 			financial guarantees, the approval of all final subdivision 	plats shall be conditioned on the accomplishment of the 		following:

1. The construction of improvements required by this title 		shall be completed by the subdivider and approved by the 		planning and zoning board; and

2. Surety acceptable to the planning and zoning board shall 	have been filed with the coordinator in the form of a cash 		deposit, certified check, negotiable bond, irrevocable bank 	letter of credit or surety bond.

E. 	Approval Period: The final plat shall be filed with the 		county recorder within one hundred eighty (180) days after 		written approval by the planning and zoning commission. 
     Otherwise such approval shall become null and void unless 
     the subdivider, prior to said expiration date, applies for 
     an extension of time and such extension is granted by the 		planning and zoning commission.

F. 	Required Certificates: The following certifications and 		signatures shall be included on the final plat prior to 		recording by the county recorder:

1. Certification and signature of the city clerk-treasurer 		verifying that the subdivision meets the requirements of 
     the city and has been approved by the planning and zoning 
     commission; and

2. Certification of the sanitation restriction on the face 		of the plat is in accordance with the provisions of Idaho 		Code section 50-1326.

G. 	Record Of Final Plat: Upon approval of the final plat by

the planning and zoning commission, the subdivider's    
     prepayment of recording fees, posting of surety bond or 
     other acceptable guarantee and the inclusion of those 
     certifications and signatures on the final plat as set 
     forth in subsection F of this section, the subdivider shall 
     furnish proof to the coordinator that the final plat has 
     been recorded. (2025 Code)

9-3-4: 	RECORD OF LOT; PLAT APPROVAL:  No final plat shall be 			     filed with the county recorder or dwelling units constructed on the property until the plat has been approved by the planning and zoning commission. (2025 Code)

9-3-5: 	SALE OF LOT; PLAT TO BE RECORDED:  No lots shall be 			     sold until the plat has been recorded in the office of the county recorder. (2025 Code)

9-3-6: 	LOT SPLITS/MINOR SUBDIVISIONS:  Any time that an 				     original tract of land, a lot of a recorded plat, or any other parcel of land is divided or partitioned into not more than two (2) lots, parcels, or sites for the purpose of transfer of ownership or development, the following application process and procedures shall be followed:

A. 	Application: An application for lot split on a form   
     provided by the administrator shall be made and shall  
     contain, at a minimum, the following information:

1. Name, address, telephone number of the applicant.

2. Legal description of property and proof of ownership or 		agency.

3. Description of existing use.

4. A list of adjoining property owners within one hundred 		feet (100') of the property line.

5. Description of lots after proposed lot split.

6. Description of proposed use.

7. Documentation of the permission of the owner of the 			parcel to be split.

B. 	Procedure: Prior to approving the application under this 		section of this chapter, the planning and zoning commission 
     shall make the following findings:

1. The minimum requirements under the city code for  
     acreage, density, lot area and setback have been complied 
     with;

2. The proposed lot split is in conformity with the 			comprehensive plan; and

3. The proposed lot split is in the public interest and 
     will not adversely impact adjoining property interests.

C. 	Notice: Within ten (10) days after a decision has been 			rendered, the administrator shall provide the applicant 
     with written notice of the action on the request.

(2025 Code)

9-3-7: 	FEES:  Fees shall be paid at the time of submission of an application for a preliminary plat, application for a final plat, lot split, or a minor subdivision.  There shall be no fees assessed for initial meetings and consultations to work on these, including Development Agreements, with city staff and/or city officials. Fees shall be assessed for any additional meetings requested by the applicant with city staff and/or city officials.  These fees shall be established by resolution of the city council.  (2025 Code)

9-3-8: 	APPEALS PROCESS:

A. 	Notice: Petitioner must notify city clerk-treasurer within 		fifteen (15) days that they want to appeal the ruling.

B. 	Council Action: The city council shall have the option to 		address the appeal as follows:

1. By calling a special meeting for this purpose within 		sixty (60) days of receiving the appeal; or

2. By placing the appeal on the agenda of the second 			regularly scheduled council meeting after the appeal has 		been received.

C. 	Final Decision: The city council should make a final 			decision on the ruling at the next regularly scheduled 			meeting following the petition hearing.

(2025 Code)

CHAPTER 4

DESIGN STANDARDS

SECTION:

9-4-1: 	Minimum Standards Required

9-4-2: 	Location

9-4-3: 	Street Widths

9-4-4: 	Intersections

9-4-5: 	Street Names

9-4-6: 	Pedestrian Walkways

9-4-7: 	Blocks

9-4-8: 	Lots

9-4-9: 	Easements

9-4-10: 	Reserve Strips

9-4-11: 	Public Sites and Open Spaces

9-4-1: 	MINIMUM STANDARDS REQUIRED:  All plats submitted 				     pursuant to the provisions of this title, and all subdivisions, improvements and facilities done, constructed or made in accordance with said provisions shall comply with minimum design standards set forth in this chapter; provided, however, that any higher standards adopted by any highway district, the Idaho transportation department or health agency shall prevail over those set forth herein. (2025 Code)

9-4-2: 	LOCATION:  Street and road location shall conform to 			the following standards:

A. 	Street Location and Arrangements: When an official street 		plan or comprehensive development plan has been adopted, 		subdivision streets shall conform to such plans.

B. 	Minor Streets: Minor streets shall be so arranged as to 		discourage their use by through traffic.

C. 	Stub Streets: Where adjoining areas are not subdivided, the 	arrangement of streets in new subdivisions shall be such 		that said streets extend to the boundary line of the tract 		to make provisions for the future extension of said streets 	into adjacent areas and shall have a cul-de-sac or 			temporary cul-de-sac. A reserve street may be required and 		held in public ownership.

D. 	Relation To Topography: Streets shall be arranged in proper 	relation to topography so as to result in usable lots, safe 	streets and acceptable gradients.

E. 	Alleys: Alleys shall be provided in multiple dwelling or 		commercial subdivisions unless other provisions are made 
     for	service access and off-street loading and parking. 
     Dead end alleys shall be prohibited in all cases.

F. 	Frontage Roads: Where a subdivision abuts or contains an 		arterial street, it shall be required that there be 
     frontage roads approximately parallel to and on each side 
     of such arterial street; or such other treatment as is 
     necessary for the adequate protection of residential 
     properties and to separate through traffic from local 
     traffic. Roads in a subdivision shall not come off a main 
     road.

G. 	Cul-De-Sac Streets: Cul-de-sac streets shall not be more 		than five hundred feet (500') in length and shall terminate 	with an adequate turnaround having a minimum radius of one 		hundred feet (100') for right of way.

H. 	Half Streets: Half streets shall be prohibited except where 	unusual circumstances make such necessary to the reasonable 	development of a tract in conformance with this title and 		where satisfactory assurance for dedication of the 
     remaining part of the street is provided. Whenever a tract 
     to be subdivided borders on an existing half or partial 
     street, the other part of the street shall be dedicated 
     within such a tract. (2025 Code)

9-4-3: 	STREET WIDTHS:

A. 	Compliance Required: Street and road right of way widths, 		grade, alignment, and so forth, shall conform to the 
     adopted major street plan, zoning ordinance and 
     comprehensive plan, and the rules of the Idaho 
     transportation department and/or other agency having 
     jurisdiction.

B. 	Access From Frontage Road onto Arterial Street: 				Notwithstanding subsection A of this section, access from a 	frontage road onto an arterial street shall be limited to 		one thousand five hundred feet (1,500') between points. 		(2025 Code)

9-4-4: 	INTERSECTIONS:  Intersections shall conform to the 			     following standards:

A. 	Angle Of Intersection: Streets shall intersect at ninety 		degrees (90̊) or as closely thereto as possible, and, in no 	case, shall streets intersect at less than seventy degrees 		(70̊).

B. 	Sight Triangles: Minimum clear sight distance at all minor 		street intersections shall permit a vehicle to be visible 
     to the driver of another vehicle when each is one hundred 
     feet (100') from the center of the intersection.

C. 	Number Of Streets: No more than two (2) streets shall cross 	at any one intersection.

D. 	T Intersections: T intersections may be used wherever such 		design will not restrict the free movement of traffic.

E. 	Centerline Offsets: Street centerline shall be offset by a 		distance of at least one hundred twenty-five feet (125').

F. 	Vertical Alignment of Intersection: A nearly flat grade 
     with appropriate drainage slopes is desirable within 				intersections. This flat section shall be extended a 
     minimum of one hundred feet (100') each way from the 
     intersection. An allowance of two percent (2%) maximum 
     intersection grade in rolling terrain, and four percent 
     (4%) in hilly terrain, will be permitted. (2025 Code)

9-4-5: 	STREET NAMES:  The naming of streets shall comply with 		city and/or Jefferson/Bonneville Counties street naming and street numbering system. (2025 Code)

9-4-6: 	PEDESTRIAN WALKWAYS:  Right of way for pedestrian 				walkways in the middle of long blocks may be required where necessary to obtain convenient pedestrian circulation to schools, parks or shopping areas. The pedestrian easement shall be at least ten feet (10') wide. (2025 Code)

9-4-7: 	BLOCKS:  Every block shall be designed to provide 			     two (2) tiers of lots, except where lots back onto an arterial street, natural feature or subdivision boundary. Blocks shall not be less than five hundred twenty-five feet by two hundred fifty feet (525' x 250') without alley. (2025 Code)

9-4-8: 	LOTS:  Lots shall conform to the following standards:

A. 	Zoning: Lots within any subdivision shall comply in all 		respects with the official height of thirty feet (30') to 		peak as set forth in title 10 of this code.

B. 	Future Arrangements: Where parcels of land are subdivided 
     in unusually large lots, the parcels shall be divided, 
     where feasible, to allow for future subdividing into 			smaller parcels. Lot arrangements shall allow for the 			ultimate extension of adjacent streets through the middle 
     of wide blocks. Whenever such future subdividing or lot 			splitting is contemplated, the plan thereof shall be 			approved by the planning and zoning commission prior to 		the taking of such action.

9-4-9: 	EASEMENTS:

A. 	Utility: Unobstructed utility easements shall be provided 		along front lot lines, rear lot lines and side lot lines 		when deemed necessary. Total easement width shall not be 		less than twelve feet (12').

B. 	Drainage: Unobstructed drainageway easements shall be 			provided as required by the planning and zoning commission.

C. 	Natural Drainage Courses: All natural drainage courses 
     shall be left undisturbed or be improved in a manner that 
     will improve the hydraulics and ease of maintenance of the 			channel. (2025 Code)

9-4-10: 	RESERVE STRIPS:  Reserve strips shall conform to the 			     following standards:

A. 	Private Reserve Strips: Privately held reserve strips 			controlling access to streets shall be prohibited.

B. 	Public Reserve Strips: A one-foot (1') reserve may be 			required to be placed along half streets which are within 		the subdivision boundaries and shall be deeded in fee 
     simple to the city for future street widening.

(2025 Code)

9-4-11: 	PUBLIC SITES AND OPEN SPACES:  Public sites and open 			     spaces shall conform to the following standards:

A. 	Natural Features: Existing natural features which add value 	to residential development and enhance the attractiveness 
     of the community (such as trees, watercourses, historic 
     spots and similar irreplaceable assets) shall be preserved 
     in the design of the subdivision.

B. 	Special Development: In the case of planned unit 				developments and large-scale developments, the planning and 	zoning commission may require sufficient public and/or

private parks or open space facilities of acceptable size, 
     location and site characteristics that may be suitable for 
     the proposed development. (2025 Code)

CHAPTER 5

REQUIRED IMPROVEMENTS

SECTION:

9-5--1: 	Improvements Required

9-5-2: 	Monuments

9-5-3: 	Streets And Alleys

9-5-4: 	Curbs And Gutters

9-5-5: 	Street And Public Safety Signs

9-5-6: 	Streetlights

9-5-7: 	Sidewalks And Pedestrian Walkways

9-5-8: 	Underground Utilities

9-5-9: 	Water Supply, Sewer Systems And Solid Waste Disposal

9-5-10: 	Storm Drainage, Flood Controls

9-5-11: 	Fire Hydrants and Water Mains

9-5-12: 	Guarantee Of Improvements

9-5-13: 	Inspections

9-5-14: 	Inspector On Development Site

9-5-15: 	Failure To Complete Public Improvement Construction

9-5-16: 	Warranty Requirement

9-5-1: 	IMPROVEMENTS REQUIRED:  Every subdivider shall be 				required to install the following public and other improvements in accordance with the following conditions and specifications. (2025 Code)

9-5-2: 	MONUMENTS:  Monuments shall be set in accordance with 			     Idaho Code section 50-1303. (2025 Code)

9-5-3: 	STREETS AND ALLEYS:  All streets and alleys shall be 			     constructed in accordance with the standards and specifications adopted by the city or other applicable agency. (2025 Code)

9-5-4: 	CURBS AND GUTTERS:

A. 	Generally:

1. Vertical Curbs and Gutters: Vertical curbs and gutters 		shall be constructed on collector and arterial streets.

2. Rolled Curbs and Gutters: Rolled curbs and gutters shall 	be required on minor streets.

3. Construction: All construction shall be in accordance 		with the standards and specifications adopted by the city 
     or other applicable agency.

B. 	Driveways; Curb Cuts: All driveway openings and curbs shall 	be as specified by the city, the Idaho transportation 			department, or other applicable agency.

C. 	Exceptions: Curbs and gutters may not be required for lot 		splits and minor subdivisions upon application and approval 	of the planning and zoning commission. (2025 Code)

9-5-5: 	STREET AND PUBLIC SAFETY SIGNS:  Street name and 
          public safety signs shall be installed by the developer in the appropriate locations at each street intersection in accordance with the local ordinance. (2025 Code)

9-5-6: 	STREETLIGHTS:  All subdividers within the city limits 			     and within the area of city impact shall be required to install, at the subdivider's expense, streetlights in accordance with city specifications and standards at locations designated by the council. After installation and acceptance by the city, the city shall have the right to pay the cost of maintenance and power and assume ownership of the streetlights.

(2025 Code)

9-5-7: 	SIDEWALKS AND PEDESTRIAN WALKWAYS:

A. 	Sidewalks: Sidewalks shall be required on both sides of the 	street; except where the average width of lots, as 		measured at the street frontage line or at the building 		setback line, is under one hundred feet (100'), sidewalks 
     on only one side of the street may be allowed.

B. 	Pedestrian Walkways: Pedestrian walkways, when required, 		shall have easements at least ten feet (10') in width and 		include a paved walk at least five feet (5') in width.

C. 	Sidewalks And Crosswalks: Sidewalks and crosswalks shall be 	constructed in accordance with the standards and 				specifications of the city or other applicable agency.

D. 	Exceptions: Sidewalks and pedestrian walkways may not be 		required for lot splits and minor subdivisions upon 			application and approval of the city council.

(2025 Code)

9-5-8: 	UNDERGROUND UTILITIES:  Underground utilities are 				required. (2025 Code)

9-5-9: 	WATER SUPPLY, SEWER SYSTEMS AND SOLID WASTE DISPOSAL:

A. 	Construction: All public water supply or sewer systems 			(serving 2 or more separate premises or households) shall 
     be constructed in accordance with any adopted local plans 
     and specifications. All new public water supply or sewer 
     systems shall be an extension of an existing public system 
     whenever possible. In the event that the proposed public 
     water supply or sewer system is not an extension of an 
     existing public system, there shall be a showing by the 
     subdivider that the extension is not feasible and not in 
     the best interest of the public.

B. 	Plans Submitted to Department Of Health And Welfare: All 		water and sewer plans shall be submitted to the Idaho 			department of health and welfare or its authorized agent 
     for approval in accordance with the provisions of Idaho 
     Code section 50-1326.

C. 	Installation; Fire Hydrants: All water mains, valves, 			hydrants and connections to the city water distribution 		system shall be provided and installed by the subdivider in 	accordance with the standards and specifications of the 
     city and in conformance with existing laws. Fire hydrants 
     shall be no farther apart than four hundred feet (400') in 			commercial areas and seven hundred fifty feet (750') in 		residential areas, unless the international fire code 
     allows less.

D. 	Irrigation And Distribution Lines: Irrigation lines and 		distribution lines shall be provided in accordance with 		standard city specifications.

E. 	Public Sanitary Sewer System: A public sanitary sewer 
     system (not to be construed as a privately sponsored 
     cooperative, mutual use or other system developed and 
     controlled by private individuals or groups) shall be 
     installed in all subdivisions where they are within the 
     service area of an existing public system. Construction 
     plans, specifications and maintenance programs for sanitary 
     sewer systems shall be approved by the agency controlling 
     the system prior to recording of the final plat and shall 
     comply with the city backflow device ordinance. The council 
     shall review a sanitary system proposed to be developed as 
     a privately sponsored and controlled facility in 
     relationship to proposed or adopted urbanizing or growth 
     area comprehensive plans and sewerage plans or programs and 
     shall base its subdivision approval on treatment 
     capabilities, such as dry lines, density requirements, the 
     need for entirely new systems, and other treatment 
     possibilities that are in harmony with officially 
     recognized practices of the city.

F. 	Public Water System: A public water system (not to be 			construed as a privately sponsored cooperative, mutual use 		or other system developed and controlled by private 			individuals or groups), shall be installed. Construction 
     plans shall be approved by the agency controlling the 
     system prior to recording of the final plat. The council 
     shall review a water system proposed to be developed as a 
     privately sponsored and controlled facility in relationship 
     to proposed or adopted urbanizing or growth area water 
     plans or programs and shall base its subdivision approval 
     on harmony with the officially recognized practices of the 
     city.

G. 	Camera Checked Prior To Acceptance by City: Before the city 	accepts any sewer main lines, same shall be camera checked 		for correct grade and ensuring there exists no breaks or 		obstruction in the line at the subdivider's expense.

H. 	Solid Waste Disposal: Solid waste disposal will comply with 	all federal, state and local laws and ordinances.

(2025 Code)

9-5-10: 	STORM DRAINAGE, FLOOD CONTROLS:  An adequate storm 			     drainage system shall be required in all subdivisions.   The city or other applicable agency shall establish the requirements, and construction shall follow the specifications and procedures established by the same.

A. 	Interceptor Ditches: Interceptor ditches shall be 				established above all cut/fill slopes, and the intercepted 		water conveyed to a stable channel or natural drainageway 		with adequate capacity.

B. 	Design: Design shall be such that water on roadways is 			prevented from flowing off the roadway.

C. 	Natural Drainageways: Natural drainageways shall be rip 		rapped or otherwise stabilized below drainage and culvert 		discharge points for a distance sufficient to convey the 		discharge without channel erosion.

D. 	Runoff: Runoff from areas of concentrated impervious cover, 	i.e., roofs, driveways and roads, shall be collected and 		transported to a natural drainage with sufficient capacity 		to accept the discharge without undue erosion.

E. 	Construction Waste Material: Waste material from 				construction, including soil and other solid materials, 		shall not be deposited within the 100-year floodplain.

F. 	Completion Of System: The overall drainage system shall be 		completed and made operational at the earliest possible 
     time during construction.

G. 	Alteration Of Major Drainageways: Alteration of major 			drainageways shall be prohibited except for approved road 		crossings and drainage structures.

H. 	Hydrologic Measuring Devices: The city reserves the right 
     to require installation of hydrologic measuring devices in 		drainageways within any development at public expense.

(2025 Code)

9-5-11: 	FIRE HYDRANTS AND WATER MAINS:  Adequate fire 				     protection shall be required in accordance with all state laws. (2025 Code)

9-5-12: 	GUARANTEE OF IMPROVEMENTS:

A. 	Financial Guarantee: In lieu of the actual installation of 		required public improvements before filing of the final 		plat, the city council may permit the subdivider to provide 	a financial guarantee of performance in one or a 
     combination of the following arrangements for those 
     requirements which are over and beyond the requirements of 
     any other agency responsible for the administration, 
     operation and maintenance of the applicable public 
     improvement.

B. 	Performance Bond: The bond shall be in an amount equal to 		one hundred ten percent (110%) of the total estimated cost 		for completing construction of the specific public 			improvements, as estimated by the city clerk-treasurer and 		approved by the city council and shall be with a surety 		company authorized to do business in the state of Idaho, 		acceptable to the city council. (2025 Code)

9-5-13: 	INSPECTIONS:  Before approving a final plat and 				     construction plans and specifications for public improvements, an agreement between the subdivider and the city council shall be made to provide for checking or inspecting the construction and its conformity to the submitted plans.

(2025 Code)

9-5-14: 	INSPECTOR ON DEVELOPMENT SITE:  The city shall have 
          the right to place an inspector of its own choosing on the development site, with all expenses to be borne by the developer. (2025 Code)

9-5-15: 	FAILURE TO COMPLETE PUBLIC IMPROVEMENT CONSTRUCTION:

In the event the subdivider shall, in any case, fail to complete such work within the period of time as required by the conditions of the guarantee for the completion of public improvements, it shall be the responsibility of the city council to proceed to have such work completed. In order to accomplish this, the city council shall reimburse itself for the cost and expense thereof by appropriating the cash deposit, certified check, irrevocable bank letter of credit, or negotiable bond which the subdivider may have deposited in lieu of a surety bond or may take such steps as may be necessary to require performance by the bonding or surety company. (2025 Code)

9-5-16: 	WARRANTY REQUIREMENT:  Developers and engineers shall 			warrant workmanship materials and engineering design for one full year from the time of acceptance by the city. Developers or engineers, at their own expense, can be required to correct any defects that may exist, for which they are notified, within one year. Whenever a developer or engineer is required to make corrections or repairs to any defect, the warranty period shall be extended for one full year after said repairs or corrections are completed on the repair work.

(2025 Code)

CHAPTER 6

ADMINISTRATION AND ENFORCEMENT

SECTION:

9-6-1: 	Administration

9-6-2: 	Vacations And Dedications

9-6-3: 	Variances

9-6-4: 	Amendments

9-6-5: 	Enforcement And Penalties

9-6-1: 	ADMINISTRATION:  The council shall appoint an 				     administrator to carry out the provisions as herein specified and to serve at the pleasure of the council. The administrator shall receive and process all subdivision applications. (2025 Code)

9-6-2: 	VACATIONS AND DEDICATIONS:

A. 	Application For Vacation or Dedication: Any property owner 		desiring to vacate an existing subdivision, public right of 	way or easement shall complete and file an application with 	the administrator. These provisions shall not apply to the 		widening of any street that is shown in the comprehensive 		plan, or the dedication of streets, rights of way or 			easements to be shown on a recorded subdivision.

B. 	Administrative Action:

1. Action By Council:

a. Vacations: When considering an application for 			     vacation procedures, the city council shall establish 
          a date for a public hearing and give such public 
          notice as required 	by law. The city council may 
          approve, deny or modify the application. Whenever 
          public rights of way or lands are vacated, the city 
          council shall provide, as law prescribes, adjacent 
          property owners with a quitclaim deed for the vacated 
          rights of way in such proportions as prescribed by 
          law.

b. Dedications: When considering an application for 		   
          dedication procedures, the city council may approve, 
          deny or modify the application. Within a proposed 
          subdivision, arterial and collector streets, as shown 
          on the comprehensive plan, shall be dedicated to the 
          public in all cases. In general, all other streets 
          also shall be dedicated to public use. When a 
          dedication is approved, the required street 
          improvements shall be constructed or a bond furnished 	
          assuring the construction prior to acceptance of the 			     dedication. To complete the acceptance of any 
          dedication of land, the owner shall furnish to the 
          city council a deed describing and conveying such 
          lands to be recorded with the county recorder.

c. Decision: The council shall review the request and 		 
          all agency responses, and, at the next regularly 
          scheduled meeting as time permits, either approve, 
          conditionally approve or deny the request. (2025 Code)

9-6-3: 	VARIANCES:  No variance shall be favorably acted upon 			     by the city council unless there is a finding as a result of a public hearing, that all of the following exist:

A. 	Special Circumstances: There are such special circumstances 	or conditions affecting the property that strict 
     application of the provisions of this title would clearly 
     be impracticable or unreasonable. In such cases, the 
     subdivider shall first state his reasons, in writing, as to 
     the specific provision or requirement involved.

B. 	Extraordinary Hardship: Strict compliance with the 			requirements of this title would result in extraordinary 		hardship to the subdivider because of unusual topography or 	other legal or physical conditions and would result in 		inhibiting the achievement of the objectives of this title.

C. 	Detrimental To Public Welfare; Injurious to Other Property: 	The granting of the specified variance will not be 			detrimental to the public welfare or injurious to other 		property in the area in which the property is situated.

D. 	Violation Of Idaho Code: Such variance will not violate the 	provisions of the Idaho Code.

E. 	Interest And Purpose Of Title: Such variance will not have 		the effect of nullifying the interest and purpose of this 		title and the comprehensive plan. (2025 Code)

9-6-4: 	AMENDMENTS:  The city council may, from time to time, 			     amend, supplement or repeal the regulations and provisions of this title in the following manner:

A. 	Hearing; Notice: The city council, prior to adopting an 		amendment, supplement, or repeal of this title, shall 			conduct at least one public hearing in which interested 		persons shall have an opportunity to be heard. At least 		fifteen (15) days prior to the hearing, notice of the time 		and place and a summary of the proposed plan to be 
     discussed shall be published in the official newspaper or 
     paper of general circulation within the jurisdiction. 
     Notice of intent to amend, supplement or repeal this title 
     shall be sent to all political subdivisions providing 
     services within the planning jurisdiction, including school 
     districts, at least fifteen (15) days prior to the public 
     hearing scheduled by the commission.

B. 	Council Action: Following the city council hearing, if the 		city council makes a material change in this ordinance 
     other than that published for the present hearing, further 
     notice and hearing shall be provided before the city 
     council takes action. A record of the hearings, findings 
     made and actions taken shall be maintained. (2025 Code)

9-6-5: 	ENFORCEMENT AND PENALTIES:

A. 	Enforcement:

1. No subdivision plat required by this title or the Idaho 		Code shall be admitted to the public land records of 			Jefferson or Bonneville County or recorded by the county 		recorder until such subdivision plat has received final 		approval of the city council.

2. No public board, agency, commission, official or other 		authority shall proceed with the construction of or 			authorize the construction of any of the public 
     improvements required by this title until the final plat 
     has received the approval of the city council.

3. No permits for construction of any structure shall be 		issued on any parcel or lot until it has been determined 		such parcel or lot meets the requirements of this title.

4. The city attorney shall, in addition to taking whatever 		criminal action deemed necessary, take steps to civilly 		enjoin any violation of this title.

B. 	Violations; Penalties: Violations of any of the provisions 		of this title or failure to comply with any of its 			requirements shall constitute a misdemeanor and be 			punishable as such. Each day such violation continues shall 	be considered a separate offense. The landowner, tenant, 		subdivider, builder, public official or any other person 
     who commits, participates in, assists in or maintains such 			violation may each be found guilty of a separate offense. 		Nothing herein contained shall prevent the city council or 		any other public official or private citizen from taking 		such lawful action as is necessary to restrain or prevent 		any violation of this title or the Idaho Code.

(2025 Code)